| Document | CoreRDKB_Spec_v1.0 |
| --- | --- |
| Prepared by | RDKM |
| Reviewed by | RDKM |
| Accepted by | RDKM |
| Abstract | This specification defines fundamental requirements for adopting RDK Broadband |

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0 | 2026-04 | RDKM | Initial release. 24 features across 7 device profiles. |

=============================================================

COPYRIGHT NOTICE, LICENSE AND DISCLAIMER

=============================================================

Copyright © RDK Management, LLC (2025-2026). All rights reserved.

The information within this document is the property of RDK Management, LLC and its use and disclosure are restricted, except as expressly set forth herein.

RDK Management, LLC hereby grants you a fully-paid, non-exclusive, nontransferable, worldwide, limited and revocable license (without the right to sublicense), under RDK Management, LLC's applicable copyright rights, to view, download, save, reproduce and use the document solely for your own internal purposes and in accordance with the terms of the license set forth herein.

This license does not authorize you to, and you expressly warrant that you shall not:

(a) permit others (outside your organization) to use this document.

(b) post or publish this document.

(c) modify, adapt, translate, or otherwise change this document in any manner or create any derivative work based on this document.

(d) remove or modify any notice or label on this document, including this Copyright Notice, License and Disclaimer.

RDK Management, LLC does not grant you any license hereunder other than as

expressly stated herein.

Elements of this document may be subject to third party intellectual property rights, including without limitation, patent, copyright or trademark rights, and any such third party may or may not be a member of the RDK community. RDK Management, LLC members grant other RDK community members certain intellectual property rights as set forth in the RDK IPR Policy. RDK Management, LLC members do not grant you any rights under this license. RDK Management, LLC is not responsible for and shall not be held responsible in any manner for, identifying or failing to identify any or all such third-party intellectual property rights.

This document may reference specifications and technologies from third-party organizations including, but not limited to, the Wi-Fi Alliance, Broadband Forum, and Connectivity Standards Alliance. RDK Management, LLC makes no endorsement of any such third-party products, services, or specifications.

Please visit www.rdkcentral.com for more information on how to become a member of the RDK community.

This document and the information contained herein are provided on an "AS IS" basis and RDK Management, LLC DISCLAIMS ALL WARRANTIES EXPRESS OR IMPLIED, INCLUDING BUT NOT LIMITED TO (A) ANY WARRANTY THAT THE USE OF THE INFORMATION HEREIN WILL NOT INFRINGE ANY RIGHTS OF THIRD PARTIES (INCLUDING WITHOUT LIMITATION ANY INTELLECTUAL PROPERTY RIGHTS INCLUDING PATENT, COPYRIGHT OR TRADEMARK RIGHTS); OR (B) ANY IMPLIED WARRANTIES OF MERCHANTABILITY, FITNESS FOR A PARTICULAR PURPOSE, TITLE OR NONINFRINGEMENT.

IN NO EVENT WILL RDK MANAGEMENT, LLC BE LIABLE FOR ANY LOSS OF PROFITS, LOSS OF BUSINESS, LOSS OF USE OF DATA, INTERRUPTION OF BUSINESS, OR FOR ANY OTHER DIRECT, INDIRECT, SPECIAL OR EXEMPLARY, INCIDENTAL, PUNITIVE OR CONSEQUENTIAL DAMAGES OF ANY KIND, IN CONTRACT OR IN TORT, IN CONNECTION WITH THIS DOCUMENT OR THE INFORMATION CONTAINED HEREIN, EVEN IF ADVISED OF THE POSSIBILITY OF SUCH LOSS OR DAMAGE.

All company, brand and product names in this document may be trademarks that are the sole property of their respective owners. This notice and disclaimer must be included on all copies of this document.

RDK Management, LLC

www.rdkcentral.com

=============================================================

Acknowledgements

This specification was developed by the RDK Management team with contributions from across the RDK Broadband community. The content reflects the collective engineering work, platform experience, and technical input of operators, silicon vendors, hardware manufacturers, system integrators, and open-source contributors who have built and deployed RDK-B at scale.

Special acknowledgement is due to the RDK Central Technical Steering Committee (TSC) for their governance oversight and strategic direction, and to the working group members who contributed feature definitions, HAL interface specifications, and testing requirements that form the technical foundation of this document.

We also acknowledge the broader open-source community — the developers who have contributed code, filed issues, reviewed pull requests, and shared their deployment experience through the rdkcentral GitHub organization. This platform exists because of that collective effort.

Contributing Organizations

This specification was developed with contributions from the following RDK Central member organizations and their architecture teams:

- Charter Communications

- Comcast

- Sky

- Liberty Global

- Vodafone

- Deutsche Telekom

- T-Mobile

- SFR

Specification Basis & Document Conventions

This document follows architecture description practices consistent with ISO/IEC/IEEE 42010, the international standard for architecture description, applying its core constructs of stakeholders, concerns, architecture viewpoints, and architecture views as reflected throughout this specification. Normative requirement language in this specification follows the keyword conventions defined in IETF RFC 2119 and RFC 8174; capitalized MUST, MUST NOT, SHOULD, SHOULD NOT, and MAY carry the meanings defined in those documents, while lowercase usage of the same words carries no normative force. Structurally, this document aligns with Broadband Forum Technical Report drafting conventions, consistent with its extensive reliance on Broadband Forum data models (TR-181, TR-369) throughout the specification. The tables below map the principal constructs of each referenced standard to their corresponding treatment in this specification.

Table A — ISO/IEC/IEEE 42010 Construct Mapping

| 42010 Construct | Treatment in This Specification | Status |
| --- | --- | --- |
| System of Interest | The Core RDK-B platform (Section 2) | Defined |
| Stakeholder | Section 8 — ISPs, SoC Vendors, OEMs/ODMs, System Integrators, Third-Party App Developers, Cloud/Platform Service Providers, Standards Bodies | Defined |
| Concern | Section 3 (Core Goals) and Section 4 (Benefits and Value Proposition) | Addressed |
| Architecture Viewpoint | Section 2.3 (Five-Tier Model) and Section 2.5 (Layering/Standpoint) | Addressed |
| Architecture View | Core RDK-B architecture diagram (component/layer view) | Defined |
| Architecture Rationale | Section 6 (Roadmap), Section 7.2 (Governance Entry and Review Criteria), and platform-level engineering decisions documented elsewhere | Addressed; consolidation into a single rationale clause planned for a future revision |
| Correspondence Rules | Section 7.1.1 (Layering & Dependency Direction), Section 7.2.6 (Platform Layer Invocation Rules) | Defined |

Table B — RFC 2119 / RFC 8174 Keyword Convention

| Location | Current Treatment | Status |
| --- | --- | --- |
| Section 10.1 (Feature Status Classification) | Capitalized MUST use correctly | Conformant |
| Section 7.1.1, 7.1.2, 7.1.3 (Development Standards) | Keywords currently expressed in lowercase form | Capitalization harmonization planned for next revision |

Table C — Broadband Forum Technical Report Structural Alignment

| BBF TR Convention | Treatment in This Specification | Status |
| --- | --- | --- |
| Issue History / Document Control table | Front-matter Document table and Version/Date/Author/Description table | Defined |
| Purpose and Scope (separate clauses) | Combined under Section 1.1 | Structurally merged |
| Definitions and Abbreviations (separate tables) | Combined under Section 1.3 Terminology | Structurally merged |
| Numbered, normative References clause | Not yet present as a standalone clause; external specifications currently referenced inline (Section 1.3, Section 7.1.3) | Planned for introduction in a future revision |
| Numbered requirement body | Sections 7, 9–13 (Architecture Standards, Device Profiles, Feature Matrix, Governance) | Defined |

TABLE OF CONTENTS

Executive Summary	15

Key Strategic Objectives	15

1. Introduction	16

1.1 Purpose and Scope	16

1.2 Document Organization	16

1.3 Terminology	16

2. What is RDK-B?	18

2.1 What is RDK-B Core & NOT RDK-B Core	18

2.2 Platform Origins and Business Governance	19

2.3 Platform Architecture — Five-Tier Model	19

2.4 Licensing	19

2.5 Core RDK-B High level Architecture/Layering/Standpoint	20

End – End	20

RDK Layering	20

RDK Ready with Core RDK Standpoint	20

3. RDK-B Core Goals	21

3.1 Goal 1: Easier Component Reuse	21

3.2 Goal 2: Improved Modularity — Build-Time Dependencies	21

3.3 Goal 3: Improved Modularity — Runtime Dependencies	22

3.4 Goal 4: RDK Ready — Vendor Certification Program	23

3.5 Goal 5: Reduced Code Size	24

3.6 Goal 6: Consistent Interface Definitions	24

4. Benefits and Value Proposition	25

5. RDK-B Adoption and Industry Leadership	26

6. Roadmap and Next Steps	27

6.1 Next Release — Quick Wins	27

6.2 Future Releases — Structural Program	27

7. Architecture Standards and Governance	28

7.1 Development Standards	28

7.1.1 Process & Architecture Governance Standards	28

7.1.2 Implementation & Coding Standards	30

7.1.3 Industry Standards Conformance	32

7.2 Technical Governance Process	35

7.2.1. Purpose	35

7.2.2. Governance Scope	35

7.2.3. Classification of Changes	35

7.2.4. Governance Entry Criteria	36

7.2.5. Architecture Review Rules	37

7.2.6. Platform Layer Invocation Rules	38

7.2.7. Dependency Governance	39

7.2.8. Development Flow	39

7.2.9. Testing Governance	40

7.2.10. Testing Expectations by Change Type	42

7.2.11. Platform Test Matrix	43

7.2.12. Branch and Build Governance	44

7.2.13. Review and Approval Model	44

7.2.14. Merge Criteria	45

7.2.15. Test Evidence Requirements	46

7.2.16. Release Governance	46

7.2.17. Flag Governance	47

7.2.18. Reference Platform and Community Contribution Rules	48

7.2.19. Governance Checklist	48

7.2.20. Final Governance Rule	49

7.3 Component Governance Process	50

7.3.1 Purpose	50

7.3.2 Component Registry	50

7.3.3 Component Lifecycle States	51

7.3.4 Component Ownership Model	51

7.3.5 Periodic Health Review	52

7.3.7 Interface Stability Tagging	54

7.3.8 Component Governance Checklist	54

7.4 Escalation Process	55

8. RDK-B Ecosystem and Stakeholders	57

8.1 Internet Service Providers (ISPs / Operators)	57

Value Proposition for ISPs	57

ISP-Specific Feature Requirements	57

Representative ISP Deployments	58

8.2 System-on-Chip (SoC) Vendors	58

Value Proposition for SoC Vendors	58

SoC Vendor Responsibilities in RDK-B	58

Representative SoC Vendors	59

HAL Integration Model	59

8.3 Original Equipment Manufacturers (OEMs / ODMs)	59

OEM Responsibilities in RDK-B	59

OEM Build System	60

Representative OEMs / ODMs	60

8.4 System Integrators (SIs)	60

What SIs Do in the RDK-B World	60

Representative System Integrators	60

8.5 Third-Party Application Developers	61

The App Container Ecosystem	61

Categories of Third-Party Applications	61

Developer Access Model	62

8.6 Cloud and Platform Service Providers	62

8.7 Standards Bodies and Alliances	62

8.8 Summary: Stakeholder Value Matrix	62

9. RDK-B Deployment Models	64

10. Device Profiles	65

10.1 Feature Status Classification	65

11. Master Feature Matrix	66

12. Feature 1: Wi-Fi LAN	68

12.1 Overview	68

12.2 Key Capabilities	68

12.3 Feature Detail and Profile Matrix	68

12.4 Component Dependencies	72

12.5 Implementation Notes	72

12.6 Testing Scope	73

13. Feature 2: Ethernet LAN	74

13.1 Overview	74

13.2 Key Capabilities	74

13.3 Feature Detail and Profile Matrix	74

13.4 Component Dependencies	75

13.6 Testing Scope	75

14. Feature 3: MoCA LAN	76

14.1 Overview	76

14.2 Key Capabilities	76

14.3 Feature Detail and Profile Matrix	76

14.4 Component Dependencies	76

14.6 Testing Scope	77

15. Feature 4: Local WebUI	78

15.1 Overview	78

15.2 Key Capabilities	78

15.3 Feature Detail and Profile Matrix	78

15.4 Component Dependencies	78

15.6 Testing Scope	79

16. Feature 5: Remote Device Management	80

16.1 Overview	80

16.2 Key Capabilities	80

16.3 Feature Detail and Profile Matrix	80

16.4 Component Dependencies	81

16.5 Implementation Notes	81

16.6 Testing Scope	81

17. Feature 6: Telemetry & Data collection	82

17.1 Overview	82

17.2 Key Capabilities	82

17.3 Feature Detail and Profile Matrix	82

17.4 Component Dependencies	83

17.5 Implementation Notes	83

17.6 Testing Scope	83

18. Feature 7: Diagnostics & Testing	84

18.1 Overview	84

18.2 Key Capabilities	84

18.3 Feature Detail and Profile Matrix	84

18.4 Component Dependencies	85

18.5 Implementation Notes	85

18.6 Testing Scope	85

19. Feature 8: System Logging	87

19.1 Overview	87

19.2 Key Capabilities	87

19.3 Feature Detail and Profile Matrix	87

19.4 Component Dependencies	88

19.5 Implementation Notes	88

19.6 Testing Scope	88

20. Feature 9: Inter Process Communication	89

20.1 Overview	89

20.2 Key Capabilities	89

20.3 Feature Detail and Profile Matrix	89

20.4 Component Dependencies	90

20.5 Implementation Notes	90

20.6 Testing Scope	90

21. Feature 10: Matter/IoT	91

21.1 Overview	91

21.2 Key Capabilities	91

21.3 Feature Detail and Profile Matrix	91

21.4 Component Dependencies	92

21.6 Testing Scope	92

22. Feature 11: Downloadable App containers	93

22.1 Overview	93

22.2 Key Capabilities	93

22.3 Feature Detail and Profile Matrix	93

22.4 Component Dependencies	94

22.5 Implementation Notes	94

22.6 Testing Scope	94

23. Feature 12: Voice/VoIP	95

23.1 Overview	95

23.2 Key Capabilities	95

23.3 Feature Detail and Profile Matrix	95

23.4 Component Dependencies	96

23.5 Implementation Notes	96

23.6 Testing Scope	96

24. Feature 13: Firmware Upgrade	97

24.1 Overview	97

24.2 Key Capabilities	97

24.3 Feature Detail and Profile Matrix	97

24.4 Component Dependencies	98

24.5 Implementation Notes	98

24.6 Testing Scope	98

25. Feature 14: Connected device discovery	99

25.1 Overview	99

25.2 Key Capabilities	99

25.3 Feature Detail and Profile Matrix	99

25.4 Component Dependencies	99

25.6 Testing Scope	100

26. Feature 15: System startup & orchestration	101

26.1 Overview	101

26.2 Key Capabilities	101

26.3 Feature Detail and Profile Matrix	101

26.4 Component Dependencies	102

26.5 Implementation Notes	102

26.6 Testing Scope	102

27. Feature 16: Configuration Persistence	103

27.1 Overview	103

27.2 Key Capabilities	103

27.3 Feature Detail and Profile Matrix	103

27.4 Component Dependencies	103

27.5 Implementation Notes	104

27.6 Testing Scope	104

28. Feature 17: Factory reset	105

28.1 Overview	105

28.2 Key Capabilities	105

28.3 Feature Detail and Profile Matrix	105

28.4 Component Dependencies	106

28.5 Implementation Notes	106

28.6 Testing Scope	106

29. Feature 18: IP Routing/NAT/Firewall	107

29.1 Overview	107

29.2 Key Capabilities	107

29.3 Feature Detail and Profile Matrix	107

29.4 Component Dependencies	108

29.5 Implementation Notes	108

29.6 Testing Scope	108

30. Feature 19: Bridge and VLAN configuration	109

30.1 Overview	109

30.2 Key Capabilities	109

30.3 Feature Detail and Profile Matrix	109

30.4 Component Dependencies	109

30.6 Testing Scope	109

31. Feature 20: Traffic redirection	110

31.1 Overview	110

31.2 Key Capabilities	110

31.3 Feature Detail and Profile Matrix	110

31.4 Component Dependencies	110

31.6 Testing Scope	110

32. Feature 21: DHCP Server	111

32.1 Overview	111

32.2 Key Capabilities	111

32.3 Feature Detail and Profile Matrix	111

32.4 Component Dependencies	112

32.6 Testing Scope	112

33. Feature 22: DNS	113

33.1 Overview	113

33.2 Key Capabilities	113

33.3 Feature Detail and Profile Matrix	113

33.4 Component Dependencies	113

33.6 Testing Scope	114

34. Feature 23: WAN Connectivity	115

34.1 Overview	115

34.2 Key Capabilities	115

34.3 Feature Detail and Profile Matrix	115

34.4 Component Dependencies	117

34.5 Implementation Notes	118

34.6 Testing Scope	118

35. Feature 24: Parental control	119

35.1 Overview	119

35.2 Key Capabilities	119

35.3 Feature Detail and Profile Matrix	119

35.4 Component Dependencies	120

35.6 Testing Scope	120

36. Feature 25: WiFi Hotspot	121

36.1 Overview	121

36.2 Key Capabilities	121

36.3 Feature Detail and Profile Matrix	121

36.4 Component Dependencies	121

36.5 Implementation Notes	121

37. Feature 26: Code Security	122

37.1 Overview	122

37.2 Key Capabilities	122

37.3 Feature Detail and Profile Matrix	122

37.4 Component Dependencies	122

37.5 Implementation Notes	122

38. Security Framework	123

38.1 Security Principles	123

38.2 Wi-Fi Security Requirements	123

38.3 Management Channel Security	123

38.4 Firmware Security	124

38.5 Container Security (App Framework)	124

38.6 Security Testing Requirements	124

39. Performance Requirements	125

39.1 System Performance	125

39.2 Wi-Fi Performance Requirements	125

39.3 WAN Performance Requirements	126

39.4 Reliability Requirements	126

40. Testing Core	127

40.1 Test Framework Architecture	127

40.2 RDK Platform Test Suite (Mandatory for all CORE Components — L2 + L3)	127

40.2.1 Mandatory Test Categories	127

40.2.2 Timing SLA Gates	128

40.2.3 Test Evidence Required for Release	128

40.3 OneWiFi Test Suite (OneWiFiTestSuite)	129

40.4 Regression Campaign Structure	129

40.5 CI/CD Pipeline	129

40.5.1 Build CI	129

40.5.2 Hardware-in-the-Loop (HIL) CI	130

40.5.3 TIP Wi-Fi Validation	130

40.6 Lab Topology	130

40.7 Acceptance Test Criteria (ATC)	131

41. Legacy Component Migration Plan	132

41.1 Migration Principles	132

42. Component Repository Reference	134

43. Revision History	137

RDK-B BROADBAND PLATFORM

CORE FEATURE SPECIFICATION

Version 1.0 | RDK Central | 2026

Confidential — Internal Reference Document

# Executive Summary

What is RDK-B Core? RDK-B Core components are the foundational, open-source, vendor-independent building blocks of the RDK Broadband platform. They are modular, actively maintained by the RDK organization and community, and designed to be combined into software builds for a wide variety of broadband gateway device types — from simple headless modems to full-featured gateways with IoT and application container support.

This document is the authoritative specification for the RDK-B Core platform. It formally captures the platform's goals, design principles, ecosystem, all 24 feature areas with comprehensive per-feature analysis, the testing framework, security requirements, performance criteria, and the architectural roadmap. It is intended for the RDK team, engineering contributors, SoC vendors, OEMs, and system integrators.

| 24 Core Feature Areas | 7 Device Profiles | 50+ Open-Source Repos | 5 Core Goals | 135M+ Devices Globally | 8+ Operator Deployments |
| --- | --- | --- | --- | --- | --- |

## Key Strategic Objectives

| Objective | Description |
| --- | --- |
| Easier Component Reuse | Core components must work across all device profile types without operator-specific modifications. |
| Improved Modularity | Eliminate unnecessary build-time and runtime dependencies between components. |
| RDK Ready Certification | Formal vendor test and certification for each HAL interface enabling mix-and-match SoC sourcing. |
| Reduced Code Size | Support the full range from small extender/modem platforms to high-end gateways. |
| Consistent Interface Standards | Uniform PSM storage and RBUS IPC across all components — improving testability and talent pool. |

# 1. Introduction

This document is the complete Core Feature Specification for the RDK-B (Reference Design Kit – Broadband) platform. It covers the platform's goals, design principles, stakeholder ecosystem, device profiles, all 24 feature areas, the testing framework, security framework, performance requirements, architectural roadmap, governance standards, and component repository references.

## 1.1 Purpose and Scope

- RDK team members defining feature scope, device profiles, and component inclusion decisions

- Engineering contributors integrating and validating RDK-B components on SoC hardware

- QA and test teams building test plans aligned to mandatory and optional features

- SoC vendors understanding HAL integration requirements and RDK Ready certification expectations

- OEMs / ODMs seeking to understand the reference software stack for product development

- System integrators defining customization scope and integration effort

- Third-party developers targeting the RDK-B application container ecosystem

## 1.2 Document Organization

Executive Summary provides a platform overview. Sections 1–2 introduce and define the platform. Section 3 covers the five Core Goals in depth. Sections 4–7 cover Benefits, Industry Leadership, Roadmap, and Governance. Sections 8–9 cover the Ecosystem and Deployment Models. Sections 10–11 define Device Profiles and the Master Feature Matrix. Sections 12–37 are the individual feature specifications (one section per feature). Section 38 is the Security Framework. Section 39 covers Performance Requirements. Section 40 is the Testing Core. Section 41 is the Legacy Component Migration Plan. Section 42 is the Repository Reference.

## 1.3 Terminology

| Term | Definition |
| --- | --- |
| CORE | Identifies a small subset of foundational components forming basic infrastructure usable by other software, required across all device profiles, and eligible to be listed as a dependency by other components and features. Marked CORE in the Core Features column. Non-conformant if absent. Test coverage required. |
| OPT | Optional. Supported; included at operator/ODM discretion. |
| N/A | Not applicable to this device profile. |
| HAL | Hardware Abstraction Layer — boundary between RDK-B middleware and vendor SoC firmware. |
| RBUS | Modern RDK-B IPC bus, successor to CCSP/D-Bus. |
| CCSP | Common Component Software Platform — legacy IPC and data-model framework. |
| TR-181 | Broadband Forum data model for home gateway device parameters. |
| TR-369 / USP | User Services Platform — modern device management protocol (commands, events, bulk data). |
| TR-069 / CWMP | CPE WAN Management Protocol — legacy ACS-based device management. |
| OneWifi | Unified Wi-Fi management component abstracting all 802.11 radio operations. |
| EasyMesh | Wi-Fi Alliance multi-AP specification for seamless whole-home mesh networking. |
| DSM | Device Software Modules — TR-181 SoftwareModules app container implementation. |
| PSM | Persistent Storage Manager — durable key-value config storage across reboots. |
| P&M | Provisioning and Management — legacy device init scripts (under active refactoring). |
| OCI | Open Container Initiative — container runtime and image specification standard. |
| T2 | Telemetry 2.0 — profile-driven configurable telemetry collection framework. |
| xConf | Cross-Configuration — cloud service for RFC feature control and firmware orchestration. |
| CPE | Customer Premises Equipment — devices deployed at subscriber premises. |
| SoC | System-on-Chip — the silicon platform on which RDK-B runs. |
| OEM | Original Equipment Manufacturer — builds and supplies the CPE hardware. |
| ODM | Original Design Manufacturer — designs CPE hardware for other brands. |
| SI | System Integrator — customizes and integrates RDK-B for specific operator deployments. |
| ISP | Internet Service Provider / Operator — deploys CPE to subscribers. |
| TSC | Technical Steering Committee — RDK Central governance body for platform direction. |
| NL80211 | Industry-standard Linux Wi-Fi kernel interface used as the preferred RDK-B Wi-Fi HAL. |
| Distro Flag | Yocto DISTRO_FEATURES build-time flag controlling conditional feature inclusion. |
| DFS | Dynamic Frequency Selection — radar detection and channel avoidance for 5/6 GHz Wi-Fi. |
| MLO | Multi-Link Operation — Wi-Fi 7 feature enabling simultaneous use of multiple radio bands. |
| PD | Prefix Delegation — IPv6 mechanism for ISP to allocate a prefix block to the gateway. |
| SLAAC | Stateless Address Autoconfiguration — IPv6 client self-configures address from RA prefix. |

# 2. What is RDK-B?

Platform Definition RDK-B (Reference Design Kit – Broadband) is an open-source, modular, operator-grade middleware platform providing a standardized software stack for broadband residential and business gateway devices. Core components are open-sourced, vendor-independent, actively maintained, and each represented by one or more GitHub repositories.

RDK-B sits between the hardware SoC/BSP layer and the operator's cloud management systems, providing a rich set of pre-integrated components for Wi-Fi, routing, diagnostics, telemetry, device management, IoT, and application containerization. Its HAL abstraction model means that a single software stack can run on radically different SoC platforms. Typically, each core component contains a logical grouping of related features, and this modular approach allows software developers to select the set of high-level features required for each software build.

## 2.1 What is RDK-B Core & NOT RDK-B Core

| Core Category | Description |
| --- | --- |
| Foundation | Core components" are foundational building blocks that can be combined in a modular way to create various software builds. Core components are open-sourced and vendor independent. |
| Building block | A core component is represented by one or more GitHub repos containing the code for that building block. |
| Logical grouping | Typically, each core component contains a logical grouping of related features. This modular approach allows software developers to choose the set of high-level features required for each software build. Many core components are considered optional to provide this flexibility |
| Actively maintained | Core components are actively maintained by the RDK organization and community. |

| Non-Core Category | Description |
| --- | --- |
| Platform-Specific Manifest Lists and Meta Layers | Yocto/BitBake manifests and meta layers specific to a particular hardware board. These are platform integration artefacts, not reusable middleware. |
| Platform-Specific Build Examples | Example builds and configurations targeting a specific SoC or reference board. Useful for developers but not part of the reusable Core. |
| Platform-Specific HAL Implementations | HAL implementations for specific platforms (e.g., Banana Pi, specific Broadcom chipsets). The HAL interface definition is Core; the implementation is not. |
| Deprecated Code Preserved for History |  |

## 2.2 Platform Origins and Business Governance

RDK was created by Liberty Global and Comcast in 2012 for set-top boxes (RDK-V), then extended to broadband gateways (RDK-B) in 2014. RDK Central (rdkcentral.com) has governed the project since 2016 as a neutral community entity.

- 2012 — RDK inception by Liberty Global and Comcast for video (RDK-V)

- 2014 — RDK-B broadband stream launched; first DOCSIS gateway deployments

- 2016 — RDK Central LLC established as neutral governance entity

- 2018 — RBUS IPC introduced, replacing legacy CCSP/D-Bus

- 2020 — Wan Manager/Interface Managers Introduced

- 2021 — DSM and Dobby container runtime merged

- 2022 — USP/TR-369 published

- 2023 — OneWifi launched;

- 2024 — 100M+ devices deployed globally

- 2025 — Wi-Fi 7 (802.11be) integrated; EasyMesh Introduced

- 2026 — Matter/IoT (BartonCore) introduced

## 2.3 Platform Architecture — Five-Tier Model

| Tier | Layer | Description |
| --- | --- | --- |
| 5 (Top) | Cloud / ACS | Operator cloud: ACS, USP Controller, xConf, WebConfig, Telemetry backends, Crash collection, Log aggregation. |
| 4 | Management Protocols | parodus/WebPA, usp-pa-vendor-rdk, tr069-protocol-agent, xconf-client, WebconfigFramework, T2 telemetry. |
| 3 | RDK-B Middleware | Core RDK-B components: OneWiFi, wan-manager, dhcp-manager, DSM, Dobby, BartonCore, all 24 feature components. All communicate via RBUS. |
| 2 | HAL / BSP | rdkb-halif-* interface libraries and vendor BSP. Boundary between RDK-B middleware and SoC firmware. |
| 1 (Base) | Hardware / SoC | Physical silicon: SoC CPU, Wi-Fi radio, modem chipset, Ethernet switch, Flash/RAM. |

## 2.4 Licensing

- Apache License 2.0 — primary license for most middleware components

- LGPL-2.1 — used by library components for compatibility with proprietary HAL implementations

## 2.5 Core RDK-B High level Architecture/Layering/Standpoint

### End – End

  - To be defined

Experience

Experience

Experience

RDK Layering

RDK platform test suite

RDK Platform – Core product functionality and enablement

Vendor test suite

Vendor Layer – Providing a consistent hardware abstraction

| Test Suite | Definition | Owner |
| --- | --- | --- |
| RDK Platform Test Suite | RDK Platform — component behavior, integration, data model, management, security, profile conformance | Core RDK-B Owner |
| Vendor Test Suite | Vendor Layer — HAL implementation correctness, SoC-specific behavior | HAL Spec Owner |

Experience Providers

RDK Ready with Core RDK Standpoint

Silicon Vendor

Finished Product

RDK Ready Vendor Layer

Device Manufacturers

RDK Management

RDK Management

RDK Management: Specs, Tests, Core RDK, Apps & Certifications services (incl via 3PL)

# 3. RDK-B Core Goals

Foundation Statement RDK-B Core is the foundation layer providing essential services for all RDK-B devices — from the smallest modem/ONU to the most capable gateway with IoT and app containers.

## 3.1 Goal 1: Easier Component Reuse

| The Goal | The Challenge |
| --- | --- |
| Components must be easy to reuse across all device profile types: - Simple Modem/ONU (headless, no Wi-Fi) - Ethernet Router / Wi-Fi AP (standalone) - Gateway Router / Wi-Fi AP (full broadband GW) - Gateway Router + EasyMesh Controller - Easy Mesh Extender (agent-only profile) - Add-on profiles: App lifecycle management, IoT | Reuse was historically difficult because: - RDK Central releases focused only on Ethernet Wi-Fi Router profile - Comcast contributions focused only on Wi-Fi Gateway profile - Code written with profile-specific assumptions making reuse in other profiles difficult - Default values (e.g., Wi-Fi SSID names) were operator-specific — required changes before reuse - Operators had to fork and modify components to use them — increasing cost and time-to-market - High memory requirements prevented deployment on smaller device platforms |
| Foundation: RDK-B Core is the foundation layer providing essential services for all RDK-B devices. |  |

## 3.2 Goal 2: Improved Modularity — Build-Time Dependencies

| The Goal | The Challenge |
| --- | --- |
| Avoid tight coupling and unnecessary build-time dependencies: - Developers must not manually edit/remove dependencies when reusing components - Optional components must be excludable without modifying other component build recipes - Build-time Distro Flags must clearly define what is included or excluded | Many unnecessary build-time dependencies exist today. Example — these components incorrectly depend on the Cable Modem HAL: - OneWifi (Wi-Fi should not depend on cable modem) - WAN Manager (should be access-technology agnostic) - DHCP Manager - Provisioning and Management (P&M) - Utopia - Test and Diagnostic - TR-069 Protocol Agent - xConf Client — and many others |
| Foundation: RDK-B Core is the foundation layer providing essential services for all RDK-B devices. |  |

## 3.3 Goal 3: Improved Modularity — Runtime Dependencies

| The Goal | The Challenge |
| --- | --- |
| Runtime dependencies to optional components must be flexible: - Components must not crash or error when an optional dependency is absent - A component using another component's RBUS API must handle the case where that component is not running - Identify all inflexible runtime dependencies and refactor to dynamic patterns | Runtime dependencies today are not flexible. Example — these components hard-depend on T2 Telemetry (which is optional): - Utopia — crashes if T2 absent - RFC (Remote Feature Control) - CcspCr (CCSP Component Registrar) - Test and Diagnostic - TR-069 Protocol Agent - xConf Client - OneWifi, CcspWiFiAgent, CcspHotspot — and many others |
| Foundation: RDK-B Core is the foundation layer providing essential services for all RDK-B devices. |  |

## 3.4 Goal 4: RDK Ready — Vendor Certification Program

| RDK Ready Pillar | Description |
| --- | --- |
| Standard and Uniform Testing | A test suite must exist for each HAL interface spec. All vendors run the same tests — published in rdkb-halif-test-* repositories. |
| Vendor Certification Program | Formal process to certify vendor compliance with specific HAL interface versions. |
| Modular Certification | Vendors certify only the HAL interfaces relevant to their silicon. Wi-Fi SoC vendors certify rdkb-halif-wifi; DOCSIS SoC vendors certify rdkb-halif-cable-modem. |
| Mix-and-Match SoC Support | Operators may combine SoCs from multiple vendors on one platform — Wi-Fi from Vendor A, GPON from Vendor B, DOCSIS from Vendor C. |
| Industry Standard Interfaces First | Where standards exist (e.g., NL80211 for Wi-Fi), use them as the RDK-B HAL. This reduces certification burden. |
| Supplementary Specs for Gaps | Where standard interfaces have functional gaps, RDK-B defines an extension spec — "NL80211 + RDK extensions" — covering the gaps. |
| Version Control and Compatibility | Tight version control between each HAL interface spec revision, the RDK-B component that consumes it, and the vendor code certified against it. |

## 3.5 Goal 5: Reduced Code Size

| The Goal | The Challenge |
| --- | --- |
| Code must scale from small modem/ONUs to high-end gateways: - Each component must support a well-defined, limited set of features - Code must allocate only the minimum required memory - Excluding components from a build must be clean and documented - Feature flags (Distro Flags) must cover sub-feature selection within components | Large, monolithic components make small lean profiles difficult: - Utopia and P&M are so large it is difficult to know what services they provide — must be refactored and split - CCSP Common Library allocates excessive memory — components must be refactored to avoid it - DHCP Manager supports multiple modes but needs more Distro Flags to select only required modes - Documentation for including/excluding optional components is insufficient |
| Foundation: RDK-B Core is the foundation layer providing essential services for all RDK-B devices. |  |

## 3.6 Goal 6: Consistent Interface Definitions

| The Goal | The Challenge |
| --- | --- |
| Uniform persistent storage and IPC improve testability, training, and talent pool: - Use PSM as the single persistent storage mechanism for all components - Use RBUS as the standard IPC for all inter-component communication - Create test code that exercises IPC interfaces as they are actually used in production - Pre-validated core reduces integration and testing cycles — faster time-to-market | Multiple inconsistent approaches fragment the developer experience: - Multiple persistent storage approaches: sysConfig, PSM, Platform HAL, mySQL — each with different APIs and test coverage - Older components still use SysEvent, DMCLI, Common Lib and other legacy IPC — must be refactored to RBUS - Test code today often does not exercise IPC interfaces — components tested without validating integration contracts - The current logging approach needs re-examination for operational adequacy |
| Foundation: RDK-B Core is the foundation layer providing essential services for all RDK-B devices. |  |

# 4. Benefits and Value Proposition

Value Proposition RDK-B Core enables easier code reuse, faster development, and consistent behavior across diverse vendors and device types.

| Benefit Area | Specific Benefits |
| --- | --- |
| Modularity, Code Reuse and Scalability | Easier and faster code reuse across device profiles. Minimal build-time dependencies. Flexible runtime dependencies. Code size scales from small extenders to full gateways. Flexible logging and diagnostics. |
| RDK Ready Certification | Formal certification program for SoC vendors. Updated HAL interface definitions. Test code per HAL interface. Mix-and-match SoC sourcing with certified interoperability. |
| Developer Experience | Faster on ramp for new developers. Easier training with consistent APIs. Larger talent pool. Better testability. Higher quality through consistent patterns. |

# 5. RDK-B Adoption and Industry Leadership

| Area | Key Points |
| --- | --- |
| Accelerated Market Adoption | Lower Entry Barriers: easier reuse reduces development time and costs. Broader Device Support: scales from low-cost hardware to full-featured gateways with IoT. Faster Time-to-Market: pre-validated Core reduces integration cycles. Ecosystem Growth: more vendors participate with lower resource requirements. |
| Industry Standardization | Reference Implementation: RDK-B Core as the industry reference for broadband gateway middleware. Compliance and Certification: RDK Ready certifies hardware for rapid adoption. Interoperability: consistent behavior across vendors and platforms. Best Practices: establishes architectural patterns for the connected device industry. |

Strategic Value Positions RDK-B as the standard for next-generation connected broadband devices. Market impact: increase RDK-B deployments across operators and device types globally.

# 6. Roadmap and Next Steps

## 6.1 Next Release — Quick Wins

| Work Area | Actions |
| --- | --- |
| Configuration Options (Distro Flags) | Increase number of Distro Flags to enable/disable features within components. Minimize unnecessary build-time dependencies identified in the dependency audit. |
| RDK Ready Configuration Management | Expand tested HAL interfaces to support vendor certification. Document how to include/exclude optional components in a build. |
| Integrated Testing and Documentation | Ramp up Test Working Group. Auto-discover component data model APIs during testing and document as the API contract. Use component-level versioning for documentation. Define core component and HAL interface tests. |

## 6.2 Future Releases — Structural Program

| Work Area | Actions |
| --- | --- |
| Legacy Code Migration | Define Architecture Standards for new components. Break up monolithic components (Utopia, P&M) into smaller reusable modules. Migrate all components to RBUS (IPC) and PSM (storage). |
| RDK9 Architecture | Define next-generation component architecture eliminating all legacy IPC and storage. Establish module boundaries, interface contracts, and dependency rules. Define migration path from current to RDK9 components. |
| Dependency Management | Full audit of all build-time and runtime dependencies. Resolve all unnecessary dependencies. Maintain published dependency matrix updated per release. |
| Testing Infrastructure | Expand HIL test capacity to cover all certified platforms. Automate full regression across all profiles nightly. Integrate TIP Wi-Fi interoperability validation into CI. |

Expected Outcome RDK-B code becomes cleaner and easier to reuse. Improves time to market and reduces development costs. Increases quality of Core and vendor code. Number of RDK-B deployments increase. Sets the standard for the industry.

# 7. Architecture Standards and Governance

## 7.1 Development Standards

### 7.1.1 Process & Architecture Governance Standards

| Standard | Requirements |
| --- | --- |
| Component IPC Guidelines | All new components must use RBUS APIs exclusively. RBUS API documented as authoritative interface contract. Distro Flags implemented for all sub-features. |
| Dependency Management Policy | Mandatory architecture review for all new inter-component dependencies. Auto-discover data model APIs during testing. Breaking dependency changes require TSC approval for CORE components. |
| Unit Testing Requirements | All new components must ship with isolated unit tests requiring no external dependencies. HAL interfaces must have rdkb-halif-test-* suite before being declared stable. Test coverage metrics published per release. |
| Documentation Standards | Each HAL interface must have a published interface specification document. Each RBUS API documented with parameter types, ranges, and event semantics. Component inclusion/exclusion guides required for all optional components. |
| Data Model Standards (TR-181) | All new parameters register in the TR-181 schema before implementation. Non-standardized parameters use the approved vendor extension prefix (X_RDKCENTRAL-COM_ / X_<ORG>_). Parameter writability, persistence, and notify behavior documented per parameter. |
| Interface Versioning & Backward Compatibility | All RBUS/HAL interfaces follow semantic versioning (MAJOR.MINOR.PATCH). Breaking changes require a major version bump and TSC approval. Deprecated interfaces remain supported a minimum of two release cycles before removal. |
| Security Architecture Standards | All inter-component IPC enforces RBUS ACL-based access control. Credentials/secrets use PSM encrypted storage; plaintext config storage is prohibited. WAN-facing components require a security review before merge. |
| Telemetry & Observability Standards | Components emit logs conforming to the RDK-B common logging format. Critical state transitions publish T2 telemetry markers. Components expose health/status via RBUS for centralized monitoring. |
| Build & Packaging Standards (Yocto/OE) | Components build cleanly under the supported Yocto release with no layer dependency cycles. Recipes declare explicit LICENSE and SPDX identifiers. Distro flags gate all profile-specific or optional features. |
| Error Handling & Resilience Standards | Components use defined RBUS error codes, not custom/undocumented codes. Crash-prone components integrate with systemd watchdog and support clean restart without state corruption. Core dumps captured via the CrashUpload framework. |
| Naming & Repository Conventions | Component repos follow the rdkb-<component> convention. RBUS/TR-181 object naming aligns with Device.X_RDKCENTRAL-COM_ hierarchy. New components follow the Ccsp*/Rdk* prefix and <Function> HAL suffix conventions already established in the architecture. Branch naming aligns with governance branch policy (7.2.12). |
| Code Quality & Static Analysis Standards | New C/C++ components pass static analysis (cppcheck/Coverity) with zero high-severity findings before merge. Coding style conforms to the published RDK-B style guide. Memory safety tooling (ASan/Valgrind) runs in CI for native builds. |
| Layering & Dependency Direction | Calls flow strictly top-down through the layer stack (RemoteMgmt → DeviceMgmt/Ccsp → Wi-Fi/Interface Managers → WAN-specific Managers → HAL → Kernel). A lower layer calling upward into a higher layer is an architecture violation regardless of functional test results. |
| Northbound Protocol Onboarding | Any new remote-management protocol (beyond TR-369/TR-069/WebPA/Webconfig/SNMP/DAC/SSH) must terminate at RBUS via a dedicated adapter, not reach into Ccsp components directly. |
| WAN Technology Onboarding | A new WAN access technology integrates through RdkWanManager plus a technology-specific manager/agent (following the GPON/xDSL/Cellular/Docsis/EPON pattern); bespoke managers outside this pattern require architecture review. |
| Edge AI / ML Integration Standards* | AI/ML inference components use vendor-neutral model formats (ONNX/RKNN), decoupled from any single NPU vendor SDK. Inference latency and resource budgets documented per model. Versioning/rollback supported for production deployment. Layer placement (dedicated color-coded layer vs folded into Ccsp) still needs a decision before this is fully ready. |

### 7.1.2 Implementation & Coding Standards

| Standard | Requirements |
| --- | --- |
| IPC Mechanism Selection | RBUS is the default and only sanctioned IPC for new CORE components. D-Bus is permitted only for legacy/vendor OSS that natively requires it (e.g. ModemManager), bridged into RBUS rather than consumed directly. USP/UDS is reserved for the USP Agent↔Controller boundary only. Any other IPC requires architecture review with documented justification. |
| Eventing Standards | State-change events publish via RBUS event/subscribe, not polling, for any parameter consumed by more than one component. Event payloads carry only the changed parameter/value. High-frequency events (>1/sec) are rate-limited or coalesced at the publisher. |
| Storage & Persistence Standards | Data-model-backed parameters requiring persistence use PSM exclusively. Transient runtime coordination state uses sysevent, not PSM. Structured local storage (logs, model artifacts) uses a defined path with documented size quotas; no writes to rootfs. |
| Footprint / CPU / RAM Budget | Every new component declares static RAM, peak RAM, and flash footprint targets at design review, scoped to its device profile tier. No polling loop faster than a documented minimum interval. Regressions beyond an agreed threshold versus prior release require architecture sign-off. Lower-tier profiles never silently inherit CORE-profile footprint assumptions. |
| system()/Shell-out Usage | Calling system()/popen() from C/C++ daemons is discouraged by default; native library calls are preferred. Where unavoidable, the call checks return codes, sanitizes interpolated input against shell injection, and never runs in a signal handler or high-frequency loop. |
| OSS Usage Standards | OSS dependencies come from the sanctioned Linux OSS layer list; new dependencies require review for license compatibility, CVE history, and maintenance activity. RDK-B wraps OSS via a thin adapter rather than embedding internals directly. Forking OSS instead of upstreaming requires TSC approval plus a reconciliation plan. |
| When to Use HAL | Any interaction with vendor/SoC-specific hardware or firmware goes through a published HAL; direct vendor SDK calls are prohibited. A HAL is required only where behavior differs across silicon vendors — platform-independent behavior belongs in Linux OSS or a native component instead. |
| No RDK-B Calls Within HAL | HAL implementations never call back into Ccsp, RBUS, or any RDK-B framework component — HAL is a leaf layer, invoked top-down only. HAL code contains no TR-181/data-model awareness, no RBUS client code, no sysevent/syscfg calls. |
| Direct Kernel Interaction | Components talk to the kernel directly (syscalls, sysfs, procfs) only for OS-level facilities with no existing HAL/OSS abstraction. Any kernel interface used by more than one component is wrapped in a shared internal library, not duplicated. New kernel module/driver dependencies require platform-layer governance review (7.2.6). |
| Netlink vs Other Kernel Interfaces | Netlink is preferred for asynchronous kernel notifications (link state, routing, neighbor changes); polling sysfs/procfs where a netlink equivalent exists is prohibited. ioctl is permitted only for legacy drivers with no netlink support, isolated behind a HAL/adapter. |
| C/C++ Coding Anti-Patterns | No blocking I/O on a component's main RBUS event-handling thread. strcpy/sprintf and unbounded buffers for variable-length input are banned in favor of bounded equivalents; malloc return values are always checked. Global mutable state shared across translation units is prohibited except through a documented, lock-protected accessor. |
| Component Size Guidance | A component handling more than one distinct responsibility is split along ownership/interface boundaries unless a documented exception is granted. (No hard LOC threshold set yet — flag if you want a specific number enforced in CI.) |
| Sysevent Usage Standards | sysevent is for lightweight, transient, non-data-model signaling between components on the same device — not a substitute for RBUS parameters or PSM persistence. New keys register against the existing sysevent registry rather than introduced ad hoc. Never used for high-frequency telemetry. |
| Scripting Usage Standards | Shell scripts are for one-shot boot/init orchestration, not long-running stateful logic. Interpreted languages are discouraged on memory/flash-constrained profiles unless explicitly justified — native C/C++ is the default for persistent daemons. Boot-time scripts have a bounded execution time and documented timeout. |
| Calling C/C++ from Scripts | Scripts call into native functionality via the component's published CLI (dmcli, rbuscli) or RBUS — never by linking against internal .so symbols outside a published API. Scripts never parse internal binary log/state formats. |
| RFC vs syscfg vs Build Flag | A behavior is RFC-gated if it needs remote, post-deployment toggling without a firmware push. It's a Distro/build flag if profile-fixed at build time. It's a plain syscfg/default if purely local and never needs remote control. Mixing these is a design review finding. |
| Threading & Concurrency Model | New daemons default to a single-threaded event loop unless a specific workload justifies multi-threading. Where threads are used, lock ordering is documented and mutexes are never held across an RBUS call. No unbounded thread spawning per request/connection. |
| Logging Level Standards | Components use rdk_logger exclusively, never raw printf/syslog. ERROR is reserved for conditions requiring operator/telemetry attention; DEBUG/TRACE are compiled out or runtime-gated in production builds. |
| Shared Library ABI Versioning | Any .so consumed by more than one component follows symbol versioning; a breaking ABI change requires a SONAME bump and follows the same major-version/TSC-approval path as Interface Versioning. |

### 7.1.3 Industry Standards Conformance

Where Core RDK-B functionality overlaps with an established global standard, the implementation conforms to that standard's normative requirements rather than introducing a proprietary deviation. Deviations require documented justification and architecture review.

#### 7.1.3.1.  Remote management & data modeling

| Standard / Body | Applies To | Note |
| --- | --- | --- |
| Broadband Forum TR-069 (CWMP) | ACS Server, TR-069 adapter | Legacy remote management protocol |
| Broadband Forum TR-369 (USP) | USP Controller, TR-369 adapter | Transport-agnostic (WebSocket/MQTT/STOMP/CoAP) |
| Broadband Forum TR-181 Device:2 Data Model | All TR-181-backed parameters (WebPA, Webconfig, TR-069, USP) | Single underlying data model across northbound protocols |
| IETF RFC 3411–3418 (SNMPv3) | SNMP Server adapter | SNMPv1/v2c retained only for legacy compatibility |
| OpenAPI 3.x | Any REST API surface (RestAPI-tagged USP/WebPA paths) | Required for documenting REST interfaces |

#### 7.1.3.2. Wireless & local networking

| Standard / Body | Applies To | Note |
| --- | --- | --- |
| IEEE 802.11ax / 802.11be | OneWifi, rdk-wifi-hal, Vendor Wi-Fi HAL | Wi-Fi 6/6E/7 PHY-MAC conformance |
| Wi-Fi Alliance EasyMesh R6 | Airties easymesh controller, OneWifi multi-AP | Multi-AP interoperability certification |
| Wi-Fi Alliance WPA3 / WPS Certification | WPS box, Wi-Fi security stack |  |
| IEEE 802.1Q | RDKVlanBridgingManager, VLAN utils, VLAN HAL | VLAN tagging for ISP/service separation |
| IEEE 802.3 | Ethernet switch HAL | Base Ethernet conformance |
| Bluetooth SIG Core Specification | Bluetoothctl, Bluetooth HCI |  |

#### 7.1.3.3. IoT & smart home standards (orchestrated via BartonCore — present in the platform, not yet depicted in the architecture diagram)

| Standard / Body | Applies To | Note |
| --- | --- | --- |
| Thread Group Thread Specification | BartonCore (Thread network layer) |  |
| Connectivity Standards Alliance (CSA) Matter Specification | BartonCore (Matter controller/bridge) |  |
| CSA Zigbee Specification | BartonCore (Zigbee bridge) | Legacy device compatibility |
| IEEE 802.15.4 | Underlying radio layer for Thread/Zigbee within BartonCore |  |
| Device.IoT.* TR-181 (WT-480 pathway) | barton-rbus-provider | Bridges BartonCore into RBUS/TR-181 |

#### 7.1.3.4. Core network services (DNS/DHCP/PPP)

| Standard / Body | Applies To | Note |
| --- | --- | --- |
| IETF RFC 1035 (DNS) | DNS proxy (Linux OSS), CcspXdns | CcspXdns is RDK-B's DNS-based filtering/redirect feature, not a separate protocol |
| IETF RFC 6762/6763 (mDNS/DNS-SD) | Local service discovery, where used by OneWifi/EasyMesh | Confirm actual usage before citing as a hard requirement |
| IETF RFC 2131 (DHCPv4) | Dhcp Manager, DHCP V4 HAL, Dnsmasq |  |
| IETF RFC 8415 (DHCPv6) | DHCP V6 HAL, Dibbler |  |
| IETF RFC 1661/1662 (PPP) + RFC 2516 (PPPoE) | RDKPPPManager | Relevant for DSL/fiber ISPs using PPP-based WAN authentication |
| IETF RFC 3376 (IGMPv3) | IGMP Proxy |  |
| UPnP Forum IGD Spec (now under Open Connectivity Foundation) | Upnp box |  |

#### 7.1.3.5. IPv6 transition mechanisms

| Standard / Body | Applies To | Note |
| --- | --- | --- |
| IETF RFC 7597/7599 (MAP-T/MAP-E) | MAP-T/E box | Ties to your validated BPI R4 kernel patch work |
| IETF RFC 6333 (DS-Lite) | Dslite box |  |

#### 7.1.3.6. Access network / WAN technologies

| Standard / Body | Applies To | Note |
| --- | --- | --- |
| ITU-T G.984 / G.988 (GPON, OMCI) | RdkGPONManager, GPON Headend (OLT) |  |
| ITU-T G.9807.1 (XGS-PON) | Forward-looking GPON evolution |  |
| IEEE 802.3ah / 802.3av (EPON / 10G-EPON) | CcspEponAgent, EPON Headend | EPON is IEEE, not ITU-T — distinct from GPON's standards body |
| ITU-T G.993.2 (VDSL2) / G.9701 (G.fast) | RdkXdslManager, XDSL HAL, DSL Headend (DSLAM) |  |
| CableLabs DOCSIS 3.1/4.0 + eRouter Specification | CcspCMAgent, Docsis Headend (CMTS) | eRouter spec is also the origin of the erouter0 interface naming used in your WAN bridge work |
| CableLabs PacketCable | CcspMTAAgent, MTA HAL | Voice over cable |
| 3GPP TS 27.007 (AT command set) | RDKCellularManager-MM, Cellular Modem HAL, ModemManager/libqmi | Independent of RAN generation (3G/4G/5G) |
| USB-IF MBIM Specification | libmbim | Mobile Broadband Interface Model |
| MoCA Alliance MoCA 2.0/2.5 | CcspMoca, MoCA HAL |  |

#### 7.1.3.7. Boot chain, kernel & firmware integrity

| Standard / Body | Applies To | Note |
| --- | --- | --- |
| Linux Kernel (GPLv2, kernel.org/Linux Foundation governance) | Linux Kernel & Device Drivers layer | UAPI stability commitment governs what RDK-B can rely on across kernel versions |
| U-Boot (GPLv2, Denx community) | Bootloader layer |  |
| Arm SystemReady (IR/SR profiles) + EBBR (Embedded Base Boot Requirements) | Bootloader layer, boot chain standardization | Standardizes the boot contract (U-Boot+EBBR for embedded profiles) for interoperability across Arm SoCs; complements PSA Certified, which covers security rather than boot interface |
| NIST SP 800-193 (Platform Firmware Resiliency Guidelines) | Bootloader → kernel verification chain | Detection/protection/recovery model for firmware integrity |
| Arm PSA Certified / TrustZone | Secure boot chain on ARM-based reference platforms (e.g. MT7988/BPI R4) | Security/attestation, distinct from SystemReady's boot-interface scope |
| GlobalPlatform TEE Client/Internal API Specifications | Where a TEE holds boot/signing keys | Only applies if the SoC's secure boot path uses a TEE rather than ROM-only verification |

#### 7.1.3.8. Cryptographic signing for secure boot

| Standard / Body | Applies To | Note |
| --- | --- | --- |
| NIST FIPS 186-5 (Digital Signature Standard — covers ECDSA) | Bootloader/kernel/firmware image signature verification |  |
| SEC 1 / SEC 2 (Certicom Standards for Efficient Cryptography) | Curve parameter selection (e.g. NIST P-256/secp256r1) | Defines the actual EC curves referenced by FIPS 186-5 |
| IETF RFC 8032 (EdDSA/Ed25519) | Alternative signing scheme | Increasingly used over ECDSA for boot-time verification due to faster, simpler constant-time implementation |

#### 7.1.3.9. Containers & application platform

| Standard / Body | Applies To | Note |
| --- | --- | --- |
| OCI Image Spec & Runtime Spec | crun, DAC, Dobby/BundleGen | Ties to your libbundlegen/Dobby OCI work |

#### 7.1.3.10. Security & coding standards

| Standard / Body | Applies To | Note |
| --- | --- | --- |
| MISRA-C:2012 / CERT C Secure Coding Standard | C/C++ coding anti-pattern enforcement (7.1.2) | External normative anchor for the internal coding rule |
| IETF RFC 8446 (TLS 1.3) / RFC 5280 (X.509 PKI) | WebPA/USP/ACS transport security, secure boot certs |  |
| NIST SP 800-57 (key management) | PSM-backed credential storage |  |
| OWASP IoT Top 10 | General embedded device security review checklist |  |

## 7.2 Technical Governance Process

### 7.2.1. Purpose

The purpose of Core RDK Broadband governance is to ensure that every new component, enhancement, bug fix, interface change, and platform integration follows a common technical standard before it is accepted into the Core RDK-B codebase.

The governance process must ensure:

- Core architecture consistency

- Platform portability

- Generic implementation suitable for community adoption

- Clear ownership and review responsibility

- Mandatory validation before merge

- Test evidence availability for every change

- Controlled release inclusion and traceability

No code change should be merged into a Core branch unless architecture, interface, dependency, test, and release expectations are satisfied.

### 7.2.2. Governance Scope

This process applies to:

- New component development

- Existing component enhancement

- Bug fixes

- HAL, Linux OSS, or platform-layer interaction

- Interface changes

- Dependency changes

- Build system changes

- Reference platform contributions

- Community contributions

- Release-bound changes

### 7.2.3. Classification of Changes

Every change must be classified before development starts.

#### 7.2.3.1 New Component

A new component is any new daemon, library, service, agent, CLI, data model provider, HAL wrapper, telemetry provider, or platform abstraction introduced into Core RDK-B.

Examples:

- New RDK-B component

- New TR-181 provider

- New USP/TR-369 provider

- New telemetry producer

- New platform abstraction module

- New diagnostics module

- New service manager or helper daemon

#### 7.2.3.2 Enhancement

An enhancement modifies or extends an existing component without changing the basic ownership of the component.

Examples:

- New feature inside existing component

- New data model parameter

- New event or telemetry marker

- New configuration support

- New runtime behavior

- New platform capability support

- New interface usage

#### 7.2.3.3 Bug Fix

A bug fix corrects an existing defect without introducing new feature behavior.

Examples:

- Crash fix

- Memory leak fix

- Functional defect fix

- Race condition fix

- Regression fix

- Build failure fix

- Test failure fix

- Platform compatibility fix

Bug fixes must still be generic and community-aligned. Local hacks, platform-specific shortcuts, or hidden behavior under flags should not be accepted unless formally justified.

### 7.2.4. Governance Entry Criteria

Before development starts, the contributor must create a design/review ticket with the following information.

#### 7.2.4.1 Mandatory Inputs for New Component

A new component proposal must include:

- Component name

- Problem statement

- Functional scope

- Non-functional scope

- Ownership

- Runtime process model

- Build-time dependency list

- Runtime dependency list

- Data model impact

- RBUS/CCSP/USP/TR-181 impact

- Telemetry impact

- Logging impact

- Security impact

- Memory and CPU expectations

- Boot-time impact

- Restart/recovery behavior

- HAL or Linux OSS interaction model

- Platform portability plan

- Test plan

- Release target

- Rollback plan

#### 7.2.4.2 Mandatory Inputs for Enhancement

An enhancement proposal must include:

- Existing component impacted

- Feature description

- Interface impact

- Data model impact

- Dependency impact

- Backward compatibility assessment

- Platform impact

- Test plan

- Release target

- Regression risk

#### 7.2.4.3 Mandatory Inputs for Bug Fix

A bug fix proposal must include:

- Defect description

- Reproduction steps

- Logs or evidence

- Root cause analysis

- Fix approach

- Regression impact

- Platforms impacted

- Test evidence

- Whether the fix is generic or platform-specific

- Whether the issue exists in stable branch, rdk-next, or both

### 7.2.5. Architecture Review Rules

All new component contributions and major enhancements require architecture review before implementation or merge.

Architecture review must validate:

- Whether the component belongs in Core RDK-B

- Whether the component duplicates existing functionality

- Whether the component follows Core design principles

- Whether the dependency model is acceptable

- Whether platform-specific logic is isolated correctly

- Whether HAL/Linux OSS access is done through approved layers

- Whether interfaces are stable and backward compatible

- Whether the component can run across reference platforms

- Whether the component is testable in CI and on hardware

TSC approval is mandatory for:

- New Core-classification component

- Breaking interface change

- New mandatory dependency

- New HAL contract

- Data model behavior change

- Release-blocking feature

- Change impacting multiple components

- Change impacting community platform portability

### 7.2.6. Platform Layer Invocation Rules

Components must not directly embed board-specific logic inside Core implementation.

#### 7.2.6.1 Preferred Access Order

A Core component should access platform functionality in the following order:

- Standard Linux OSS interface

- Existing RDK-B abstraction or service

- Approved HAL interface

- Vendor/platform plugin

- Platform-specific fallback only when formally approved

#### 7.2.6.2 Linux OSS Access

Use Linux OSS interfaces when the functionality is standard and portable.

Examples:

- netlink

- sysfs

- procfs

- ubus only if platform-approved

- iproute2-compatible behavior

- standard kernel networking APIs

- standard systemd/D-Bus interaction where applicable

Linux OSS usage must be validated across supported kernel versions and reference platforms.

#### 7.2.6.3 HAL Access

Use HAL when:

- Hardware-specific behavior is required

- Vendor implementation may differ

- Acceleration path is SoC-specific

- Radio, switch, PHY, WAN, LAN, or hardware diagnostics behavior is not portable through standard Linux APIs

HAL usage must follow these rules:

- HAL API must be generic

- HAL must not expose vendor-private implementation details

- HAL header must be reviewed before implementation

- HAL behavior must be documented

- HAL failure handling must be defined

- Stub/reference implementation must be available where possible

- Component must degrade gracefully if capability is not supported

#### 7.2.6.4 Direct Vendor Calls

Direct vendor SDK/API calls inside Core component code are not allowed unless formally approved.

If vendor-specific logic is unavoidable, it must be isolated in:

- HAL implementation

- Platform adapter

- Vendor plugin

- Board-specific layer

- Compile-time optional backend with clear justification

### 7.2.7. Dependency Governance

Every component must declare and justify:

- Build-time dependencies

- Runtime dependencies

- Optional dependencies

- Platform-specific dependencies

- Service startup dependencies

- Library dependencies

- Package dependencies

New dependencies must be reviewed for:

- Portability

- License compatibility

- Security risk

- Maintenance ownership

- Image size impact

- Boot-time impact

- Runtime memory impact

- Impact on minimal Core builds

No new mandatory dependency should be added to Core unless it is required for generic broadband functionality.

### 7.2.8. Development Flow

#### 7.2.8.1 New Component Flow

The required flow is:

- Create architecture proposal ticket

- Complete architecture review

- Identify component owner and reviewers

- Define interface and dependency contract

- Define HAL/Linux OSS interaction model

- Define test plan

- Develop implementation

- Run local build and unit tests

- Run platform validation

- Submit PR

- Run CI checks

- Complete peer review

- Complete TSC/Core review if required

- Attach test evidence

- Merge only after sanity pass

- Track release inclusion

#### 7.2.8.2 Enhancement Flow

The required flow is:

- Create enhancement ticket

- Document design and impact

- Review interface and dependency changes

- Confirm backward compatibility

- Develop change

- Run unit, functional, and regression tests

- Submit PR

- Attach test evidence

- Complete peer/component owner review

- Complete TSC approval if Core behavior changes

- Merge after sanity pass

- Include in release notes if user-visible or interface-impacting

#### 7.2.8.3 Bug Fix Flow

The required flow is:

- Create defect ticket

- Attach reproduction evidence

- Complete root cause analysis

- Confirm affected platforms and branches

- Implement generic fix

- Validate original failure scenario

- Run regression tests

- Submit PR

- Attach before/after evidence

- Merge only after sanity pass

- Decide whether fix is needed in stable branch, rdk-next, or both

### 7.2.9. Testing Governance

Testing is mandatory for every new component, enhancement, and bug fix.

No code change should be merged if required sanity testing fails.

#### 7.2.9.1 Test Types Required

Each change must be evaluated against the following test categories.

Build Validation

- Clean build

- Incremental build

- Component build

- Image build

- Package dependency validation

- Yocto recipe validation

- Static build dependency check

Unit Testing

- Function-level test

- Error path test

- Boundary condition test

- Invalid input test

- API contract test

- Mock HAL test if HAL is involved

Functional Testing

- Feature enable/disable

- Basic workflow

- Runtime behavior

- Service start/stop/restart

- Configuration persistence

- Data model get/set validation

- RBUS/CCSP/USP validation where applicable

- CLI/API validation where applicable

Integration Testing

- Interaction with dependent services

- Boot order validation

- Systemd dependency validation

- Sysevent/RBUS/event validation

- Telemetry/logging validation

- HAL/Linux OSS integration validation

- Upgrade/reboot persistence

Regression Testing

- Existing feature sanity

- Component regression

- Impacted component regression

- Data model regression

- Boot sanity

- Connectivity sanity

- Memory/process sanity

Negative Testing

- Dependency missing

- HAL failure

- Invalid configuration

- Permission failure

- Interface unavailable

- Process crash/restart

- Unsupported platform capability

Performance Testing

- CPU usage

- Memory usage

- Boot-time impact

- Runtime latency

- Event storm behavior

- Log volume

- Telemetry volume

Security Testing

- Input validation

- Command injection risk

- File permission validation

- Secret handling

- Privilege usage

- Network exposure

- API authentication/authorization where applicable

Stability Testing

- Reboot validation

- Process restart validation

- Long-run validation

- Memory leak check

- Crash check

- Log flooding check

### 7.2.10. Testing Expectations by Change Type

#### 7.2.10.1 New Component

Minimum required evidence:

- Successful image build

- Successful component start/stop/restart

- Functional test results

- Unit test results

- Integration test results

- Reboot test results

- Memory and CPU observation

- Negative test results

- Logs showing normal behavior

- Data model/API validation if applicable

- Platform validation report

- CI result link

- Known limitations

#### 7.2.10.2 Enhancement

Minimum required evidence:

- Build pass

- Existing functionality regression pass

- New functionality test pass

- Interface backward compatibility result

- Impacted component sanity result

- Logs before and after change

- Platform validation result

- CI result link

#### 7.2.10.3 Bug Fix

Minimum required evidence:

- Original issue reproduction evidence

- Root cause analysis

- Fix validation evidence

- Before/after logs

- Regression test result

- Impacted platform validation

- Confirmation that fix is generic

- CI result link

### 7.2.11. Platform Test Matrix

Each change must define which platforms were tested and why.

#### 7.2.11.1 Minimum Platform Validation

At minimum, Core RDK-B changes should be validated on:

- Primary reference platform

- One active hardware reference platform

- Emulated or CI platform where available

Example platform categories:

- qemux86broadband or equivalent CI reference

- Raspberry Pi or generic reference platform

- Banana Pi R4 / Filogic / active broadband reference hardware

- Vendor platform if the change is hardware-specific

#### 7.2.11.2 Platform Selection Rules

Test on hardware platform when the change impacts:

- WAN

- LAN

- Wi-Fi

- Switch

- PHY

- HAL

- Kernel interface

- Boot sequence

- Telemetry

- TR-181 runtime behavior

- USP/TR-369 runtime behavior

- System service behavior

Test on CI/emulated platform when the change impacts:

- Generic component logic

- Build system

- Data model provider

- Library behavior

- Static dependency

- Configuration parsing

- Non-hardware-specific functionality

#### 7.2.11.3 Hardware-Specific Changes

For hardware-specific changes:

- Validate on the target hardware

- Validate that generic platforms are not broken

- Isolate platform logic outside Core when possible

- Provide fallback behavior for unsupported capability

### 7.2.12. Branch and Build Governance

Every change must state which branch/build was validated.

#### 7.2.12.1 rdk-next

Use rdk-next for:

- New component development

- New feature development

- Major enhancement

- Interface change

- New dependency

- HAL contract update

- Experimental Core work

- Community-first development

#### 7.2.12.2 Stable Working Branch

Use stable branch for:

- Critical bug fixes

- Release-bound fixes

- Regression fixes

- Security fixes

- Operator-required fixes

- Backport candidates

#### 7.2.12.3 Testing Rule

For new Core development:

- Validate first on rdk-next

- Validate release need on target stable branch if required

- Backport only after fix/feature is stable and approved

For bug fixes:

- Validate on the branch where issue was found

- Validate on rdk-next if the code exists there

- Validate backport branch if release requires it

### 7.2.13. Review and Approval Model

#### 7.2.13.1 Where to Raise Reviews

All reviews must be tracked using:

- GitHub issue for feature/defect tracking

- GitHub PR for code review

- Project board for release tracking

- Architecture review record for new Core component or major change

- TSC review record for Core-classification or breaking changes

#### 7.2.13.2 Required Reviewers

Reviewers should include:

- Component owner

- Peer reviewer

- Platform/reference platform owner

- Architecture owner for new component or major enhancement

- HAL owner if HAL is impacted

- Test owner or QA representative

- Release engineer for release-bound changes

- TSC for Core-classification or breaking interface changes

#### 7.2.13.3 Review Checklist

Reviewers must confirm:

- Design is generic

- Code follows Core standards

- Dependency model is acceptable

- No unnecessary platform-specific logic exists

- HAL/Linux OSS usage is correct

- Backward compatibility is preserved

- Error handling is complete

- Logs are useful but not excessive

- Telemetry is meaningful

- Security risks are addressed

- Tests are sufficient

- CI passed

- Test evidence is attached

- Release impact is documented

### 7.2.14. Merge Criteria

A change can be merged only when all applicable conditions are met.

#### 7.2.14.1 Mandatory Merge Criteria

- PR is linked to issue/ticket

- Architecture review completed if required

- TSC approval completed if required

- CI checks passed

- Build validation passed

- Required sanity tests passed

- Required platform tests passed

- Test evidence attached

- No unresolved critical review comments

- No unexplained functional test failures

- Release impact documented

- Rollback plan documented for high-risk changes

#### 7.2.14.2 Blocking Conditions

Code must not be merged if:

- Sanity test failed

- CI failed without approved exception

- Functional test failure has no root cause

- Test evidence is missing

- Dependency impact is not reviewed

- HAL/interface change is not approved

- Change is hidden under a flag without justification

- Fix is platform-specific but placed in Core generic code

- Release engineer cannot confirm primary expectation is met

### 7.2.15. Test Evidence Requirements

All test results must be attached to the ticket or PR.

Evidence must include:

- Test summary

- Test case list

- Pass/fail status for each test case

- Logs for important test cases

- Failure logs where applicable

- Platform details

- Build details

- Branch details

- Image version

- Commit ID

- Date of validation

- Tester name or automation job link

- Known issues

- Deviations or exceptions

For every failed functional test:

- A development ticket must be created

- Root cause analysis must be provided

- Fix must be generic enough for community alignment

- Release impact must be assessed

No release-bound change should be accepted with unexplained failures.

### 7.2.16. Release Governance

#### 7.2.16.1 Release Inclusion Criteria

A change can be included in release only when:

- Feature/bug ticket is closed or release-approved

- PR is merged into correct branch

- CI passed

- Required platform validation passed

- Test evidence is available

- Release engineer confirms primary expectation is met

- Known issues are documented

- Release notes are updated if needed

#### 7.2.16.2 Release Notes

Release notes must include:

- New component details

- Enhancement summary

- Bug fix summary

- Interface changes

- Data model changes

- HAL changes

- Dependency changes

- Migration steps if any

- Known limitations

- Test platforms

- Validation summary

#### 7.2.16.3 Release Timing

Changes should be released based on classification:

New Component

Release in planned major/minor release after full validation.

Enhancement

Release in planned feature release unless it is required for a committed operator/customer milestone.

Bug Fix

Release in next maintenance release or patch release depending on severity.

Critical/Security Fix

Release through expedited patch process after mandatory validation and approval.

### 7.2.17. Flag Governance

No code change should be accepted under a flag unless truly required.

Flags are allowed only when:

- Feature is optional by design

- Platform capability is not universally available

- Migration requires staged rollout

- Release risk needs controlled enablement

- Backward compatibility requires coexistence

Flags are not allowed for:

- Hiding incomplete code

- Avoiding proper architecture review

- Avoiding test failures

- Carrying platform-specific hacks

- Bypassing community alignment

Every flag must have:

- Clear owner

- Default behavior

- Documentation

- Test coverage for enabled and disabled modes

- Removal plan if temporary

### 7.2.18. Reference Platform and Community Contribution Rules

All community contributions and Reference Platform team changes must be validated before merge.

Required expectations:

- Code must build on reference platform

- Core sanity must pass

- Functional validation must be attached

- No reference-only assumption should enter Core

- Platform-specific changes must remain in platform layer

- Generic code must remain portable

- Test evidence must be available in ticket/PR

### 7.2.19. Governance Checklist

#### 7.2.19.1 New Component Checklist

- Architecture proposal created

- Component ownership defined

- Core classification confirmed

- Dependency list reviewed

- Interface contract reviewed

- HAL/Linux OSS usage reviewed

- Data model impact documented

- Security impact reviewed

- Telemetry/logging plan reviewed

- Test plan approved

- CI passed

- Reference platform tested

- Hardware platform tested if applicable

- Reboot/restart tested

- Negative testing completed

- Test evidence attached

- TSC approval completed if required

- Release plan documented

#### 7.2.19.2 Enhancement Checklist

- Enhancement ticket created

- Impacted component identified

- Backward compatibility checked

- Interface/data model impact documented

- Dependency impact reviewed

- Unit/functional tests passed

- Regression tests passed

- Platform validation completed

- Test evidence attached

- Release notes updated if required

#### 7.2.19.3 Bug Fix Checklist

- Defect ticket created

- Reproduction steps attached

- Root cause analysis completed

- Fix approach reviewed

- Before/after evidence attached

- Regression testing completed

- Branch impact checked

- Backport need assessed

- Test evidence attached

- Release inclusion decision made

### 7.2.20. Final Governance Rule

Core RDK Broadband changes must be generic, portable, testable, and community-aligned.

No code change should be merged or released unless:

- The primary expectation is met

- Sanity testing is passed

- Test evidence is attached

- Failures are understood and tracked

- Review ownership is clear

- Release impact is documented

- The implementation aligns with Core RDK-B architecture principles

## 7.3 Component Governance Process

### 7.3.1 Purpose

The purpose of Component Governance is to ensure that every Core RDK-B component has a defined owner, a known lifecycle state, a health baseline, and a clear exit path. While Section 7.2 governs how changes are introduced and validated, Section 7.3 governs what happens to a component across its entire life — from registration through active maintenance to deprecation and removal.

Component governance must ensure:

- Every Core RDK-B component has a registered owner at all times

- Component lifecycle state is known and current

- Orphaned or unhealthy components are identified and resolved

- Deprecated components have a defined migration path

- HAL contracts and TR-181 parameters are transferred cleanly when components change state

- Community contributors and operators have visibility into component health and roadmap

### 7.3.2 Component Registry

Every component accepted into Core RDK-B must be registered in the Component Registry at merge time. The registry is the authoritative record of all active, deprecated, and removed components.

#### 7.3.2.1 Mandatory Registry Fields

Each component entry must include:

- Component name

- Primary GitHub repository

- Supporting repositories (if any)

- Core classification (CORE / Required / Optional)

- Current lifecycle state (Active / Stable / Deprecated / Removed)

- Primary owner (name and organization)

- Secondary owner (name and organization)

- Primary interfaces (RBUS / HAL / USP / TR-181)

- HAL contract reference (if applicable)

- TR-181 data model scope

- Last validated release

- Known open critical issues

- Distro flag(s) controlling inclusion

#### 7.3.2.2 Registry Maintenance Rules

- Registry must be updated at every release

- Any change to owner, state, or interfaces must trigger a registry update

- TSC is responsible for registry accuracy

- Registry must be publicly accessible to the RDK community

### 7.3.3 Component Lifecycle States

Every Core RDK-B component must be in exactly one lifecycle state at all times.

| State | Meaning |
| --- | --- |
| Active | Component is under active development and maintenance. New features, enhancements, and bug fixes are accepted. Full governance process applies. |
| Stable | Component interface is frozen. Only critical bug fixes and security fixes are accepted. No new features. HAL contract is locked. |
| Deprecated | Component is scheduled for removal. No new features or enhancements accepted. Critical security fixes only. Migration path is defined and published. |
| Removed | Component is no longer part of Core RDK-B. Repository archived. No changes accepted. |

#### 7.3.3.1 State Transition Rules

- Active → Stable: Requires architecture owner approval. Interface freeze declared. HAL contract version locked.

- Stable → Deprecated: Requires TSC approval. Migration path must be defined before transition.

- Deprecated → Removed: Requires TSC approval. Migration path must be confirmed complete. Operator notification period must have elapsed.

- Any State → Active: Requires TSC approval if previously Stable or Deprecated.

### 7.3.4 Component Ownership Model

Every Core RDK-B component must have a defined primary owner and a secondary owner at all times.

#### 7.3.4.1 Owner Responsibilities

The primary owner is responsible for:

- Architecture decisions for the component

- Review and approval of all PRs touching the component

- Maintaining the component's test suite

- Representing the component in TSC and architecture reviews

- Keeping the registry entry current

- Participating in periodic health reviews

- Defining the migration plan if deprecation is required

#### 7.3.4.2 Ownership Transfer Process

When a primary owner is no longer available:

- Secondary owner automatically assumes primary responsibility

- A new secondary owner must be nominated within one release cycle

- If no secondary owner exists, the component is escalated to TSC for ownership resolution

- TSC may assign ownership to another organization, open it to community volunteers, or initiate deprecation if no owner is found

#### 7.3.4.3 Orphan Resolution

A component is considered orphaned when:

- Primary and secondary owner are both unavailable

- No PR reviews have occurred in two consecutive release cycles

- Health review has not been completed for one year

Orphaned components are escalated to TSC. TSC must resolve ownership or initiate deprecation within one release cycle.

### 7.3.5 Periodic Health Review

Every Core RDK-B component classified as CORE or Required must undergo a health review once per release cycle.

#### 7.3.5.1 Health Review Criteria

The health review must confirm:

- Ownership is current and active

- Component builds cleanly on the current reference platform

- CI is passing

- Test suite is current and passing

- No unresolved critical or security defects older than one release cycle

- HAL contract is current and matches active vendor implementations

- TR-181 data model scope is accurate

- Registry entry is up to date

- No known community-reported regressions are unaddressed

#### 7.3.5.2 Health Review Outcome

| Outcome | Action |
| --- | --- |
| Healthy | No action required. Registry updated with review date. |
| At Risk | Owner must submit a remediation plan within 30 days. |
| Unhealthy | Escalated to TSC. TSC decides: remediation, ownership transfer, or deprecation. |

#### 7.3.5.3 Health Review Responsibility

- Component owner conducts the review

- Architecture owner validates the result

- TSC reviews all At Risk and Unhealthy outcomes

#### 7.3.6 Deprecation and Migration Policy

#### 7.3.6.1 Deprecation Declaration

A component may be deprecated when:

- Functionality is superseded by a newer component

- Component cannot meet Core architecture standards and remediation is not feasible

- No active owner can be identified

- TSC determines the component no longer aligns with Core RDK-B goals

Deprecation requires:

- TSC approval

- Migration path defined before deprecation is declared

- Operator notification published in release notes

- Minimum one full release cycle between Deprecated state and Removed state

#### 7.3.6.2 Migration Path Requirements

A migration plan must define:

- Replacement component or approach

- Step-by-step migration guide for operators and integrators

- Timeline for migration

- Support period during coexistence (deprecated component + replacement)

- Test coverage for the migration path

#### 7.3.6.3 HAL Contract Handoff

When a deprecated component owns a HAL contract:

- HAL contract must be transferred to the replacement component before removal

- HAL contract version must be incremented

- Vendor re-validation must be required against the new contract owner

- Transition period must be defined allowing vendors to update implementations

#### 7.3.6.4 TR-181 Parameter Transfer

When a deprecated component owns TR-181 parameters:

- Parameters must be transferred to the replacement component

- Data model backward compatibility must be maintained during the transition period

- Breaking data model changes require TSC approval and operator notification

#### 7.3.6.5 Removal Criteria

A component can move to Removed state only when:

- TSC approval is obtained

- Migration path is confirmed complete by at least one reference platform

- Operator notification period has elapsed

- HAL contract handoff is complete

- TR-181 parameter transfer is complete

- Repository is archived and marked as removed in registry

### 7.3.7 Interface Stability Tagging

Every HAL interface and RBUS API owned by a Core component must carry an explicit stability tag.

#### 7.3.7.1 Stability Levels

| Tag | Meaning |
| --- | --- |
| Experimental | Interface is under active design. Breaking changes may occur without notice. Not for production use. |
| Stable | Interface is production-ready. Breaking changes require TSC approval and versioning. |
| Frozen | Interface will not change. Only bug fixes to documentation or behavior clarifications permitted. |
| Deprecated | Interface is scheduled for removal. Migration path is defined. No new consumers should adopt it. |

#### 7.3.7.2 Tagging Requirements

- All HAL headers must carry a stability tag in the file header comment

- All RBUS APIs must carry a stability tag in the API documentation

- Tag changes must be reviewed by the architecture owner

- Downgrade from Stable to Experimental is not permitted without TSC approval

#### 7.3.7.3 Freeze and Unfreeze Process

- Interface freeze requires architecture owner declaration and registry update

- Unfreeze (Frozen → Stable) requires TSC approval and a versioning increment

- Frozen interfaces that need behavioral change must be versioned as a new interface, not modified in place

### 7.3.8 Component Governance Checklist

For every release cycle, the component owner must confirm:

- Health review completed

- Registry entry current

- Ownership confirmed

- CI passing

- Test suite current

- No unresolved critical defects

- HAL contract current

- TR-181 scope accurate

- Interface stability tags current

- Deprecation or migration actions on track (if applicable)

## 7.4 Escalation Process

| Step | Escalation Stage | When to Use | Escalation Owner | Required Participants | Required Evidence / Inputs | Expected Decision / Output | Exit Criteria |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Identify Blocker | Change is blocked due to architecture, review, test, platform, merge, or release issue | Contributor / PR Owner | Contributor | Problem summary, impacted component, PR/ticket link | Confirm blocker type | Blocker is clearly identified |
| 2 | Record Issue in Ticket / PR | Any blocker needs formal tracking | Contributor / PR Owner | Contributor, Component Owner if needed | Logs, test results, branch, build, platform, failure details, expected decision | Ticket/PR updated with escalation details | Ticket/PR has enough information for review |
| 3 | Level 1: Component Owner Review | Implementation issue, component design question, unclear fix approach | Component Owner | Contributor, Peer Reviewer | Code change, design note, unit/functional test result | Approve, request rework, request more tests, or escalate | Component-level decision recorded |
| 4 | Level 2: Platform / Test Review | Platform behavior issue, HAL/Linux OSS concern, test failure, missing validation | Platform Owner or Test Owner | Component Owner, Platform Owner, Test Owner, Contributor | Platform logs, test report, HAL/Linux OSS behavior, failed test details | Resolve, request platform validation, create defect, or escalate | Platform/test decision recorded |
| 5 | Level 3: Core Architecture Review | Core design disagreement, Core vs platform boundary issue, dependency/interface dispute | Core Architecture Owner | Component Owner, Platform Owner, HAL Owner if applicable, Test Owner, Contributor | Architecture proposal, dependency impact, interface contract, portability assessment | Approve design, reject design, request modification, or escalate to TSC | Architecture decision recorded |
| 6 | Level 4: TSC Review | Core-classification change, breaking interface change, major dependency, unresolved architecture conflict | TSC Chair / TSC Delegate | Core Architecture Owner, Component Owner, Release Engineer if impacted, Platform Owner, Contributor | Architecture decision, risk summary, test evidence, interface/dependency impact | Approve, reject, approve with conditions, defer, or escalate | TSC decision recorded |
| 7 | Level 5: Release Governance | Release inclusion, release blocker, backport, waiver, release cutoff issue | Release Engineer / Release Manager | TSC Delegate, Component Owner, Test Owner, Platform Owner, Contributor | Release ticket, validation summary, known issues, test evidence, waiver request if any | Include, defer, backport, reject, approve waiver, or escalate | Release decision recorded |
| 8 | Document Final Decision | Any escalation reaches a decision | Current Escalation Owner | All required stakeholders | Decision summary, owner, risk, evidence reviewed, missing validation, follow-up actions | Final decision documented in PR/ticket | Decision is traceable |
| 9 | Execute Decision | Code/test/release action is required after decision | Contributor + Component Owner + Test Owner | Contributor, Component Owner, Test Owner, Release Engineer if needed | Updated code, new test result, updated release notes, follow-up tickets | Complete required action | Required actions completed |
| 10 | Close Escalation | Escalation decision and actions are complete | Current Escalation Owner | Contributor, Component Owner, Test Owner, Release Engineer if needed | Closure summary, final evidence, release impact, follow-up tickets | Close escalation | Escalation closed with evidence |
| 11 | Organization Leadership Escalation | Issue remains unresolved after Step 10, or has business, ownership, resource, operator/customer, or cross-organization impact | Organization Leadership Owner | TSC Chair/Delegate, Release Manager, Core Architecture Owner, Component Owner, Platform Owner, Test Owner, Program Manager, Contributor if needed | Full escalation history, unresolved risks, business/release impact, ownership gap, resource need | Assign final owner, priority, release decision, waiver decision, target closure date | Leadership decision recorded and issue closed or converted to leadership-tracked action |

Expected Outcome RDK-B code becomes cleaner and easier to reuse. Improves time to market. Increases quality of Core and vendor code. New developers onboard faster with consistent, documented patterns.

# 8. RDK-B Ecosystem and Stakeholders

RDK-B's strength comes not from any single vendor but from a multi-party ecosystem of operators, silicon vendors, hardware manufacturers, system integrators, and application developers. This section describes each stakeholder category, their relationship to the platform, their specific needs and value drivers, and representative examples.

Ecosystem Overview The RDK-B ecosystem operates as an inverted pyramid: ISPs at the top set requirements and fund the platform; SoC vendors provide the hardware foundation; OEMs build the physical devices; SIs integrate and customize; third-party developers extend with applications. Each layer depends on the layers below and enables the layers above.

## 8.1 Internet Service Providers (ISPs / Operators)

Primary Role ISPs are the primary consumers and primary funders of the RDK-B platform. They deploy RDK-B devices to millions of subscribers and use the platform's management, telemetry, and configuration capabilities to operate their broadband networks.

### Value Proposition for ISPs

ISPs adopt RDK-B because it eliminates the need to build and maintain a proprietary CPE software stack. A single RDK-B codebase can be deployed across multiple hardware vendors and SoC platforms, reducing fragmentation and allowing operators to consolidate their engineering investment. The platform provides:

- Vendor independence: the same management APIs and telemetry schemas regardless of underlying hardware supplier

- Rapid feature deployment: RFC-based remote feature control enables fleet-wide feature toggles without firmware upgrades

- Standardized data collection: T2 telemetry with configurable profiles provides consistent KPIs across all device types

- Reduced time-to-market: pre-integrated components accelerate device certification and deployment

- Lower total cost of ownership: shared open-source development amortizes engineering cost across the community

### ISP-Specific Feature Requirements

| ISP Requirement | RDK-B Capability |
| --- | --- |
| Remote Management | USP/TR-369 and TR-069 for centralized ACS-based configuration, xConf for RFC-based runtime feature control, WebConfig for configuration push without full firmware upgrades. |
| Telemetry & KPIs | T2 profile-driven telemetry for Wi-Fi performance metrics, WAN health statistics, error rates, and device-level KPIs published to Backoffice analytics platforms. |
| Firmware Lifecycle | OTA firmware orchestration via xConf and platform-manager enabling phased rollouts, A/B firmware updates, and emergency rollback. |
| Subscriber Self-Service | Local WebUI for residential subscribers to configure Wi-Fi, set parental controls, view connected devices, and run diagnostics without calling customer support. |
| Network Quality | DFS, band steering, 802.11kv neighbor reporting, and EasyMesh R6 for whole-home Wi-Fi quality management. |
| Security Compliance | WPA3/WPA3 Enterprise, stateful firewall, secure management channels (TLS), and signed firmware to meet regulatory and compliance obligations. |
| Public Wi-Fi | Passpoint/Hotspot 2.0 and public SSID support for operator-managed public Wi-Fi hotspot overlays at subscriber premises. |
| Parental Controls | DNS-based filtering, device pause, and managed site lists as value-added subscriber services. |

### Representative ISP Deployments

- Comcast / Xfinity (USA) — one of the founding members; deploys RDK-B on DOCSIS cable gateways (xFi Gateway) with deep telemetry and managed Wi-Fi integration

- Charter Communications / Spectrum (USA) — active TSC member; deploys RDK-B on DOCSIS and Ethernet WAN gateways with EasyMesh and advanced diagnostics

- Liberty Global (Europe/Americas) — founding member; major deployments in the Netherlands (Ziggo), UK (Virgin Media), Belgium (Telenet)

- Cox Communications (USA) — deploys RDK-B-based cable gateways with T2 telemetry and xConf-driven management

- Vodafone (Germany/Europe) — RDK-B deployments for DSL and fiber gateways with USP-based management

- Proximus (Belgium) — fiber/DSL deployments with RDK-B and EasyMesh mesh extenders

- Sky (UK/Italy) — RDK-B-based fiber and hybrid cable gateways

## 8.2 System-on-Chip (SoC) Vendors

Primary Role SoC vendors provide the silicon foundation on which RDK-B runs. Their primary responsibility is implementing the HAL layer — the set of APIs that RDK-B middleware calls to control Wi-Fi radios, DOCSIS modems, DSL chipsets, Ethernet switches, and other hardware blocks.

### Value Proposition for SoC Vendors

SoC vendors who support RDK-B gain access to the entire operator community that has standardized on the platform. A certified RDK-B HAL implementation means their SoC can be qualified for operator CPE program without the operator needing a custom software integration effort for that silicon. This dramatically reduces time-to-design-win and lowers the barrier for OEMs to adopt the vendor's chipset.

### SoC Vendor Responsibilities in RDK-B

- Implement all required HAL interfaces (rdkb-halif-wifi, rdkb-halif-moca, rdkb-halif-mta, rdkb-halif-fwupgrade, etc.) for their chipset

- Pass the HAL test suites published alongside each HAL interface definition (rdkb-halif-test-*)

- Provide a Linux BSP (kernel, drivers, firmware blobs) compatible with the Yocto/OE build system used by RDK-B

- Contribute NL80211-compatible or extension-based Wi-Fi driver interfaces for OneWifi integration

- Maintain HAL compatibility across RDK-B releases and platform versions

- Participate in TSC feature discussions to ensure new features are achievable on their silicon roadmap

### Representative SoC Vendors

| Vendor | RDK-B Relevance |
| --- | --- |
| Broadcom | DOCSIS, DSL, and Ethernet SoCs. Major presence in cable gateway silicon. BCM chips power many Comcast, Charter, and Cox gateway deployments. |
| MediaTek | Wi-Fi 6/6E/7 (MT7622, MT7988, MT7916, MT7915) and fiber/PON SoCs. Growing presence in residential gateways; active RDK-B HAL contributor. MT7988 (BananaPi R4) is a reference development platform. |
| Qualcomm | Wi-Fi 6/7 (IPQ series) and 5G/cellular SoCs. Powers high-end mesh and business gateway solutions. QMI cellular interface supported natively in RDK-B cellular-manager. |
| Intel (Lantiq/Maxlinear) | DSL and Ethernet SoCs (FALCON, VRX, URX families). Major in European DSL deployments for Vodafone, Deutsche Telekom, and others. |
| Marvell | Ethernet switching and 5G SoCs. Powers enterprise-class gateway and business CPE designs. |
| Realtek | Lower-cost Wi-Fi and Ethernet SoCs for budget residential gateways and extenders. |
| Econet / Airoha | Fiber/PON gateway SoCs used in Asia-Pacific deployments. |

### HAL Integration Model

SoC vendors ship their HAL implementations as pre-compiled binary libraries or as source code under NDA. The rdkb-halif-* header packages define the API contract. The rdkb-halif-test-* packages provide automated test scripts that SoC vendors must pass before claiming RDK-B compatibility. This test-first HAL model has dramatically improved cross-vendor portability and reduced integration defects.

## 8.3 Original Equipment Manufacturers (OEMs / ODMs)

Primary Role OEMs and ODMs design and manufacture the physical CPE hardware that operators deploy to subscribers. They assemble the SoC, memory, Wi-Fi radios, WAN interfaces, and enclosure into a finished product, then integrate RDK-B (sometimes with SI support) to produce a certifiable firmware image.

### OEM Responsibilities in RDK-B

- Select the SoC and confirm HAL availability for the target RDK-B features

- Integrate the SoC vendor's BSP and HAL libraries into the Yocto/BitBake build environment

- Configure RDK-B build recipes (meta-rdk, meta-rdk-broadband layers) for the specific board

- Develop board-specific platform init, LED management, button handling, and thermal management

- Conduct EMC/RF certification and operator device qualification testing

- Maintain firmware branches for the operational lifetime of the deployed device (often 5–10 years)

- Provide factory imaging tools and production test fixtures

### OEM Build System

RDK-B is built using Yocto Project / Open Embedded (OE). OEMs maintain a machine configuration layer (meta-<platform>) that defines the target board, SoC BSP path, and hardware-specific overrides. This sits alongside the RDK Central community layers (meta-rdk, meta-rdk-broadband, meta-rdk-bsp-*) that provide the RDK-B component recipes.

### Representative OEMs / ODMs

- Arris / CommScope — major gateway OEM for Comcast, Charter, Cox, and others; deploys millions of RDK-B devices annually

- Technicolor / Vantiva — gateway ODM with broad operator customer base including Liberty Global and Sky

- Sagemcomm — European gateway OEM with DSL/fiber focus; deployed in Vodafone and Proximus networks

- Netgear — consumer Wi-Fi mesh products with operator customization using RDK-B

- Sercomm — gateway ODM for Asian and North American markets

- Humax — STB and gateway OEM; RDK-B gateways for Sky and other operators

- AVM (FRITZ!Box) — exploring RDK-B integration for certain operator OEM variants

- Askey / ASUS — gateway ODMs active in Asian and North American deployments

## 8.4 System Integrators (SIs)

Primary Role System Integrators bridge the gap between the generic RDK-B platform and operator-specific deployment requirements. They take the community baseline, apply operator customizations, integrate proprietary cloud backends, and deliver a production-ready firmware binary that meets the operator's certification requirements.

### What SIs Do in the RDK-B World

- Operator customization: branding, feature flags, default configuration, SKU-specific builds

- Cloud backend integration: connecting T2 telemetry, WebPA/USP, Xconf, and crash upload to operator-specific cloud infrastructure

- Security hardening: applying operator security policies, disabling unused services, adding secure boot keys

- Certification support: conducting operator device qualification (ODQ) test campaigns and resolving defects

- Legacy protocol bridging: adapting RDK-B management interfaces to existing operator NMS/OSS systems

- Field upgrade tooling: building OTA distribution infrastructure and fleet management tooling

- Performance optimization: tuning Wi-Fi parameters, memory footprint, boot time, and throughput for specific hardware

- Long-term maintenance: applying security patches and CVE fixes for the operational fleet lifetime

### Representative System Integrators

- Capgemini — major RDK platform SI, works with multiple European and North American operators

- Infosys / Edge Verve — RDK-B integration services for telecom customers

- Cognizant — broadband CPE integration and managed services

- Tata Consultancy Services (TCS) — RDK-B SI work for operator customers globally

- Aricent / Altran (now Capgemini Engineering) — embedded CPE firmware integration

- GlobalLogic — RDK-B customization and CI/CD automation for operators

- Evoca / Alticast — RDK platform integration and managed OTA services

- LTTS — RDK platform integration and managed OTA services

## 8.5 Third-Party Application Developers

Primary Role The RDK-B container runtime (Dobby + DSM) enables third-party software companies to deploy their applications directly onto gateway devices as downloadable OCI containers. This transforms the broadband gateway from a fixed-function appliance into an open application platform.

### The App Container Ecosystem

With DSM (Downloadable Software Modules) and Dobby, operators can install, start, stop, and remove third-party applications on live deployed gateways without requiring a full firmware update. Applications run in isolated OCI containers with defined resource limits, security policies, and network access controls.

### Categories of Third-Party Applications

| Application Category | Description and Representative Companies |
| --- | --- |
| Home Security & Safety | Smart home security applications: motion detection, door/window sensor hubs, smoke/CO alert integration, and real-time alert notification. Companies: Alarm.com, Ring (Amazon), SimpliSafe. |
| Parental Control & Digital Wellbeing | Advanced parental control platforms with content categorization, screen time management, per-device scheduling, and threat protection beyond basic DNS filtering. Companies: Circle (Disney), Bark, Qustodio, Gryphon. |
| Network Security & Threat Intelligence | Inline threat detection, botnet prevention, malware blocking, and anomaly detection for all LAN devices. Companies: F-Secure (With Secure), Trend Micro Home Network Security, McAfee Secure Home Platform, Cujo AI. |
| Smart Home & IoT Hubs | Matter/Thread/Zigbee gateway applications that bridge IoT devices to cloud platforms. Companies: Amazon Alexa hub, Google Home hub integrations, SmartThings (Samsung), Home Assistant. |
| Wi-Fi Analytics & Optimization | Crowd-sourced and AI-driven Wi-Fi channel selection, interference mitigation, and quality optimization. Companies: Plume (Adapt), Cognitive Systems (Wi-Fi motion), Minim. |
| Voice Assistants & Smart Speakers | Embedded voice assistant runtimes that use the gateway as an always-on hub with multi-room audio routing. Companies: Amazon Alexa, Google Assistant (select ISP deployments). |
| Operator Value-Added Services | Operator-specific app bundles: advanced VPN, premium DNS (security/privacy), cloud backup for home, digital legacy, and enhanced parental dashboards offered as subscription upsells. |
| Network Performance & Gaming | Low-latency optimization, QoS gaming profiles, and bandwidth allocation applications. Companies: Netgear Armor (Bitdefender), TP-Link HomeCare. |
| Business / SMB Applications | VPN gateway, remote worker support, SD-WAN edge, and business continuity applications for the small business gateway profile. Companies: Cisco Meraki (select), OpenVPN. |
| Energy Management | Smart meter integration, energy disaggregation, and home energy management system (HEMS) bridges for utilities. Companies: Itron, Landis+Gyr, Sense. |

### Developer Access Model

Third-party developers access the RDK-B app platform through operator-managed app stores. Operators curate and approve applications, define resource quotas, and control deployment schedules via DSM. Developers package their software as OCI-compliant containers conforming to the RDK-B application container specification, which defines networking namespaces, RBUS IPC access permissions, and resource limit declarations.

## 8.6 Cloud and Platform Service Providers

Several cloud and platform companies provide backend services that RDK-B devices connect to:

- RDK Central xConf Service — the reference cloud for RFC feature control and firmware orchestration; most major operators run their own xConf instance or equivalent

- Plume / OpenSync — provides cloud-managed Wi-Fi optimization and the OpenSync open-source agent that integrates with the GW OpenSync and EXT OpenSync device profiles

- Xmidt (Comcast open-source) — the WebPA/Parodus cloud router and message relay infrastructure

- AWS IoT / Azure IoT Hub — used by some operators for USP controller infrastructure

- Broadband Forum ACS implementations (Axiros, Incognito, FreeACS) — TR-069 ACS backends

- Device42 / ServiceNow — CMDB and device lifecycle management integration for large operator fleet management

## 8.7 Standards Bodies and Alliances

RDK-B is deeply embedded in the broader broadband standards ecosystem:

- Broadband Forum (BBF) — TR-181 data model, TR-369 (USP), TR-069 (CWMP), TR-104 (VoIP). RDK-B implements all of these.

- Wi-Fi Alliance — WPA3, EasyMesh, Wi-Fi 6/6E/7, Passpoint/Hotspot 2.0, WPS certifications. RDK-B's OneWiFi implements the full WFA certification feature set.

- IEEE 802.11 — the underlying wireless standard. RDK-B supports 802.11n/ac/ax/be.

- CableLabs — DOCSIS 3.1/3.0 specification, Packet Cable MTA. RDK-B cable-modem-agent and MTA agent implement these.

- Open Connectivity Foundation (OCF) / CSA — Matter protocol. RDK-B's BartonCore implements Matter specification.

- Open Container Initiative (OCI) — container runtime and image spec. Dobby and crun implement OCI.

- RDK Central TSC — RDK-B's own governance and specification body.

## 8.8 Summary: Stakeholder Value Matrix

| Stakeholder | Role in Ecosystem | Primary Value from RDK-B |
| --- | --- | --- |
| ISPs / Operators | Platform Consumer, Funder, Roadmap Driver | Vendor independence, operational consistency, rapid feature deployment, telemetry, fleet management, reduced TCO |
| SoC Vendors | HAL Implementer, Silicon Supplier | Design-win acceleration, operator access, HAL certification, faster customer qualification cycles |
| OEMs / ODMs | Hardware Builder, Firmware Integrator | Reduced software development cost, access to operator program, shorter certification timelines, reuse across SKUs |
| System Integrators | Customization & Integration Services | Revenue from platform expertise, long-term managed services, operator relationships, differentiated CI/CD toolchains |
| Third-Party App Developers | Application Publisher | Access to 100M+ deployed gateway devices as an application platform, operator billing integration, secure container deployment model |
| Cloud Service Providers | Backend Connectivity | Integration with management, telemetry, and orchestration endpoints; extension of cloud service reach to the home edge |
| Standards Bodies | Specification Authors | Reference implementation of their standards, broad industry adoption, feedback from real-world deployments |

# 9. RDK-B Deployment Models

RDK-B supports a wide range of physical deployment models. Each model maps to a combination of device profiles, features, and cloud backends.

| Deployment Model | Description |
| --- | --- |
| DOCSIS Cable Gateway | The original and most widely deployed RDK-B configuration. A single device combining a DOCSIS 3.0/3.1 cable modem with a Wi-Fi router and 4-port Ethernet LAN switch. Managed via TR-069 or USP by an operator ACS. Profiles: Modem/ONU + GW Key features: cable-modem-agent, MTA (for voice), OneWiFi, wan-manager, full management stack. |
| Fiber / DSL Broadband Gateway | A multi-WAN capable gateway with xDSL, GPON, or Ethernet WAN uplink. Wi-Fi, routing, NAT, and full management stack. Common in European deployments. Profiles: EthWAN WiFi Router or GW Key features: xdsl-manager / gpon-manager, EasyMesh, OneWiFi Wi-Fi 6/7, USP management. |
| Wi-Fi Mesh System (Gateway + Extenders) | A primary gateway (GW EasyMesh or GW OpenSync profile) paired with one or more wireless extenders (EXT EasyMesh or EXT OpenSync profile). Provides whole-home Wi-Fi coverage. The gateway is the EasyMesh Controller; extenders are Agents. Profiles: GW EasyMesh + EXT EasyMesh (or OpenSync variants) Key features: unified-wifi-mesh, ieee1905-rs, EasyMesh R6, 802.11kv, band steering. |
| Standalone Wi-Fi Router | A device with Ethernet WAN and multi-band Wi-Fi for subscribers with a separate modem (e.g., fiber ONT). No DOCSIS/DSL modem. Profile: EthWAN WiFi Router Key features: OneWiFi Wi-Fi 6/7, NAT/routing, DHCP server, WebUI, parental controls. |
| Business / SMB Gateway | A gateway optimized for small business use with business-grade security, VLAN segmentation, multi-WAN failover, VPN support, and business management portal. Profile: GW with business-grade features Key features: webui-bwg, multi-WAN, cellular backup, VPN, advanced firewall. |
| Hybrid / Multi-WAN Gateway | A gateway with primary broadband WAN (cable/fiber/DSL) and a cellular backup WAN. Supports automatic WAN failover/restore. Profiles: GW or EthWAN WiFi Router with cellular-manager Key features: wan-manager failover, cellular-manager (QMI/MBIM), GPS, USB RNDIS. |
| Edge Compute / App Platform | A gateway acting as an edge compute node hosting operator and third-party containerized applications. Requires the downloadable app container stack. Profile: GW with DSM/Dobby Key features: DSM, Dobby/crun, OCI bundle management, BartonCore IoT hub. |

# 10. Device Profiles

RDK-B defines seven device profile archetypes. A profile is a high-level description of a device type — each built from required and optional Core components. "EthWAN WiFi Router" is a combined profile covering Ethernet-WAN-attached devices with integrated Wi-Fi.

| Profile | Description |
| --- | --- |
| Modem / ONU | Headless cable modem or ONT. WAN termination and basic platform services only. No Wi-Fi, no LAN switching. Typical: DOCSIS cable modem, fiber ONT. |
| EthWAN WiFi Router | Ethernet WAN with full Wi-Fi, routing, NAT, and LAN capabilities. Covers Ethernet-WAN routers and Wi-Fi routers. IPv4/IPv6 router with simple Ethernet WAN port and Wi-Fi AP features. |
| GW (Gateway) | Full broadband gateway: multi-band Wi-Fi, routing, NAT, firewall, DHCP, DNS, device management. One or more WAN access technologies. Primary reference deployment target. |
| GW OpenSync | GW profile with OpenSync cloud management plane. Cloud-managed Wi-Fi optimization and remote configuration. |
| GW EasyMesh | GW with EasyMesh R6 Controller+Agent. Supports multi-AP mesh networks; gateway as Controller. Wi-Fi AP fronthaul and Wi-Fi/Ethernet backhaul. |
| EXT OpenSync | EasyMesh extender under OpenSync cloud management. Mesh agent/extender only. Wi-Fi AP fronthaul and Wi-Fi/Ethernet backhaul. |
| EXT EasyMesh | Pure EasyMesh extender without OpenSync. 802.11 relay and EasyMesh agent. Minimal system stack. |

## 10.1 Feature Status Classification

| Status | Meaning |
| --- | --- |
| CORE | Applies to the Core Features column only. Identifies a small subset of foundational components that: (1) represent a minimal code footprint, (2) form basic infrastructure reusable by other software, (3) are required across all device profiles, and (4) may be listed as a dependency by other components and features. CORE components must be implemented and functional for any profile to be conformant. Test coverage required. |
| Required | Applies to profile columns. The feature or sub-feature MUST be implemented and functional for the device to be conformant with this profile. Test coverage required. |
| OPT | Optional. Supported by the platform; included at operator/ODM discretion. Must comply with the specification when included. |
| N/A | Not applicable. The feature has no architectural relevance to this device profile. |
| — | Not defined for this profile/feature combination in the current specification. |

# 11. Master Feature Matrix

Consolidated top-level summary of all 26 RDK-B feature areas mapped against all seven device profiles. Status reflects the highest-priority classification found across all sub-features within each area. Refer to Sections 12–36 for complete per-feature specifications.

| # | Feature | Sub-System | Core Features | Modem /ONU | EthWAN WiFi Router | GW | GW OpenSync | GW EasyMesh | EXT OpenSync | EXT EasyMesh |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Wi-Fi LAN | LAN | — | — | Required | Required | Required | Required | Required | Required |
| 2 | Ethernet LAN | LAN | — | — | OPT | — | — | — | — | — |
| 3 | MoCA LAN | LAN | — | — | — | OPT | OPT | OPT | — | — |
| 4 | Local WebUI | Device Mgmt | — | — | OPT | OPT | OPT | OPT | — | — |
| 5 | Remote Device Management | Remote Mgmt | — | — | OPT | OPT | OPT | OPT | OPT | OPT |
| 6 | Telemetry & Data collection | Diagnostics | — | — | OPT | OPT | OPT | OPT | OPT | OPT |
| 7 | Diagnostics & Testing | Diagnostics | CORE | — | Required | Required | Required | Required | OPT | OPT |
| 8 | System Logging | Diagnostics | CORE | Required | Required | Required | Required | Required | Required | Required |
| 9 | Inter Process Communication | IPC | CORE | Required | Required | Required | Required | Required | Required | Required |
| 10 | Matter/IoT | IOT | — | — | OPT | — | — | — | — | — |
| 11 | Downloadable App containers | App Framework | — | — | OPT | — | — | — | — | — |
| 12 | Voice/VoIP | Voice | — | OPT | OPT | OPT | OPT | OPT | — | — |
| 13 | Firmware Upgrade | Device Mgmt | — | — | OPT | OPT | OPT | OPT | OPT | OPT |
| 14 | Connected device discovery | LAN | — | — | Required | Required | Required | Required | OPT | OPT |
| 15 | System startup & orchestration | System | CORE | Required | Required | Required | Required | Required | Required | Required |
| 16 | Configuration Persistence | System | — | Required | Required | Required | Required | Required | — | — |
| 17 | Factory reset | System | — | Required | Required | Required | Required | Required | — | — |
| 18 | IP Routing/NAT/Firewall | Routing & Bridging | — | Required | Required | Required | Required | Required | Required | OPT |
| 19 | Bridge and VLAN configuration | Routing & Bridging | — | — | Required | OPT | OPT | OPT | — | — |
| 20 | Traffic redirection | Routing & Bridging | — | — | OPT | OPT | OPT | OPT | — | — |
| 21 | DHCP Server | LAN | — | — | Required | Required | Required | Required | — | — |
| 22 | DNS | DNS | — | — | OPT | Required | Required | Required | — | — |
| 23 | WAN Connectivity | WAN | — | OPT | Required | Required | Required | Required | — | — |
| 24 | Parental control | Parental Control | — | — | OPT | Required | Required | Required | — | — |
| 25 | WiFi Hotspot | LAN | — | — | — | OPT | OPT | OPT | OPT | OPT |
| 26 | Code Security | Security | CORE | Required | Required | Required | Required | Required | — | — |

# 12. Feature 1: Wi-Fi LAN

Sub-System: LAN    Primary Repository: https://github.com/rdkcentral/OneWifi

## 12.1 Overview

Wi-Fi LAN is the single largest and most complex feature area in the RDK-B platform. It is delivered entirely through the OneWiFi component, which provides a unified, vendor-independent management layer over all 802.11 radio hardware on the device. OneWiFi abstracts vendor-specific driver behavior through the industry-standard NL80211 kernel interface, with a defined extension mechanism for features not yet covered by NL80211.

The feature spans the full IEEE 802.11 generational stack — from 802.11n (Wi-Fi 4) through 802.11ac (Wi-Fi 5), 802.11ax (Wi-Fi 6/6E), and the latest 802.11be (Wi-Fi 7). Security coverage is equally comprehensive: WPA2, WPA2-Enterprise (802.1X/RADIUS), WPA3-Personal (SAE), WPA3-Enterprise, WPA3-Transition mode, WPA3 Enhanced Open (OWE), and WPA3 PCM (Post-Connection Management). Each security mode is independently selectable per SSID.

Multi-band operation covers 2.4 GHz, 5 GHz (lower/upper sub-bands), and 6 GHz. The platform supports up to 8 Virtual Access Points (VAPs) per physical radio, enabling SSID segmentation for residential, business, public hotspot, and IoT traffic. EasyMesh R6 Controller and Agent functionality is implemented through the unified-wifi-mesh component, using the ieee1905-rs IEEE 1905 transport layer for backhaul communication between mesh nodes.

## 12.2 Key Capabilities

- 802.11n/ac/ax/be multi-generation multi-band radio management through a single unified component

- Full WPA3 security suite including SAE, Enterprise, Transition, Enhanced Open (OWE), and PCM modes

- EasyMesh R6 Controller+Agent with IEEE 1905.1 backhaul transport via ieee1905-rs

- Wi-Fi 7 Single-Link and Multi-Link Operation (SLO/MLO) with 320 MHz channel bandwidth support

- NL80211-based HAL interface with vendor extension support for new features

- Dynamic Frequency Selection (DFS) with radar detection and Channel Switch Announcement (CSA)

- Band steering, client steering, 802.11kv neighbor reporting and BSS transition

- Up to 8 Virtual APs per radio with independent SSID, security, and QoS settings

- Public Wi-Fi SSID with encrypted WAN, Passpoint/Hotspot 2.0 GAS/ANQP support

- Off-channel scanning, MAC filtering per network, connection admission control

- Comprehensive event reporting: assoc/disassoc events, management frames, RSSI thresholds

- Spectrum analyzer, CSI data collection for motion sensing, advanced telemetry data collection

- Seamless dynamic reconfiguration without subscriber service interruption

## 12.3 Feature Detail and Profile Matrix

| Feature Detail | Repository | Modem /ONU | EthWAN WiFi Router | GW | GW OpenSync | GW EasyMesh | EXT OpenSync | EXT EasyMesh |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Standards |  |  |  |  |  |  |  |  |
| 802.11n | OneWiFi | — | Required | Required | Required | Required | Required | Required |
| 802.11ac | OneWiFi | — | Required | Required | Required | Required | Required | Required |
| 802.11ax (Wi-Fi 6/6E) | OneWiFi | — | OPT | OPT | OPT | OPT | OPT | OPT |
| 802.11be (Wi-Fi 7) | OneWiFi | — | OPT | OPT | OPT | OPT | OPT | OPT |
| Security |  |  |  |  |  |  |  |  |
| WPA2 | OneWiFi | — | Required | Required | Required | Required | Required | Required |
| WPA2 Enterprise | OneWiFi | — | OPT | OPT | OPT | OPT | OPT | OPT |
| WPA3 | OneWiFi | — | OPT | OPT | OPT | OPT | OPT | OPT |
| WPA3 Enterprise | OneWiFi | — | OPT | OPT | OPT | OPT | OPT | OPT |
| WPA3 Transition | OneWiFi | — | OPT | OPT | OPT | OPT | OPT | OPT |
| WPA3 Enhanced Open | OneWiFi | — | OPT | OPT | OPT | OPT | OPT | OPT |
| WPA3 PCM | OneWiFi | — | OPT | OPT | OPT | OPT | OPT | OPT |
| Easy Mesh |  |  |  |  |  |  |  |  |
| EasyMesh R6 – Controller+Agent | unified-wifi-mesh | — | OPT | — | — | Required | — | Required |
| EasyMesh R6 – IEEE 1905 transport ▸ Dependency for EasyMesh | ieee1905-rs | — | OPT | — | — | Required | — | Required |
| Wi-Fi 7 |  |  |  |  |  |  |  |  |
| 802.11 be | OneWiFi | — | OPT | OPT | OPT | OPT | OPT | OPT |
| 320 MHz | OneWiFi | — | OPT | OPT | OPT | OPT | OPT | OPT |
| SLO (Single Link Operation) and related WFA Data Model | OneWiFi | — | OPT | OPT | OPT | OPT | OPT | OPT |
| MLO (Multi-Link Operation) and related WFA Data Model | OneWiFi | — | OPT | OPT | OPT | OPT | OPT | OPT |
| HAL interface to vendor software |  |  |  |  |  |  |  |  |
| Support industry standard NL80211 vendor interface | OneWiFi | — | Required | — | — | — | — | — |
| Support extensions to NL80211 interface where needed for new features | OneWiFi | — | Required | — | — | — | — | — |
| Multi Band |  |  |  |  |  |  |  |  |
| 2.4 GHz | OneWiFi | — | OPT | OPT | OPT | OPT | OPT | OPT |
| 5 GHz / 5U GHz / 5L GHz | OneWiFi | — | OPT | OPT | OPT | OPT | OPT | OPT |
| 6 GHz | OneWiFi | — | OPT | OPT | OPT | OPT | OPT | OPT |
| Supported Operational Modes |  |  |  |  |  |  |  |  |
| AP mode supporting up to 8 Virtual APs per radio ▸ AP mode | OneWiFi | — | OPT | Required | Required | Required | Required | Required |
| STA only mode ▸ Station mode | OneWiFi | — | N/A | — | OPT | OPT | — | — |
| Extender mode ▸ Extender profile | OneWiFi | — | N/A | — | — | — | Required | Required |
| Event reporting |  |  |  |  |  |  |  |  |
| 802.11 event reporting | OneWiFi | — | Required | Required | Required | Required | Required | Required |
| Reporting of all Mgmt Frames | OneWiFi | — | OPT | OPT | OPT | OPT | OPT | OPT |
| Data Collection for Wi-Fi Assisted device typing | OneWiFi | — | OPT | OPT | OPT | OPT | — | — |
| Connection admission control | OneWiFi | — | OPT | OPT | OPT | OPT | — | — |
| CSI data collection for Motion Sensing and other applications | — | — | OPT | — | — | — | — | — |
| Band Steering | OneWiFi | — | OPT | OPT | OPT | OPT | — | — |
| Client steering to another AP | — | — | OPT | — | — | — | — | — |
| Data collection for Levl Fingerprinting | OneWiFi | — | OPT | OPT | OPT | OPT | — | — |
| Mac Filters Per Network | OneWiFi | — | OPT | OPT | OPT | OPT | OPT | OPT |
| Data Collection for Harvester Telemetry ▸ WiFi telemetry | OneWiFi | — | OPT | OPT | OPT | OPT | — | — |
| 802.11 kv ▸ Neighbor report & BSS transition | OneWiFi | — | OPT | Required | Required | Required | Required | Required |
| Off channel scan | OneWiFi | — | OPT | OPT | OPT | OPT | — | — |
| DFS ▸ Dynamic Frequency Selection | OneWiFi | — | OPT | Required | Required | Required | Required | Required |
| Management frame Power control | OneWiFi | — | OPT | OPT | OPT | OPT | — | — |
| Data Collection for Advanced Telemetry | OneWiFi | — | OPT | OPT | OPT | OPT | — | — |
| Radius Failover & IPV6 Support for Public Access ▸ Encrypted WAN is needed for security with Public Hotspot | OneWiFi | — | OPT | OPT | OPT | OPT | — | — |
| Cloud control WiFi optimization | OneWiFi | — | OPT | OPT | OPT | OPT | — | — |
| Public Wi-Fi SSID |  |  |  |  |  |  |  |  |
| public SSID ▸ Encrypted WAN is needed for security with Public Hotspot | hotspot | — | OPT | OPT | — | — | — | — |
| Passpoint / Hotspot 2.0 |  |  |  |  |  |  |  |  |
| Passpoint ▸ Encrypted WAN is needed for security with Public Hotspot | OneWiFi | — | OPT | OPT | OPT | OPT | — | — |
| WPS |  |  |  |  |  |  |  |  |
| WiFi Protected Setup | OneWiFi | — | OPT | OPT | OPT | OPT | OPT | OPT |
| Connected client reporting |  |  |  |  |  |  |  |  |
| Connected client list & assoc. events | OneWiFi | — | Required | Required | Required | Required | Required | Required |
| Spectrum analyzer |  |  |  |  |  |  |  |  |
| Spectrum analyzer | OneWiFi | — | OPT | OPT | OPT | OPT | — | — |
| Test framework support |  |  |  |  |  |  |  |  |
| Onewifi test suite & Vendor test suite | OneWifitestsuite | — | N/A | — | — | — | — | — |
| Reconfiguration |  |  |  |  |  |  |  |  |
| seamless WiFi dynamic reconfiguration support | OneWiFi | — | Required | Required | Required | Required | Required | Required |

## 12.4 Component Dependencies

The following RDK-B components are required or closely related to this feature:

- OneWiFi

- unified-wifi-mesh

- ieee1905-rs

- OneWiFiTestSuite

- hotspot

## 12.5 Implementation Notes

EasyMesh R6 – IEEE 1905 transport: Dependency for EasyMesh

AP mode supporting up to 8 Virtual APs per radio: AP mode

STA only mode: Station mode

Extender mode: Extender profile

Data Collection for Harvester Telemetry: WiFi telemetry

802.11 kv: Neighbor report & BSS transition

DFS: Dynamic Frequency Selection

Radius Failover & IPV6 Support for Public Access: Encrypted WAN is needed for security with Public Hotspot

public SSID: Encrypted WAN is needed for security with Public Hotspot

Passpoint: Encrypted WAN is needed for security with Public Hotspot

## 12.6 Testing Scope

- Radio bring-up and configuration validation for all supported bands and channel widths

- WPA2/WPA3 security handshake validation (SAE, OWE, 802.1X/RADIUS) per mode

- EasyMesh R6 Controller/Agent pairing, topology convergence, and STA steering validation

- Wi-Fi 7 MLO link setup/teardown and per-link statistics data model validation

- DFS radar detection compliance and CSA propagation timing

- 802.11kv neighbor report generation and BSS transition request/response

- Band steering trigger conditions, request generation, and client response handling

- VAP isolation — validate that traffic between SSIDs is correctly firewalled

- WPS PBC and PIN mode: session, credential exchange, and AP config update

- Passpoint GAS/ANQP query/response and Hotspot 2.0 indication element

- Connected client list accuracy, assoc event timing, and RSSI reporting precision

# 13. Feature 2: Ethernet LAN

Sub-System: LAN    Primary Repository: —

## 13.1 Overview

Ethernet LAN provides wired connectivity management for devices connected to the gateway's LAN-side Ethernet ports. The Ethernet Agent monitors port link state, manages link speed negotiation, and provides operators with the ability to force IP reconfiguration on connected devices.

The forced IP reconfiguration capability is particularly valuable for operators managing device replacement scenarios — by momentarily dropping and re-establishing a LAN Ethernet link, the gateway triggers the connected device to release and reacquire its DHCP lease, ensuring it receives up-to-date network configuration without requiring the subscriber to manually power-cycle their client device.

Link speed detection and management enable the gateway to report and control the negotiated speed on each Ethernet port, providing visibility into potential bottlenecks caused by legacy cabling or client hardware limitations.

## 13.2 Key Capabilities

- Per-port link up/down detection with event generation for connected device tracking

- Forced IP reconfiguration on reconnect — momentarily drops link to trigger DHCP renewal on connected client

- Link speed detection and reporting for all LAN Ethernet ports

- Integration with Device.Hosts table for connected device inventory updates

## 13.3 Feature Detail and Profile Matrix

| Feature Detail | Repository | Modem /ONU | EthWAN WiFi Router | GW | GW OpenSync | GW EasyMesh | EXT OpenSync | EXT EasyMesh |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Connected device detection |  |  |  |  |  |  |  |  |
| Link up/down detection | Ethernet Agent | — | OPT | — | — | — | — | — |
| Enable/disable link including forced IP reconfig on reconnect |  |  |  |  |  |  |  |  |
| Includes ability to force a connected device to reconfigure IP by momentarily dropping the link | Ethernet Agent | — | OPT | — | — | — | — | — |
| Link Speed management |  |  |  |  |  |  |  |  |
| Link speed detection and management | Ethernet Agent | — | OPT | — | — | — | — | — |

## 13.4 Component Dependencies

The following RDK-B components are required or closely related to this feature:

- Ethernet Agent

- lan-manager-lite

- dhcp-manager

## 13.6 Testing Scope

- Link state event generation on cable connect/disconnect

- Forced IP reconfiguration — validate client DHCP renewal after momentary link drop

- Link speed negotiation reporting accuracy across 10/100/1000 Mbps rates

# 14. Feature 3: MoCA LAN

Sub-System: LAN    Primary Repository: https://github.com/rdkcentral/moca-agent

## 14.1 Overview

MoCA (Multimedia over Coax Alliance) LAN enables high-performance home networking over existing coaxial cable infrastructure, providing an alternative to Wi-Fi backhaul for EasyMesh extenders or wired connectivity for LAN devices without rewiring.

RDK-B implements MoCA support through the moca-agent component, which monitors node status, link quality, PHY rates, and network topology. A dedicated HAL interface (rdkb-halif-moca) provides the abstraction layer between the moca-agent middleware and the silicon vendor's MoCA firmware.

The rdkb-halif-test-moca test suite validates all mandatory HAL functions before a MoCA SoC implementation can be considered RDK-B compatible, covering node initialization, link establishment, frequency selection, and statistics reporting.

## 14.2 Key Capabilities

- MoCA LAN-over-coax connectivity for devices without Wi-Fi capability

- MoCA network node monitoring including PHY rate, SNR, and topology

- HAL abstraction enabling multiple MoCA silicon implementations

- HAL test suite for vendor certification of MoCA implementations

## 14.3 Feature Detail and Profile Matrix

| Feature Detail | Repository | Modem /ONU | EthWAN WiFi Router | GW | GW OpenSync | GW EasyMesh | EXT OpenSync | EXT EasyMesh |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MoCA LAN Features |  |  |  |  |  |  |  |  |
| MoCA LAN-over-cable | moca-agent | — | N/A | OPT | OPT | OPT | — | — |
| MoCA HAL interface | rdkb-halif-moca | — | N/A | OPT | OPT | OPT | — | — |
| MoCA HAL test | rdkb-halif-test-moca | — | N/A | OPT | OPT | OPT | — | — |

## 14.4 Component Dependencies

The following RDK-B components are required or closely related to this feature:

- moca-agent

- rdkb-halif-moca

- rdkb-halif-test-moca

## 14.6 Testing Scope

- MoCA node initialization and network join/leave event reporting

- PHY rate, SNR, and frequency reporting accuracy via HAL

- HAL test suite: all mandatory rdkb-halif-moca functions validated

# 15. Feature 4: Local WebUI

Sub-System: Device Mgmt    Primary Repository: https://github.com/rdkcentral/webui

## 15.1 Overview

The Local WebUI provides a browser-accessible management interface served directly from the gateway, enabling subscribers and network administrators to configure the device without requiring a cloud connection or operator support call.

Two portal variants are provided: a residential portal (webui) delivering a consumer-friendly interface covering Wi-Fi settings, connected device management, parental controls, and basic diagnostics; and a business gateway portal (webui-bwg) providing additional features appropriate for SMB deployments such as VLAN configuration, advanced firewall rules, and multi-WAN status.

Both portals are served over HTTPS and require authentication, ensuring that local management access is protected even in shared or semi-public network environments.

## 15.2 Key Capabilities

- HTTPS-secured local management portal accessible from any browser on the LAN

- Residential portal: Wi-Fi SSID/password configuration, connected device list, parental controls, speed test, diagnostics

- Business gateway portal: VLAN configuration, advanced firewall, multi-WAN status, business-specific reporting

- Authentication-protected access with session management

## 15.3 Feature Detail and Profile Matrix

| Feature Detail | Repository | Modem /ONU | EthWAN WiFi Router | GW | GW OpenSync | GW EasyMesh | EXT OpenSync | EXT EasyMesh |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Residential Portal |  |  |  |  |  |  |  |  |
| HTML/HTTPS residential mgmt portal | webui | — | OPT | OPT | OPT | OPT | — | — |
| Business Gateway Portal |  |  |  |  |  |  |  |  |
| Business gateway HTML portal | webui-bwg | — | OPT | OPT | OPT | OPT | — | — |

## 15.4 Component Dependencies

The following RDK-B components are required or closely related to this feature:

- webui

- webui-bwg

## 15.6 Testing Scope

- HTTPS certificate validation and redirect from HTTP

- Authentication enforcement — verify unauthenticated access is blocked

- Wi-Fi configuration changes via UI propagate to OneWiFi RBUS data model

- Connected device list accuracy matches Device.Hosts TR-181 table

- Business portal: VLAN and firewall rule application from UI

# 16. Feature 5: Remote Device Management

Sub-System: Remote Mgmt    Primary Repository: https://github.com/xmidt-org/parodus2ccsp https://github.com/xmidt-org/parodus https://github.com/rdkcentral/start-parodus

## 16.1 Overview

Remote Device Management is the set of protocols and frameworks through which operators remotely configure, monitor, and update deployed gateway devices at scale. RDK-B supports the complete spectrum of management protocols, from legacy SNMP through modern USP/TR-369, providing operators with flexibility to choose the protocol that best fits their management infrastructure.

WebPA (parodus) provides the legacy cloud-based parameter access mechanism for WebPA-based management systems such as Comcast's XPC platform. USP (TR-369, via usp-pa-vendor-rdk) is the modern management protocol providing full device management including commands, events, and bulk data collection over MQTT or WebSocket. TR-069/CWMP (tr069-protocol-agent) provides ACS-based management for operators using traditional CWMP infrastructure.

RFC (Remote Feature Control) enables operators to toggle platform features at runtime without a firmware upgrade — a capability critical for operators managing fleets of 10M+ devices where firmware rollout timelines span months. WebConfig enables configuration push of structured JSON payloads to specific component namespaces. SNMP is supported for legacy NMS integration but is not recommended for new development.

## 16.2 Key Capabilities

- WebPA/parodus: cloud-based parameter get/set over WebSocket; Comcast XPC and compatible systems

- USP (TR-369): modern device management with commands, events, bulk data; MQTT/WebSocket transport

- TR-069/CWMP: ACS-based configuration, firmware management, diagnostics; legacy operators

- SNMP: legacy NMS integration for operators with existing SNMP infrastructure

- RFC (Remote Feature Control): runtime feature enable/disable via xConf without firmware upgrade

- WebConfig: structured configuration push to component namespaces without full TR-181 support

## 16.3 Feature Detail and Profile Matrix

| Feature Detail | Repository | Modem /ONU | EthWAN WiFi Router | GW | GW OpenSync | GW EasyMesh | EXT OpenSync | EXT EasyMesh |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| WebPA |  |  |  |  |  |  |  |  |
| WebPA device management ▸ No commands or event support | parodus start-parodus xmidt-org | — | OPT | OPT | OPT | OPT | OPT | OPT |
| USP |  |  |  |  |  |  |  |  |
| USP (TR-369) device management | usp-pa-vendor-rdk | — | OPT | OPT | OPT | OPT | OPT | OPT |
| TR069 |  |  |  |  |  |  |  |  |
| CWMP / ACS device management | tr069-protocol-agent | — | OPT | OPT | OPT | OPT | OPT | OPT |
| SNMP |  |  |  |  |  |  |  |  |
| SNMP device management ▸ Legacy – not for new dev | snmp-protocol-agent | — | OPT | OPT | OPT | OPT | — | — |
| RFC (remote feature control) |  |  |  |  |  |  |  |  |
| Runtime enable/disable via xConf | xconf-client | — | OPT | — | — | — | — | — |
| WebConfig |  |  |  |  |  |  |  |  |
| WebConfig based device configuration ▸ No TR-181 support | WebConfig | — | OPT | OPT | OPT | OPT | — | — |

## 16.4 Component Dependencies

The following RDK-B components are required or closely related to this feature:

- parodus

- start-parodus

- usp-pa-vendor-rdk

- tr069-protocol-agent

- snmp-protocol-agent

- xconf-client

- WebconfigFramework

## 16.5 Implementation Notes

WebPA device management: No commands or event support

SNMP device management: Legacy – not for new dev

WebConfig based device configuration: No TR-181 support

## 16.6 Testing Scope

- WebPA: parameter get/set/notify round-trip through parodus to RBUS

- USP: device management session establishment, command execution, event subscription

- TR-069: ACS bootstrap, provisioning, inform cycle, firmware download trigger

- RFC: feature flag toggle via xConf propagates to component behavior within defined SLA

- WebConfig: configuration payload delivery and namespace-specific application

- All management protocols: verify TLS channel security and certificate validation

# 17. Feature 6: Telemetry & Data collection

Sub-System: Diagnostics    Primary Repository: https://github.com/rdkcentral/telemetry

## 17.1 Overview

Telemetry and Data Collection provide the mechanisms through which operators gather operational metrics from deployed devices to monitor fleet health, detect issues proactively, and measure QoS KPIs across millions of simultaneous deployments.

The primary telemetry mechanism is T2 (Telemetry 2.0), a profile-driven framework that allows operators to define telemetry collection profiles specifying which metrics to collect, how frequently, how to aggregate them, and where to upload them. T2 profiles can be delivered and updated at runtime via xConf without a firmware upgrade, giving operators dynamic control over what data is collected from their fleet.

Crash data upload (crashupload) provides automatic capture and delivery of crash dumps, core files, and stack traces to the operator Backoffice. This component is intentionally kept dependency-free to ensure it can operate even when other components have failed. The Harvester component provides legacy hard-coded telemetry for backward compatibility with WebPA-based collection systems.

## 17.2 Key Capabilities

- T2 Telemetry 2.0: profile-driven telemetry collection with configurable markers, reports, and upload destinations

- Runtime profile delivery via xConf — no firmware upgrade required to change what is collected

- Crash data upload: zero-dependency crash dump capture, core file collection, and Backoffice delivery

- Harvester: legacy hard-coded WebPA telemetry pipeline for backward-compatible collection

- Configurable upload frequency, aggregation windows, and backoff policies

- Support for structured (JSON) and unstructured (log-based) telemetry formats

## 17.3 Feature Detail and Profile Matrix

| Feature Detail | Repository | Modem /ONU | EthWAN WiFi Router | GW | GW OpenSync | GW EasyMesh | EXT OpenSync | EXT EasyMesh |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Configurable Telemetry |  |  |  |  |  |  |  |  |
| Profile-driven telemetry collection | T2 telemetry | — | OPT | OPT | OPT | OPT | OPT | OPT |
| Crash Reporting |  |  |  |  |  |  |  |  |
| Crash data upload ▸ Zero deps | crashupload | — | OPT | OPT | OPT | OPT | OPT | OPT |
| Harvester Telemetry |  |  |  |  |  |  |  |  |
| Harvester – hard-coded WebPA telemetry ▸ Legacy – not for new dev | harvester | — | OPT | OPT | OPT | OPT | OPT | OPT |

## 17.4 Component Dependencies

The following RDK-B components are required or closely related to this feature:

- telemetry

- crashupload

- harvester

- xconf-client

- rdk_logger

## 17.5 Implementation Notes

Crash data upload: Zero deps

Harvester – hard-coded WebPA telemetry: Legacy – not for new dev

## 17.6 Testing Scope

- T2: profile delivery via xConf and marker activation within specified latency

- T2: report generation accuracy against reference dataset

- T2: upload success rate and retry behavior on connectivity loss

- crashupload: crash dump captured and uploaded on controlled fault injection

- Validate T2 operates without errors when Harvester is not present (OPT dependency)

# 18. Feature 7: Diagnostics & Testing

Sub-System: Diagnostics    Primary Repository: https://github.com/rdkcentral/test-and-diagnostic

## 18.1 Overview

Diagnostics and Testing provide the tools through which operators, field engineers, and automated systems validate connectivity, measure performance, investigate issues, and ensure continuous device health. These tools are designed to be remotely triggerable via RBUS/USP, enabling fleet-wide diagnostic campaigns without physical device access.

The self-heal module is a critical CORE component that continuously monitors the device's operational state and triggers recovery actions when components become unresponsive or when connectivity is lost. Self-heal implements a tiered recovery strategy: first attempting component restart, then partial reboot sequences, and finally a full device reboot if softer recovery actions are insufficient.

TCP latency measurement provides a standardized mechanism for operators to measure round-trip latency from the gateway to a specified target, enabling detection of WAN congestion, ISP network issues, and subscriber-reported slowness at scale.

## 18.2 Key Capabilities

- Layer 4 connectivity tests: ping, traceroute, DNS resolution, HTTP reachability

- Self-heal monitoring: continuous health checks with tiered recovery (restart → partial reboot → full reboot)

- TCP latency measurement to configurable targets; results published via RBUS/T2

- Gateway speed test (IMP): measures WAN bandwidth from the gateway perspective

- Live field debug via remote_debugger: SSH-based access to running device for L2/L3 issue investigation

- CPU and process analyzer (cpuprocanalyzer): on-demand resource profiling for memory and CPU investigations

- WAN health check (IPoE): validates IPoE WAN connectivity and DHCP lease health

- Legacy notification component for hard-coded DM value change notifications

## 18.3 Feature Detail and Profile Matrix

| Feature Detail | Repository | Modem /ONU | EthWAN WiFi Router | GW | GW OpenSync | GW EasyMesh | EXT OpenSync | EXT EasyMesh |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Connectivity Tests |  |  |  |  |  |  |  |  |
| Layer 4 connectivity tests | test-and-diagnostic | — | OPT | OPT | OPT | OPT | OPT | OPT |
| WAN Health |  |  |  |  |  |  |  |  |
| IPOE WAN health check ▸ NOT planned for release | ipoe-health-check | — | OPT | OPT | OPT | OPT | — | — |
| Field Debug |  |  |  |  |  |  |  |  |
| Live field issue debugging | remote_debugger | CORE | Required | Required | Required | Required | Required | Required |
| cpuproc analyzer |  |  |  |  |  |  |  |  |
| Process Analyzer | cpuprocanalyzer | — | OPT | OPT | OPT | OPT | OPT | OPT |
| Legacy Notifications |  |  |  |  |  |  |  |  |
| Hard-coded DM value change notify ▸ Legacy | notify-component | — | OPT | OPT | OPT | OPT | — | — |
| Gateway speed test |  |  |  |  |  |  |  |  |
| IMP speed test | test-and-diagnostic | — | OPT | — | — | — | — | — |
| TCP Latency measurement |  |  |  |  |  |  |  |  |
| Latency | Test & Diagnostics | — | OPT | OPT | OPT | OPT | — | — |
| Self heal |  |  |  |  |  |  |  |  |
| monitoring and recovery module | Test & Diagnostics | — | OPT | OPT | OPT | OPT | — | — |

## 18.4 Component Dependencies

The following RDK-B components are required or closely related to this feature:

- test-and-diagnostic

- remote_debugger

- cpuprocanalyzer

- notify-component

- rdk_logger

## 18.5 Implementation Notes

IPOE WAN health check: NOT planned for release

Hard-coded DM value change notify: Legacy

## 18.6 Testing Scope

- Self-heal: inject component failure and validate correct recovery action and timing

- TCP latency: validate measurement accuracy against reference latency source

- Layer 4 tests: validate each test type (ping, traceroute, DNS, HTTP) against known good/bad targets

- remote_debugger: validate secure session establishment and command execution

- cpuprocanalyzer: validate process list accuracy and resource metrics against OS ground truth

# 19. Feature 8: System Logging

Sub-System: Diagnostics    Primary Repository: https://github.com/rdkcentral/rdk_logger

## 19.1 Overview

System Logging is a foundational platform service required across all device profiles and all component types. The rdk_logger framework provides a unified, consistent logging API that any RDK-B component can use without introducing additional dependencies — the library is intentionally kept zero-dependency to ensure it can be included in the most resource-constrained Modem/ONU or Extender profiles.

The logging framework enforces consistent log formatting across all components, including mandatory fields such as component name, log level, timestamp, and process ID. This consistency is essential for automated log parsing, log aggregation in Backoffice systems, and for support engineers who need to correlate logs from multiple components across different device types.

Log rotation prevents flash storage exhaustion on resource-constrained devices by managing log file size limits and rotation policies. The DCM agent (Device Configuration Manager) handles secure upload of collected logs to the operator Backoffice infrastructure, supporting both on-demand upload triggered by operator request and scheduled automatic upload.

## 19.2 Key Capabilities

- Zero-dependency unified logging API — included in all device profiles from Modem/ONU to Gateway

- Consistent log format: component name, timestamp, log level, process ID, thread ID

- Configurable log levels (FATAL, ERROR, WARN, INFO, DEBUG, TRACE) per component

- Log rotation with configurable file size limits and retention policies for flash-constrained devices

- DCM agent: secure log upload to operator Backoffice on schedule or on-demand trigger

- Log buffering to handle temporary upload failures with automatic retry

## 19.3 Feature Detail and Profile Matrix

| Feature Detail | Repository | Modem /ONU | EthWAN WiFi Router | GW | GW OpenSync | GW EasyMesh | EXT OpenSync | EXT EasyMesh |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Logger |  |  |  |  |  |  |  |  |
| Unified log framework – zero deps ▸ Required in all profiles | rdk_logger | CORE | Required | Required | Required | Required | Required | Required |
| Log rotation |  |  |  |  |  |  |  |  |
| Log file rotation management | rdk_logger | CORE | Required | Required | Required | Required | Required | Required |
| Log upload |  |  |  |  |  |  |  |  |
| Log upload to backoffice | dcm | — | OPT | OPT | OPT | OPT | OPT | OPT |

## 19.4 Component Dependencies

The following RDK-B components are required or closely related to this feature:

- rdk_logger

- dcm-agent

## 19.5 Implementation Notes

Unified log framework – zero deps: Required in all profiles

## 19.6 Testing Scope

- Validate rdk_logger compiles and functions with zero external library dependencies

- Log format conformance: all mandatory fields present and correctly formatted

- Log rotation: validate file size limits are enforced and rotated files are managed correctly

- DCM upload: validate upload success, retry on failure, and upload confirmation

- Log level filtering: validate that setting DEBUG level does not emit TRACE output

# 20. Feature 9: Inter Process Communication

Sub-System: IPC    Primary Repository: https://github.com/rdkcentral/rbus

## 20.1 Overview

Inter-Process Communication (IPC) is the fundamental communication backbone of the RDK-B platform. All data model access, configuration changes, event notifications, and method invocations between components flow through the RBUS framework. RBUS is the modern, actively developed IPC system for RDK-B, providing a clean, typed, publish-subscribe plus RPC communication model based on the TR-181 data model hierarchy.

RBUS provides five core primitives: get (read a TR-181 parameter value), set (write a TR-181 parameter value), multi-instance object management (create/delete/iterate TR-181 table rows), method invocation (RPC call to a component's registered handler), and event pub/sub (subscribe to notifications when parameter values change). All five primitives are available over the RBUS socket, accessible from any component in the system.

For backward compatibility with the large installed base of components written against the legacy CCSP/DMCLI IPC model, the CCSP shim layer in common-library provides a compatibility API that translates CCSP calls to RBUS operations at runtime. This allows legacy components to continue functioning without modification while the platform migrates to pure RBUS. The inter-device-manager component extends RBUS connectivity across LAN-attached devices, enabling a gateway to access data model parameters on connected extender nodes.

## 20.2 Key Capabilities

- RBUS get/set: read/write TR-181 data model parameters with type validation and access control

- RBUS multi-instance: create, delete, and iterate TR-181 table rows (e.g., Device.WiFi.SSID.{i})

- RBUS method invocation: typed RPC with defined input/output parameter schemas

- RBUS event pub/sub: value-change notification with configurable filter and debounce

- RBUS CLI: command-line interface for operator and developer data model access

- CCSP shim (common-library): backward-compatible CCSP API over RBUS for legacy components

- DMCLI (data-model-cli): command-line CCSP data model access for legacy scripting

- inter-device-manager: RBUS bridge extending IPC connectivity across LAN-connected nodes

## 20.3 Feature Detail and Profile Matrix

| Feature Detail | Repository | Modem /ONU | EthWAN WiFi Router | GW | GW OpenSync | GW EasyMesh | EXT OpenSync | EXT EasyMesh |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RBUS – Modern IPC |  |  |  |  |  |  |  |  |
| RBUS CLI Support | rbus | CORE | Required | Required | Required | Required | Required | Required |
| Support for get and set | rbus | CORE | Required | Required | Required | Required | Required | Required |
| Support for multi-instance objects | rbus | — | Required | — | — | — | — | — |
| Method / command invocation | rbus | CORE | Required | Required | Required | Required | Required | Required |
| Event subscription & notification | rbus | CORE | Required | Required | Required | Required | Required | Required |
| Legacy CCSP Shim |  |  |  |  |  |  |  |  |
| Legacy DMCLI support | DMCLI | — | Required | — | — | — | — | — |
| CCSP backward-compat API over RBUS ▸ Legacy – needed by existing components | common-library | CORE | Required | Required | Required | Required | Required | Required |
| Inter-Device IPC |  |  |  |  |  |  |  |  |
| RBUS bridge between LAN devices | inter-device-manager | — | OPT | OPT | OPT | OPT | — | — |

## 20.4 Component Dependencies

The following RDK-B components are required or closely related to this feature:

- rbus

- common-library

- data-model-cli

- inter-device-manager

## 20.5 Implementation Notes

CCSP backward-compat API over RBUS: Legacy – needed by existing components

## 20.6 Testing Scope

- RBUS get/set round-trip latency measurement against TR-181 reference parameters

- RBUS event subscription and notification delivery accuracy and timing

- RBUS method invocation with valid and invalid argument sets

- Multi-instance CRUD: create/read/update/delete TR-181 table row lifecycle

- CCSP shim: DMCLI get/set maps correctly to RBUS outcome

- inter-device-manager: validate parameter access across device bridge

- Concurrent access: validate correct serialization under multi-threaded access patterns

# 21. Feature 10: Matter/IoT

Sub-System: IOT    Primary Repository: https://github.com/rdkcentral/BartonCore

## 21.1 Overview

Matter/IoT provides native smart home protocol support, enabling the RDK-B gateway to act as a Matter border router and IoT hub for the subscriber's home network. This positions the broadband gateway as the central home intelligence node, rather than requiring a separate smart home hub device.

The BartonCore framework implements Matter (formerly CHIP — Connected Home over IP), Zigbee, Thread, and Bluetooth Low Energy (BLE) commissioning. Matter support enables the gateway to discover, commission, and control Matter-certified IoT devices from any ecosystem — Apple Home, Google Home, Amazon Alexa, and Samsung SmartThings all support Matter interoperability.

Thread is the IPv6-based mesh networking protocol used as the transport layer for many Matter devices. BartonCore implements a Thread Border Router, connecting Thread network devices to the broader IP network. Zigbee bridging allows legacy Zigbee IoT devices to be integrated into the Matter ecosystem through the gateway.

## 21.2 Key Capabilities

- Matter border router: discover, commission, and control Matter-certified smart home devices

- Thread Border Router: connect Thread mesh network to IPv6 home network

- Zigbee bridging integrate legacy Zigbee devices into Matter/IP ecosystem

- BLE for Matter commissioning: Bluetooth LE as the out-of-band commissioning transport

- Multi-ecosystem interoperability: Apple, Google, Amazon, Samsung via Matter standard

## 21.3 Feature Detail and Profile Matrix

| Feature Detail | Repository | Modem /ONU | EthWAN WiFi Router | GW | GW OpenSync | GW EasyMesh | EXT OpenSync | EXT EasyMesh |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Matter |  |  |  |  |  |  |  |  |
| Matter IoT device support | BartonCore | — | OPT | — | — | — | — | — |
| Zigbee |  |  |  |  |  |  |  |  |
| Zigbee IoT device support | BartonCore | — | OPT | — | — | — | — | — |
| Thread |  |  |  |  |  |  |  |  |
| Thread IoT device support | BartonCore | — | OPT | — | — | — | — | — |
| Bluetooth |  |  |  |  |  |  |  |  |
| Bluetooth support for Matter comissioning | BartonCore | — | OPT | — | — | — | — | — |

## 21.4 Component Dependencies

The following RDK-B components are required or closely related to this feature:

- BartonCore

## 21.6 Testing Scope

- Matter device commissioning flow: QR code scan, BLE discovery, onboarding, and control

- Thread Border Router: Thread device join, IP connectivity, and mesh formation

- Zigbee bridge: device pairing and command relay to Matter ecosystem

- Multi-device concurrency: validate stable operation with 10+ simultaneous IoT devices

# 22. Feature 11: Downloadable App containers

Sub-System: App Framework    Primary Repository: https://github.com/rdkcentral/DSM

## 22.1 Overview

Downloadable App Containers enable operators to deploy, update, and remove third-party and operator-developed applications on live deployed gateways without requiring a full firmware upgrade. This transforms the broadband gateway from a fixed-function appliance into an open, programmable edge compute platform.

The Downloadable Software Modules (DSM) component implements the TR-181 SoftwareModules data model, which defines the data model objects and RPCs for application lifecycle management: install, update, start, stop, and remove. DSM communicates with operator app stores and orchestration systems via USP or WebConfig, and manages the application inventory and state on the device.

Application execution is handled by Dobby, an OCI-compatible container runtime purpose-built for embedded Linux environments. Dobby uses crun as the low-level container runtime, implementing the OCI Runtime Specification. Containers run in isolated Linux namespaces with defined CPU, memory, and network resource limits declared in OCI bundle configuration. Dobby enforces security policies preventing container escape and unauthorized access to host resources.

## 22.2 Key Capabilities

- TR-181 SoftwareModules data model: application inventory, version, and lifecycle state management

- App install/update/start/stop/remove via USP RPC or WebConfig delivery

- OCI-compliant container runtime (Dobby + crun) with Linux namespace isolation

- Resource limits: CPU, memory, network bandwidth per container via cgroup constraints

- Security isolation: container escape prevention, restricted host filesystem access

- Multiple container runtimes supported: crun (primary), youki (optional alternative)

- App store integration: operator-curated app delivery and versioning

## 22.3 Feature Detail and Profile Matrix

| Feature Detail | Repository | Modem /ONU | EthWAN WiFi Router | GW | GW OpenSync | GW EasyMesh | EXT OpenSync | EXT EasyMesh |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| App LCM |  |  |  |  |  |  |  |  |
| TR-181 SoftwareModules data model ▸ Active dev – TR-181 compliance | DSM | — | OPT | — | — | — | — | — |
| App install/start/stop/remove | DSM | — | OPT | — | — | — | — | — |
| Container Runtime |  |  |  |  |  |  |  |  |
| OCI container runtime (crun) | Dobby | — | OPT | — | — | — | — | — |
| Container lifecycle management | Dobby | — | OPT | — | — | — | — | — |

## 22.4 Component Dependencies

The following RDK-B components are required or closely related to this feature:

- DSM

- Dobby

- crun

## 22.5 Implementation Notes

TR-181 SoftwareModules data model: Active dev – TR-181 compliance

## 22.6 Testing Scope

- App install via USP: OCI bundle download, verification, and container instantiation

- App lifecycle: start/stop/restart transitions and state machine validation

- Resource limits: validate CPU and memory constraints are enforced by cgroup

- Isolation: validate container cannot access host filesystem paths outside its mount

- App update: validate running app can be updated in-place with minimal service interruption

- Remove: validate full cleanup of container rootfs and data volumes on removal

# 23. Feature 12: Voice/VoIP

Sub-System: Voice    Primary Repository: https://github.com/rdkcentral/media-terminal-adapter-agent

## 23.1 Overview

Voice/VoIP support in RDK-B covers two distinct deployment scenarios: Packet Cable MTA (Multimedia Terminal Adapter) for traditional cable operator voice services, and TR-104-based telco voice stack management for DSL and fiber operator VoIP deployments.

The Packet Cable MTA agent (media-terminal-adapter-agent) monitors the status of the embedded Digital Voice Adapter (eDVA) within a DOCSIS gateway, reporting registration state, call state, line status, and diagnostic information via the TR-181 data model. This enables operators to remotely diagnose voice service issues, trigger diagnostics, and monitor battery backup status for E911 compliance.

The telco-voice-manager implements TR-104 data model management for SIP-based VoIP stacks, supporting operators who deploy RDK-B on DSL or fiber gateways where the voice stack is provided by a third-party SIP client library. Battery backup management for MTA scenarios ensures continuous E911 service availability during power outages.

## 23.2 Key Capabilities

- Packet Cable MTA status monitoring: registration state, call state, line status, endpoint status

- MTA diagnostics: loopback, tone generation, hook state reporting

- MTA HAL interface (rdkb-halif-mta): vendor-agnostic MTA firmware abstraction

- MTA HAL test suite for RDK Ready certification of MTA silicon implementations

- TR-104 telco voice stack management: SIP registration, call state, line configuration

- Battery backup management: power mode transitions, battery health, runtime estimation

- Power manager HAL and test suite for battery SoC integration

## 23.3 Feature Detail and Profile Matrix

| Feature Detail | Repository | Modem /ONU | EthWAN WiFi Router | GW | GW OpenSync | GW EasyMesh | EXT OpenSync | EXT EasyMesh |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PacketCable MTA |  |  |  |  |  |  |  |  |
| MTA status & diagnostics ▸ PacketCable 2.0 eDVA | media-terminal-adapter-agent | OPT | N/A | OPT | OPT | OPT | — | — |
| MTA HAL interface | rdkb-halif-mta | OPT | N/A | OPT | OPT | OPT | — | — |
| MTA HAL test | rdkb-halif-test-mta | OPT | N/A | OPT | OPT | OPT | — | — |
| Telco Voice |  |  |  |  |  |  |  |  |
| TR104 telco voice stack mgmt | telco-voice-manager | OPT | OPT | OPT | OPT | OPT | — | — |
| Battery Backup |  |  |  |  |  |  |  |  |
| Battery-backed power modes ▸ For MTA battery scenarios | power-manager | — | N/A | OPT | OPT | OPT | — | — |
| Power manager HAL | rdk-halif-power_manager | — | N/A | OPT | OPT | OPT | — | — |
| Power manager HAL test | rdk-halif-test-power_manager | — | N/A | OPT | OPT | OPT | — | — |

## 23.4 Component Dependencies

The following RDK-B components are required or closely related to this feature:

- media-terminal-adapter-agent

- rdkb-halif-mta

- rdkb-halif-test-mta

- telco-voice-manager

- power-manager

- rdk-halif-power_manager

## 23.5 Implementation Notes

MTA status & diagnostics: Packet Cable 2.0 eDVA

Battery-backed power modes: For MTA battery scenarios

## 23.6 Testing Scope

- MTA HAL: all mandatory rdkb-halif-mta functions validated by test suite

- MTA registration state monitoring: validate state transitions are reported accurately

- Battery backup: validate power mode transitions and runtime estimation

- TR-104: SIP registration state and line status reporting accuracy

- MTA diagnostics: validate loopback and tone test trigger/result via RBUS

# 24. Feature 13: Firmware Upgrade

Sub-System: Device Mgmt    Primary Repository: https://github.com/rdkcentral/platform-manager

## 24.1 Overview

Firmware Upgrade is a critical operational capability enabling operators to remotely update device software across their entire deployed fleet. Firmware updates deliver security patches, bug fixes, and new feature activations — maintaining device security and service quality throughout the device's operational lifetime, which commonly spans 5–10 years.

The platform-manager component handles the complete firmware upgrade lifecycle: receiving the upgrade request from the cloud, downloading the firmware image from the CDN, validating the image signature, applying the image to the inactive flash partition, and triggering the reboot to the new firmware. On failure, the device automatically falls back to the previous firmware version, ensuring the device remains operational.

The xConf client manages the cloud-side firmware orchestration: checking for available updates, receiving update schedules, managing maintenance windows, and reporting upgrade status back to the xConf service. This enables operators to perform phased rollouts to subsets of their fleet, reducing the risk of fleet-wide issues from a single bad firmware release.

## 24.2 Key Capabilities

- OTA firmware downloads with CDN redirect support and resume-on-failure

- Image signature verification before applying any firmware update

- Dual-partition (A/B) firmware update with automatic rollback on boot failure

- xConf-based cloud orchestration: update scheduling, maintenance windows, phased rollout

- Firmware upgrade HAL (rdkb-halif-fwupgrade): vendor-agnostic flash write abstraction

- HAL test suite for RDK Ready certification of firmware upgrade implementations

- Update status reporting via RBUS and T2 telemetry for fleet-wide upgrade tracking

## 24.3 Feature Detail and Profile Matrix

| Feature Detail | Repository | Modem /ONU | EthWAN WiFi Router | GW | GW OpenSync | GW EasyMesh | EXT OpenSync | EXT EasyMesh |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| OTA Upgrade |  |  |  |  |  |  |  |  |
| Firmware download & apply ▸ Platforms without CM Agent | platform-manager | — | OPT | OPT | OPT | OPT | OPT | OPT |
| FW upgrade HAL interface | rdkb-halif-fwupgrade | — | OPT | OPT | OPT | OPT | OPT | OPT |
| FW upgrade HAL test | rdkb-halif-test-fwupgrade | — | N/A | OPT | OPT | OPT | OPT | OPT |
| Cloud Connection |  |  |  |  |  |  |  |  |
| xConf-based FW orchestration | xconf-client | — | OPT | OPT | OPT | OPT | OPT | OPT |

## 24.4 Component Dependencies

The following RDK-B components are required or closely related to this feature:

- platform-manager

- xconf-client

- rdkb-halif-fwupgrade

- rdkb-halif-test-fwupgrade

## 24.5 Implementation Notes

Firmware download & apply: Platforms without CM Agent

## 24.6 Testing Scope

- FW upgrade HAL: all mandatory functions validated by rdkb-halif-test-fwupgrade

- End-to-end OTA: trigger via xConf, download, signature verify, apply, reboot, verify new version

- Rollback: inject controlled boot failure on new firmware, verify automatic rollback to previous

- Phased rollout: validate device checks eligibility against xConf criteria before applying

- Download resume: interrupt download mid-transfer, validate resume from checkpoint

- Signature failure: present deliberately corrupted image, validate rejection without flash write

# 25. Feature 14: Connected device discovery

Sub-System: LAN    Primary Repository: https://github.com/rdkcentral/lan-manager-lite

## 25.1 Overview

Connected Device Discovery maintains a real-time inventory of all devices connected to the gateway's LAN — both wired and wireless. This inventory is exposed through the TR-181 Device.Hosts data model table, which records each device's MAC address, IP address, hostname, connection type, and associated interface.

The lan-manager-lite component drives device discovery through multiple detection mechanisms: ARP monitoring for wired clients, DHCP lease tracking for IP assignment events, and Wi-Fi association event consumption from OneWiFi for wireless clients. Join and leave events are generated in near-real-time as devices connect and disconnect, enabling operators and applications to respond to fleet-level connectivity events.

Secure UPnP (secure-upnp) extends device visibility by providing UPnP service discovery while enforcing security policies that prevent malicious UPnP abuse. Applications and the local WebUI consume the Device.Hosts table to display connected device lists and enable per-device management features.

## 25.2 Key Capabilities

- Device.Hosts TR-181 table: real-time inventory of all LAN-connected devices

- Multi-method detection: ARP, DHCP lease events, Wi-Fi association events

- Device join/leave events with near-real-time latency from connection to discovery

- Per-device metadata: MAC address, IP address, hostname, interface type, active/inactive state

- Secure UPnP service discovery with security policy enforcement

- Integration with parental controls and local WebUI for per-device management actions

## 25.3 Feature Detail and Profile Matrix

| Feature Detail | Repository | Modem /ONU | EthWAN WiFi Router | GW | GW OpenSync | GW EasyMesh | EXT OpenSync | EXT EasyMesh |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Device.Hosts Table |  |  |  |  |  |  |  |  |
| Device.Hosts TR-181 data model | lan-manager-lite | — | Required | Required | Required | Required | — | — |
| Device Discovery |  |  |  |  |  |  |  |  |
| Detect devices joining/leaving LAN | lan-manager-lite | — | Required | Required | Required | Required | — | — |
| Secure device discovery |  |  |  |  |  |  |  |  |
| Secure UPnP services | secure-upnp | — | OPT | OPT | OPT | OPT | — | — |

## 25.4 Component Dependencies

The following RDK-B components are required or closely related to this feature:

- lan-manager-lite

- secure-upnp

- dhcp-manager

- OneWiFi

## 25.6 Testing Scope

- Device join detection: connect wired client and validate Device.Hosts entry within SLA

- Device leave detection: disconnect client and validate entry marked inactive within SLA

- Wi-Fi client: associate Wi-Fi device, validate Device.Hosts entry with correct SSID/BSSID

- DHCP integration: validate IP address and hostname from DHCP lease match Device.Hosts

- Secure UPnP: validate device discovery while testing for port mapping abuse prevention

# 26. Feature 15: System startup & orchestration

Sub-System: System    Primary Repository: https://github.com/rdkcentral/provisioning-and-management

## 26.1 Overview

System Startup and Orchestration manage the complete device boot sequence, from kernel initialization through to all RDK-B middleware components reaching their operational state. The startup framework must correctly sequence service dependencies, handle partial boot failures, and emit system-ready signals at appropriate milestones.

The provisioning-and-management (P&M) scripts package is the incumbent startup implementation, containing the shell scripts that initialize interfaces, apply persistent configuration, start service daemons, and configure networking at boot time. P&M is acknowledged as a legacy component — it is monolithic, difficult to extend, and scheduled for refactoring into smaller, modular components.

The Utopia framework provides the sysEvent IPC mechanism used by P&M scripts for inter-script communication during the boot sequence, and the sysConfig key-value store for persistent configuration. Like P&M, Utopia is a legacy component with planned deprecation. The component-registry (system-ready signaling) and miscellaneous-broadband (platform init) components provide the non-legacy parts of the startup infrastructure.

## 26.2 Key Capabilities

- Boot sequence orchestration: ordered service startup with dependency resolution

- sysEvent IPC: event-based inter-script communication for boot stage coordination

- sysConfig: persistent key-value configuration store for network and system settings

- component-registry: system-ready event signaling for components waiting on dependencies

- miscellaneous-broadband: platform-specific initialization tasks (MAC setup, RNG seed, etc.)

- LED management: device status indication via LED state management component

## 26.3 Feature Detail and Profile Matrix

| Feature Detail | Repository | Modem /ONU | EthWAN WiFi Router | GW | GW OpenSync | GW EasyMesh | EXT OpenSync | EXT EasyMesh |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Startup Scripts |  |  |  |  |  |  |  |  |
| Device init & provisioning scripts ▸ Legacy – will be split/deprecated | provisioning-and-management | CORE | Required | Required | Required | Required | OPT | OPT |
| sysEvent IPC + sysConfig storage ▸ Legacy – deprecation planned | utopia | CORE | Required | Required | Required | Required | OPT | OPT |
| Service Registry |  |  |  |  |  |  |  |  |
| System-ready event signaling ▸ Legacy (dbus era) – not for new dev | component-registry | — | Required | Required | Required | Required | Required | Required |
| Platform Misc |  |  |  |  |  |  |  |  |
| Miscellaneous platform init ▸ Refactor candidate | miscellaneous-broadband | — | Required | Required | Required | Required | Required | Required |
| LED Management |  |  |  |  |  |  |  |  |
| LED state management | led-manager | — | OPT | OPT | OPT | OPT | OPT | OPT |

## 26.4 Component Dependencies

The following RDK-B components are required or closely related to this feature:

- provisioning-and-management

- utopia

- component-registry

- miscellaneous-broadband

- led-manager

## 26.5 Implementation Notes

Device init & provisioning scripts: Legacy – will be split/deprecated

sysEvent IPC + sysConfig storage: Legacy – deprecation planned

System-ready event signaling: Legacy (dbus era) – not for new dev

Miscellaneous platform init: Refactor candidate

## 26.6 Testing Scope

- Cold boot: validate all CORE services reach operational state within boot time ATC

- sysEvent: validate events are delivered to registered listeners during boot sequence

- component-registry: validate system-ready signal is emitted only when all CORE components are up

- LED management: validate correct LED state transitions at each boot stage

- Power cycle soak: validate clean boot/shutdown cycle across 100 iterations without hang

# 27. Feature 16: Configuration Persistence

Sub-System: System    Primary Repository: https://github.com/rdkcentral/persistent-storage-manager

## 27.1 Overview

Configuration Persistence ensures that all device settings configured by operators, subscribers, or management systems survive device reboots, power cycles, and controlled resets. Without reliable persistence, every reboot would return the device to factory defaults, requiring complete reconfiguration.

PSM (Persistent Storage Manager) is the primary, preferred persistence mechanism for RDK-B. It provides a typed key-value store with RBUS-accessible get/set/notify APIs, factory-default management, and atomic write semantics to prevent configuration corruption on unexpected power loss. PSM is the target storage backend for all new RDK-B components.

sysConfig (via Utopia) is the legacy flat-file key-value persistence mechanism still used by P&M scripts and older components. It is being progressively replaced by PSM but remains required for backward compatibility with existing components during the migration period.

## 27.2 Key Capabilities

- PSM: typed key-value persistence with RBUS APIs, factory-default management, atomic writes

- PSM factory reset: restore all PSM-managed parameters to their factory default values

- sysConfig: legacy flat-file key-value store for P&M script compatibility

- Power-loss safe writes: atomic operations prevent configuration corruption on unexpected shutdown

- Component-specific namespace partitioning prevents configuration collisions

## 27.3 Feature Detail and Profile Matrix

| Feature Detail | Repository | Modem /ONU | EthWAN WiFi Router | GW | GW OpenSync | GW EasyMesh | EXT OpenSync | EXT EasyMesh |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Config Storage |  |  |  |  |  |  |  |  |
| Persistent config + factory reset ▸ Not all components use PSM | persistent-storage-manager | CORE | Required | Required | Required | Required | — | — |
| sysConfig |  |  |  |  |  |  |  |  |
| Persistent config ▸ Not all components use sysConfig | utopia | CORE | Required | Required | Required | Required | — | — |

## 27.4 Component Dependencies

The following RDK-B components are required or closely related to this feature:

- persistent-storage-manager

- utopia

## 27.5 Implementation Notes

Persistent config + factory reset: Not all components use PSM

Persistent config: Not all components use sysConfig

## 27.6 Testing Scope

- PSM write/reboot/read cycle: validate value survives power cycle

- Factory reset: validate PSM returns all values to factory defaults

- Power-loss simulation: interrupt write mid-operation, validate config integrity on next boot

- Concurrent access: multiple components writing to PSM simultaneously — validate no corruption

- sysConfig legacy compatibility: validate existing P&M scripts read/write sysConfig correctly

# 28. Feature 17: Factory reset

Sub-System: System    Primary Repository: https://github.com/rdkcentral/persistent-storage-manager

## 28.1 Overview

Factory Reset restores the device to its original factory-default software configuration state, clearing all subscriber-specific settings, operator-applied configurations, and persisted application data. Factory reset is a critical operational tool used for device reuse (subscriber moves), defect resolution, and end-of-life data erasure.

RDK-B implements factory reset through a coordinated multi-component process: PSM restores all its managed parameters to their recorded factory defaults; Utopia/sysConfig clears its persistent configuration files; P&M scripts execute platform-specific reset actions such as clearing wireless credentials and resetting network interface configuration; and the device reboots to apply the clean state.

Factory reset can be triggered through multiple paths: local physical button press, local WebUI action, remote management protocol trigger (USP, TR-069), or RFC/xConf command. All trigger paths must produce an identical end state.

## 28.2 Key Capabilities

- PSM factory default restoration: all PSM parameters restored to their factory-default values

- sysConfig/Utopia erasure: persistent configuration files cleared

- P&M reset scripts: platform-specific reset actions (Wi-Fi credentials, network config)

- Multiple trigger paths: physical button, WebUI, USP/TR-069 command, RFC/xConf

- Controlled reboot: device restarts cleanly after reset completion

## 28.3 Feature Detail and Profile Matrix

| Feature Detail | Repository | Modem /ONU | EthWAN WiFi Router | GW | GW OpenSync | GW EasyMesh | EXT OpenSync | EXT EasyMesh |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Config Reset |  |  |  |  |  |  |  |  |
| Restore factory default config ▸ PSM manages factory defaults | persistent-storage-manager/utopia/PSM | CORE | Required | Required | Required | Required | — | — |
| Factory reset scripts ▸ Via Utopia/P&M scripts | persistent-storage-manager/utopia/PSM | CORE | Required | Required | Required | Required | — | — |

## 28.4 Component Dependencies

The following RDK-B components are required or closely related to this feature:

- persistent-storage-manager

- utopia

- provisioning-and-management

## 28.5 Implementation Notes

Restore factory default config: PSM manages factory defaults

Factory reset scripts: Via Utopia/P&M scripts

## 28.6 Testing Scope

- Factory reset from each trigger path (button, WebUI, USP, TR-069) — validate identical end state

- Post-reset state: validate all Wi-Fi credentials, IP settings, and parental controls are cleared

- Post-reset boot: device boots cleanly and reaches operational state after reset

- Factory reset timing: reset completes within 180s ATC

- Multi-cycle: 10 consecutive factory reset cycles without degradation

# 29. Feature 18: IP Routing/NAT/Firewall

Sub-System: Routing & Bridging    Primary Repository: https://github.com/rdkcentral/core-net-library

## 29.1 Overview

IP Routing, NAT, and Firewall form the core data-plane of the RDK-B gateway, responsible for forwarding traffic between WAN and LAN interfaces, translating private LAN addresses to the public WAN address, and protecting LAN devices from unsolicited inbound traffic.

IPv4 and IPv6 routing rules are managed by provisioning-and-management scripts working in concert with the WAN Manager. When the WAN interface type changes (e.g., failover from primary to secondary WAN), the routing table is updated accordingly. The core-net-library provides a zero-dependency utility library for interface, bridge, VLAN, and route management operations used by multiple RDK-B components.

The stateful packet filter (firewall) is implemented using iptables (and transitionally nftables) to enforce security policies protecting LAN devices. NAT/PAT (Network Address Translation/Port Address Translation) maps LAN device connections through the single WAN IP. Port forwarding and port triggering rules enable LAN-hosted services to be accessible from the WAN.

## 29.2 Key Capabilities

- IPv4 + IPv6 dual-stack routing rules with WAN interface change tracking

- Stateful packet filtering (iptables/nftables): SYN flood protection, connection tracking, zone-based policies

- NAT/PAT: LAN-to-WAN address translation for all LAN devices through single WAN IP

- Port forwarding: static mapping of WAN port to LAN device/port for hosted services

- Port triggering: dynamic port mapping activated by outbound trigger traffic

- core-net-library: zero-dependency interface, bridge, VLAN, and route utility functions

- DMZ host configuration: single LAN device exposed to all WAN ports

## 29.3 Feature Detail and Profile Matrix

| Feature Detail | Repository | Modem /ONU | EthWAN WiFi Router | GW | GW OpenSync | GW EasyMesh | EXT OpenSync | EXT EasyMesh |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Network Utilities |  |  |  |  |  |  |  |  |
| Interface/bridge/VLAN/route utils ▸ Zero dependencies | core-net-library | — | Required | Required | Required | Required | Required | OPT |
| IPv4/IPv6 Routing |  |  |  |  |  |  |  |  |
| IPv4 + IPv6 routing rules ▸ Legacy – still required today | provisioning-and-management | CORE | Required | Required | Required | Required | — | — |
| Route management utilities | core-net-library/Utopia | — | Required | Required | Required | Required | Required | OPT |
| Firewall |  |  |  |  |  |  |  |  |
| Stateful packet filtering | provisioning-and-management,Utopia | CORE | Required | Required | Required | Required | — | — |
| NAT / PAT | provisioning-and-management/Utopia | CORE | Required | Required | Required | Required | — | — |
| Port forwarding |  |  |  |  |  |  |  |  |
| Port forwarding rules ▸ Via P&M iptables rules | provisioning-and-management/Firewall/Utopia | — | OPT | Required | Required | Required | — | — |
| Port triggering rules ▸ Via P&M iptables rules | provisioning-and-management/Firewall/Utopia | — | OPT | Required | Required | Required | — | — |

## 29.4 Component Dependencies

The following RDK-B components are required or closely related to this feature:

- core-net-library

- provisioning-and-management

- utopia

- wan-manager

## 29.5 Implementation Notes

Interface/bridge/VLAN/route utils: Zero dependencies

IPv4 + IPv6 routing rules: Legacy – still required today

Port forwarding rules: Via P&M iptables rules

Port triggering rules: Via P&M iptables rules

## 29.6 Testing Scope

- IPv4 routing: validate LAN client can reach internet via WAN after gateway boot

- IPv6 routing: validate LAN client receives IPv6 prefix and can reach IPv6 internet

- NAT: validate multiple LAN clients can simultaneously access the internet through one WAN IP

- Firewall: validate unsolicited inbound SYN packets are dropped at WAN interface

- Port forwarding: configure rule, validate external access to LAN-hosted service on specified port

- Port triggering: trigger outbound traffic, validate dynamic inbound port mapping activation

- WAN failover: validate routing table updates correctly when primary WAN fails

# 30. Feature 19: Bridge and VLAN configuration

Sub-System: Routing & Bridging    Primary Repository: https://github.com/rdkcentral/wan-manager https://github.com/rdkcentral/miscellaneous-broadband/

## 30.1 Overview

Bridge and VLAN Configuration enables the gateway to operate in network topologies beyond the standard NAT router mode. WAN bridge mode disables the NAT and routing functions, passing frames transparently between WAN and LAN interfaces — used by operators for scenarios where a separate router handles IP routing.

VLAN configuration enables traffic segmentation across the gateway's LAN and WAN interfaces, supporting operator use cases such as separating residential traffic from business traffic on a shared access link, or isolating IoT device traffic from primary LAN devices.

## 30.2 Key Capabilities

- WAN bridge mode: transparent MAC-level bridging between WAN and LAN without NAT

- VLAN tagging and untagging on LAN and WAN interfaces

- Bridge interface creation and management for multi-interface bridging scenarios

- VLAN-to-interface mapping for traffic segmentation

## 30.3 Feature Detail and Profile Matrix

| Feature Detail | Repository | Modem /ONU | EthWAN WiFi Router | GW | GW OpenSync | GW EasyMesh | EXT OpenSync | EXT EasyMesh |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bridge mode support (no router/NAT) |  |  |  |  |  |  |  |  |
| WAN bridge mode configuration | wan-manager/bridge utils | — | OPT | OPT | OPT | OPT | — | — |
| Bridge Utilities |  |  |  |  |  |  |  |  |
| VLAN/bridge interface management | core-net-library | — | Required | Required | Required | Required | — | — |

## 30.4 Component Dependencies

The following RDK-B components are required or closely related to this feature:

- wan-manager

- core-net-library

- miscellaneous-broadband

## 30.6 Testing Scope

- WAN bridge mode: validate LAN client receives IP address from upstream DHCP server (not gateway)

- VLAN: validate tagged traffic on VLAN-configured interface is isolated from untagged traffic

- Bridge: validate frames bridge correctly between two specified interfaces

# 31. Feature 20: Traffic redirection

Sub-System: Routing & Bridging    Primary Repository: —

## 31.1 Overview

Traffic Redirection enables the gateway to intercept subscriber HTTP traffic and redirect it to a captive portal for operator notification or authentication purposes. This is used in "no service" scenarios — when the WAN connection is technically functional, but the subscriber's service has been suspended, restricted, or not yet activated.

Two redirection modes are supported: network-based captive portal redirection (forwarding to an operator-hosted portal on the WAN), and local captive portal redirection (serving a locally hosted portal when there is no DOCSIS RF WAN signal at all, i.e., in full offline scenarios).

## 31.2 Key Capabilities

- Network-based captive portal: redirect HTTP traffic to operator-hosted notification portal

- Local captive portal: serve locally hosted portal page when no WAN connectivity is present

- DNS-based redirect for portal URL resolution before WAN routing is available

## 31.3 Feature Detail and Profile Matrix

| Feature Detail | Repository | Modem /ONU | EthWAN WiFi Router | GW | GW OpenSync | GW EasyMesh | EXT OpenSync | EXT EasyMesh |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Forward user traffic to network based captive portal |  |  |  |  |  |  |  |  |
| For "no service" and other similar use cases | provisioning-and-management | — | OPT | OPT | OPT | OPT | — | — |
| Forward user traffic to local Captive portal when no DOCSIS RF WAN |  |  |  |  |  |  |  |  |
| No RF Captive portal | provisioning-and-management | — | N/A | OPT | OPT | OPT | — | — |

## 31.4 Component Dependencies

The following RDK-B components are required or closely related to this feature:

- provisioning-and-management

## 31.6 Testing Scope

- Network captive portal: simulate no-service WAN state, validate HTTP redirect to portal URL

- Local portal: simulate no-RF state, validate locally hosted page served on redirect

- HTTPS handling: validate redirect behavior for HTTPS requests (browser certificate warning expected)

# 32. Feature 21: DHCP Server

Sub-System: LAN    Primary Repository: https://github.com/rdkcentral/dhcp-manager

## 32.1 Overview

DHCP Server provides automatic IP address assignment and network configuration delivery to all LAN-connected devices. The dhcp-manager component implements both DHCPv4 and DHCPv6 server functionality, handling the complete address lifecycle: discover/offer/request/acknowledge for IPv4, and solicit/advertise/request/reply for IPv6.

IPv6 prefix delegation (PD) enables the gateway to request an IPv6 prefix from the upstream ISP network and sub-delegate portions of that prefix to LAN devices for SLAAC or DHCPv6 address formation. When the WAN connection fails over to a secondary WAN, the delegated prefix may change — dhcp-manager handles this transition gracefully, renewing LAN device prefixes with minimal service interruption.

IPv6 SLAAC (Stateless Address Autoconfiguration) support enables LAN devices to self-configure IPv6 addresses from the delegated prefix using their interface MAC address, without requiring explicit DHCP exchange. This is complementary to DHCPv6 and both may be active simultaneously.

## 32.2 Key Capabilities

- DHCPv4 server: address pool management, lease allocation, renewal, and release

- DHCPv6 server: IA_NA (non-temporary address) and IA_PD (prefix delegation) support

- DHCPv4 options: DNS servers, domain name, NTP server, vendor-specific options (43, 60, 82)

- IPv6 prefix delegation (PD): upstream prefix request and LAN sub-delegation

- PD change handling on WAN failover/restore; graceful prefix transition to LAN clients

- IPv6 SLAAC: Router Advertisement with delegated prefix for stateless address configuration

- Static DHCP lease assignment based on MAC address for known LAN devices

## 32.3 Feature Detail and Profile Matrix

| Feature Detail | Repository | Modem /ONU | EthWAN WiFi Router | GW | GW OpenSync | GW EasyMesh | EXT OpenSync | EXT EasyMesh |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DHCP Server for LAN Clients |  |  |  |  |  |  |  |  |
| DHCPv4 server | dhcp-manager | — | Required | Required | Required | Required | — | — |
| DHCPv6 servier | dhcp-manager | — | OPT | OPT | OPT | OPT | — | — |
| IPv6 prefix delegation | dhcp-manager | CORE | Required | Required | Required | Required | — | — |
| IPv6 prefix delegation change on WAN failover/restore | dhcp-manager | — | OPT | — | — | — | — | — |
| IPv6 SLAAC support (stateless address autoconfig) | dhcp-manager | CORE | Required | Required | Required | Required | — | — |

## 32.4 Component Dependencies

The following RDK-B components are required or closely related to this feature:

- dhcp-manager

- wan-manager

## 32.6 Testing Scope

- DHCPv4: client discovers, receives offer, requests, and receives ACK with valid IP in pool

- DHCPv4 lease renewal: client renews lease before expiry, validate same IP retained

- DHCPv6 IA_NA: client receives unique IPv6 address in delegated prefix

- Prefix delegation: gateway requests /56 from upstream, delegates /64 to LAN

- PD change on WAN failover: trigger failover, validate LAN clients receive updated prefix

- SLAAC: validate Router Advertisement is sent with correct prefix and prefix length

- DHCP options: validate DNS server option matches gateway DNS relay address

# 33. Feature 22: DNS

Sub-System: DNS    Primary Repository: https://github.com/rdkcentral/dhcp-manager

## 33.1 Overview

DNS services in RDK-B provide local name resolution for LAN clients, enabling them to use the gateway as their DNS resolver rather than communicating directly with upstream DNS servers. The gateway acts as a DNS relay, forwarding queries it cannot resolve locally to configured upstream DNS servers.

DNS-based parental controls (xdns) extend the local DNS relay with content filtering capability, enabling subscribers and operators to block access to categories of content or specific domains by returning negative responses to DNS queries for blocked domains. xdns integrates with configurable upstream DNS filtering services that categorize domains by content type.

Dynamic DNS (DDNS) enables subscribers with dynamic WAN IP addresses to maintain a stable DNS hostname that resolves to their gateway's current WAN IP, enabling remote access to home-hosted services without a fixed IP address.

## 33.2 Key Capabilities

- DNS relay: forward LAN client queries to configured upstream DNS servers

- Local DNS override: respond to specific hostname queries with locally configured addresses

- DNS-based parental controls (xdns): category-based content filtering via upstream filtering DNS

- Per-device DNS filtering policy: different filtering rules per LAN device MAC address

- DDNS: dynamic DNS record update on WAN IP change via standard DDNS provider APIs

- DNS-over-HTTPS (DoH) proxy support for privacy-preserving resolution

## 33.3 Feature Detail and Profile Matrix

| Feature Detail | Repository | Modem /ONU | EthWAN WiFi Router | GW | GW OpenSync | GW EasyMesh | EXT OpenSync | EXT EasyMesh |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DNS |  |  |  |  |  |  |  |  |
| DNS relay for LAN clients | dhcp-manager/PandM | — | OPT | Required | Required | Required | — | — |
| DNS-based parental controls | xdns | — | OPT | OPT | OPT | OPT | — | — |
| DDNS | provisioning-and-management/Utopia | — | OPT | OPT | OPT | OPT | — | — |

## 33.4 Component Dependencies

The following RDK-B components are required or closely related to this feature:

- dhcp-manager

- provisioning-and-management

- xdns

- utopia

## 33.6 Testing Scope

- DNS relay: LAN client query forwarded to upstream and response returned correctly

- DNS negative caching: validate NXDOMAIN is cached and not re-queried for TTL period

- xdns: configure block list, attempt query for blocked domain, validate blocked response

- DDNS: trigger WAN IP change, validate DDNS provider receives update within SLA

- Upstream failover: block primary DNS, validate failover to secondary DNS upstream

# 34. Feature 23: WAN Connectivity

Sub-System: WAN    Primary Repository: https://github.com/rdkcentral/wan-manager

## 34.1 Overview

WAN Connectivity is the broadest feature area in RDK-B, encompassing all access technologies, WAN protocol support, multi-WAN orchestration with failover, IPv6 transition mechanisms, and DHCP client-side address acquisition. The wan-manager component is the central orchestrator for all WAN connectivity, managing a configurable set of primary and secondary WAN profiles and handling failover/restore transitions transparently.

Access technology support covers the full range of broadband access types: Ethernet WAN (for fiber and Ethernet-delivered services), DOCSIS cable modem (cable-modem-agent), xDSL (xdsl-manager), GPON (gpon-manager), EPON (CcspEPONAgent, legacy), Qualcomm QMI cellular (cellular-manager), MBIM generic cellular (Cellular Modem Mgr), USB RNDIS cellular dongles, and Wi-Fi as a WAN backup link. Each access technology is managed by a dedicated agent that exposes WAN link state, signal quality, and statistics via RBUS.

IPv6 transition mechanisms address the challenge of providing IPv6 connectivity to LAN clients in networks where the ISP only provides IPv4 WAN addresses. MAP-T (Mapping of Address and Port using Translation) and MAP-E (encapsulation variant) translate IPv6 packets to IPv4+port for upstream transport. DS-Lite encapsulates IPv4 within IPv6. 464XLAT (CLAT on the gateway, PLAT in the network) enables IPv4 applications to work over an IPv6-only WAN.

## 34.2 Key Capabilities

- WAN profile orchestration: configurable primary and secondary WAN profiles with automatic failover

- WAN failover: primary failure detection within 30s, secondary activation within 60s

- Ethernet WAN: managed by Ethernet Agent for fiber/cable handoff scenarios

- DOCSIS: cable-modem-agent for cable modem status, events, and CM-status reporting

- xDSL: xdsl-manager for ADSL/VDSL modem status, line statistics, and diagnostics

- GPON: gpon-manager for GPON ONU registration, signal level, and link state

- EPON: CcspEPONAgent for EPON ONU management (legacy)

- Cellular QMI: cellular-manager for Qualcomm QMI modem management, data sessions, GPS

- Cellular MBIM: generic MBIM modem management for non-Qualcomm cellular modems

- USB RNDIS: cellular dongle connectivity via USB RNDIS network interface

- Wi-Fi WAN: OneWiFi STA mode as an upstream WAN link for backup connectivity

- DHCP client IPv4: WAN address acquisition, renewal, and release

- DHCP client IPv6/PD: upstream prefix delegation request

- PPPoE: PPP over Ethernet for DSL and some fiber operator deployments

- GRE tunnelling: WAN GRE tunnel for public hotspot encrypted WAN scenarios

- MAP-T / MAP-E: IPv4 traffic mapping over IPv6 WAN with shared address space

- DS-Lite: IPv4-in-IPv6 encapsulation for IPv6-only WAN access networks

- 464XLAT: CLAT function enabling IPv4 applications over IPv6-only WAN

## 34.3 Feature Detail and Profile Matrix

| Feature Detail | Repository | Modem /ONU | EthWAN WiFi Router | GW | GW OpenSync | GW EasyMesh | EXT OpenSync | EXT EasyMesh |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Profile based definition of primary and backup WAN links |  |  |  |  |  |  |  |  |
| WAN profile orchestration | wan-manager | — | Required | Required | Required | Required | — | — |
| Primary WAN |  |  |  |  |  |  |  |  |
| Primary WAN | wan-manager | — | Required | — | — | — | — | — |
| Secondary WAN |  |  |  |  |  |  |  |  |
| Backup WAN | wan-manager | — | OPT | Required | Required | Required | — | — |
| WAN Failover and Restore |  |  |  |  |  |  |  |  |
| Failover and restore use cases | wan-manager | — | OPT | — | — | — | — | — |
| DHCP Client (WAN-side IP acquisition) |  |  |  |  |  |  |  |  |
| DHCP Client – WAN IPv4 acquisition | dhcp-manager | — | OPT | Required | Required | Required | — | — |
| DHCP Client – WAN IPv6 / prefix delegation | dhcp-manager | — | Required | Required | Required | Required | — | — |
| IPv6 transition/translation mechanisms |  |  |  |  |  |  |  |  |
| MAP-T | DHCP Manager | — | OPT | — | — | — | — | — |
| MAP-E | DHCP Manager | — | OPT | — | — | — | — | — |
| DSLITE | DHCP Manager | — | OPT | — | — | — | — | — |
| 464XLAT | DHCP Manager | — | OPT | OPT | OPT | OPT | — | — |
| WAN Protocol – PPP |  |  |  |  |  |  |  |  |
| PPP over Ethernet / ATM | ppp-manager | — | OPT | OPT | OPT | OPT | — | — |
| WAN Protocol – GRE |  |  |  |  |  |  |  |  |
| WAN GRE tunnel | hotspot | — | OPT | — | — | — | — | — |
| Access Technology - Ethernet WAN |  |  |  |  |  |  |  |  |
| Ethernet WAN interface mgmt | Ethernet Agent | — | Required | — | — | — | — | — |
| Access Technology – DOCSIS |  |  |  |  |  |  |  |  |
| Cable modem status & events | cable-modem-agent | OPT | N/A | OPT | OPT | OPT | — | — |
| Access Technology – xDSL |  |  |  |  |  |  |  |  |
| xDSL modem status & diag | xdsl-manager | OPT | N/A | OPT | OPT | OPT | — | — |
| Access Technology – EPON |  |  |  |  |  |  |  |  |
| EPON ONU management ▸ Legacy | CcspEPONAgent | OPT | N/A | OPT | OPT | OPT | — | — |
| Access Technology – GPON |  |  |  |  |  |  |  |  |
| GPON WAN interface mgmt | gpon-manager | OPT | N/A | OPT | OPT | OPT | — | — |
| Access Technology – Cellular |  |  |  |  |  |  |  |  |
| Qualcomm QMI cellular | cellular-manager | — | N/A | OPT | OPT | OPT | — | — |
| GPS ▸ Location detection | cellular-manager | — | N/A | OPT | OPT | OPT | — | — |
| MBIM generic cellular | Cellular Modem Mgr | — | N/A | OPT | OPT | OPT | — | — |
| USB RNDIS | Cellular Modem Mgr | — | N/A | OPT | OPT | OPT | — | — |
| Access Technolgoy - WiFi link to external WAN backup device |  |  |  |  |  |  |  |  |
| WiFi used as a WAN link | OneWiFi WAN Manager | — | OPT | — | — | — | — | — |

## 34.4 Component Dependencies

The following RDK-B components are required or closely related to this feature:

- wan-manager

- dhcp-manager

- Ethernet Agent

- cable-modem-agent

- xdsl-manager

- gpon-manager

- cellular-manager

- cellular-modem-manager

- ppp-manager

- hotspot

- OneWiFi

## 34.5 Implementation Notes

EPON ONU management: Legacy

GPS: Location detection

## 34.6 Testing Scope

- Ethernet WAN: DHCP address acquisition and internet reachability

- WAN failover: primary WAN link-down event triggers secondary within SLA

- WAN restore: primary WAN recovers, validate failback and routing table correction

- DHCP client IPv4: discover/offer/request/ack cycle, lease renewal, expiry handling

- DHCP client IPv6/PD: upstream PD request and /64 sub-delegation to LAN

- MAP-T: configure MAP-T rule, validate IPv4 traffic is correctly mapped to IPv6 for WAN transit

- 464XLAT: validate IPv4-only application connectivity over IPv6-only WAN

- PPPoE: dial-up, authentication, and DNS acquisition via PPP

- Cellular: QMI data session establishment, IP address assignment, and internet reachability

# 35. Feature 24: Parental control

Sub-System: Parental Control    Primary Repository: https://github.com/rdkcentral/xdns

## 35.1 Overview

Parental Controls provide operators and subscribers with tools to manage and restrict internet access on a per-device basis, protecting children from inappropriate content and enforcing household internet usage policies.

DNS-based filtering (xdns) intercepts DNS queries and returns blocked responses for domains in operator-configured content categories. This approach is effective, lightweight, and cross-platform — it works for any device on the LAN regardless of operating system, without requiring any client-side software installation.

Device Pause allows subscribers to instantly cut internet access for a specific LAN device without disconnecting it from the network or changing any device settings. Managed sites, blocked keywords, and trusted device exemptions provide fine-grained control over which sites and devices are subject to filtering policies.

## 35.2 Key Capabilities

- DNS-based filtering (xdns): category-based content blocking for all LAN devices

- Device Pause: per-device internet access pause without network disconnection

- Managed sites: per-device allow/block site lists overriding default policy

- Blocked keywords: URL keyword-based blocking for domains matching configured strings

- Trusted devices: exemption list for devices that bypass all parental control filtering

- Scheduling: time-of-day and day-of-week filtering schedules per device

## 35.3 Feature Detail and Profile Matrix

| Feature Detail | Repository | Modem /ONU | EthWAN WiFi Router | GW | GW OpenSync | GW EasyMesh | EXT OpenSync | EXT EasyMesh |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DNS Filtering |  |  |  |  |  |  |  |  |
| DNS-based parental control rules | xdns | — | OPT | OPT | OPT | OPT | — | — |
| Device Pause |  |  |  |  |  |  |  |  |
| Device Pause | provisioning-and-management/Utopia | — | OPT | Required | Required | Required | — | — |
| Managed sites |  |  |  |  |  |  |  |  |
| Managed sites | provisioning-and-management/Utopia | — | OPT | Required | Required | Required | — | — |
| Blocked keywords |  |  |  |  |  |  |  |  |
| Blocked keywords | provisioning-and-management/Utopia | — | OPT | Required | Required | Required | — | — |
| Trusted devices |  |  |  |  |  |  |  |  |
| Trusted devices | provisioning-and-management/Utopia | — | OPT | Required | Required | Required | — | — |

## 35.4 Component Dependencies

The following RDK-B components are required or closely related to this feature:

- xdns

- provisioning-and-management

- utopia

## 35.6 Testing Scope

- DNS filtering: configure category block, query blocked domain from LAN, validate blocked response

- Device Pause: pause specific device, validate internet blocked only for that MAC

- Trusted device: add device to trusted list, validate filtering bypassed for that MAC only

- Managed sites: add domain to per-device block list, validate blocked from that device only

- Keyword block: configure keyword, validate URL containing keyword is blocked

# 36. Feature 25: WiFi Hotspot

## 36.1 Overview

WiFi Hotspot provides public Wi-Fi SSID support and Passpoint / Hotspot 2.0 capability for operator deployments. This feature enables ISPs to offer public hotspot access over the same CPE used for residential broadband, supporting GRE tunnel-based traffic isolation to the operator backhaul. Encrypted WAN is required for security when deploying Public Hotspot functionality.

## 36.2 Key Capabilities

Public Wi-Fi SSID (hotspot). Passpoint / Hotspot 2.0 via OneWiFi (GAS/ANQP query/response, Hotspot 2.0 indication element). WAN GRE tunnel for traffic isolation to operator backhaul (hotspot). Radius Failover and IPv6 support for public access.

## 36.3 Feature Detail and Profile Matrix

| Feature Detail | Repository | Modem /ONU | EthWAN WiFi Router | GW | GW OpenSync | GW EasyMesh | EXT OpenSync | EXT EasyMesh |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Public Wi-Fi SSID | hotspot | n/a | n/a | OPT | OPT | OPT | OPT | OPT |
| Passpoint / Hotspot 2.0 | OneWiFi | n/a | n/a | OPT | OPT | OPT | OPT | OPT |

## 36.4 Component Dependencies

hotspot, OneWiFi, wan-manager (GRE tunnel support)

## 36.5 Implementation Notes

Encrypted WAN (GRE tunnel to operator backhaul) is required for security when Public Hotspot is deployed. Public SSID must be fully isolated from residential LAN traffic. Passpoint / Hotspot 2.0 requires GAS/ANQP support in OneWiFi and a valid Hotspot 2.0 configuration from the operator.

# 37. Feature 26: Code Security

## 37.1 Overview

Code Security encompasses the cryptographic, privilege, and runtime protection mechanisms that harden the RDK-B platform against exploitation. It includes certificate management for mTLS, secure system call wrappers, privilege-dropping utilities, and AppArmor profiles. libSyscallWrapper is a CORE component — a small, zero-external-dependency library used as basic infrastructure by most other components, required across all device profiles.

## 37.2 Key Capabilities

Secure system call wrapper (libSyscallWrapper) — CORE, required across all profiles. Certificate selector (rdk-cert-config) for mTLS configuration. Secure crypto API wrapper stubs for ISP-specific implementations (rdk-cert-config). Security Services Architecture (rdkssa) for MSO-specific crypto and secure mount. Privilege dropping for non-root process operation (rdk-libunpriv). AppArmor mandatory access control profiles (rdkb-apparmor-profiles).

## 37.3 Feature Detail and Profile Matrix

| Feature Detail | Repository | Modem /ONU | EthWAN WiFi Router | GW | GW OpenSync | GW EasyMesh | EXT OpenSync | EXT EasyMesh |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| secure system call (libSyscallWrapper) | libSyscallWrapper | CORE | Required | Required | Required | Required | Required | Required |
| Certificate selector (mTLS) | rdk-cert-config | OPT | OPT | OPT | OPT | n/a | n/a | n/a |
| Privilege drop (rdk-libunpriv) | rdk-libunpriv | OPT | OPT | OPT | OPT | n/a | n/a | n/a |
| AppArmor profiles | rdkb-apparmor-profiles | OPT | OPT | OPT | OPT | n/a | n/a | n/a |

## 37.4 Component Dependencies

libSyscallWrapper (CORE), rdk-cert-config, rdkssa, rdk-libunpriv, rdkb-apparmor-profiles

## 37.5 Implementation Notes

libSyscallWrapper is a CORE dependency — a small, zero-external-dependency library forming basic infrastructure used by most RDK-B components. rdk-cert-config and rdkssa are optional and required only for mTLS deployments where ISP-specific crypto backends are needed. Privilege dropping via rdk-libunpriv should be applied to all new components running as non-root. AppArmor profiles provide mandatory access control and should be included in all production builds.

# 38. Security Framework

Security is a mandatory design requirement across the RDK-B platform. Every component is expected to meet defined security baselines. This section defines the security framework, requirements, and validation approach applicable to all RDK-B Core components and device profiles.

## 38.1 Security Principles

- Secure by Default: all interfaces, services, and credentials are locked down at factory default; no open ports, no default passwords, no insecure services enabled without explicit operator configuration

- Defense in Depth: security controls exist at multiple layers — hardware (secure boot), firmware (signed images), middleware (firewall, process isolation), management (TLS), and application (container isolation)

- Least Privilege: each process runs with the minimum required Linux capabilities and filesystem permissions; no unnecessary root processes

- Patch Currency: CVE tracking and patch application are mandatory for all CORE components; patch SLA is 30 days for Critical, 90 days for High severity

- Open Audit: all security-sensitive code is open-source and subject to community review

## 38.2 Wi-Fi Security Requirements

| Security Mode | Profile Requirement | Notes |
| --- | --- | --- |
| WPA2-Personal | CORE for GW, EthWAN WiFi Router, all EasyMesh/OpenSync profiles | Baseline minimum for all Wi-Fi-capable profiles |
| WPA2-Enterprise (802.1X) | OPT for all Wi-Fi profiles | RADIUS server integration required; not typically deployed on residential profiles |
| WPA3-Personal (SAE) | CORE for GW and above | Mandatory for all new gateway deployments; replaces WPA2-Personal as default on new SSIDs |
| WPA3-Enterprise | OPT | For high-security business gateway deployments |
| WPA3-Transition | CORE for GW and above | Required to support mixed WPA2/WPA3 client environments during transition |
| WPA3 Enhanced Open (OWE) | OPT | For public Wi-Fi SSID deployments; provides opportunistic encryption |
| WPA3 PCM | OPT | Post-Connection Management; additional security controls post-association |
| PMKSA Caching | CORE for all Wi-Fi profiles | Required for fast roaming in EasyMesh environments |

## 38.3 Management Channel Security

- All management protocols (WebPA, USP, TR-069, WebConfig) MUST use TLS 1.2 or higher — no plain-text management channels permitted

- Server certificate validation is mandatory for all TLS connections — self-signed certificates only permitted during manufacturing provisioning

- USP controller authentication uses mutual TLS (mTLS) — device certificate and controller certificate both validated

- Local WebUI: HTTPS only, with HSTS header; HTTP redirected to HTTPS; session tokens invalidated on logout

- RBUS: local socket access controlled by Linux file system permissions; remote RBUS bridge (inter-device-manager) requires authentication

## 38.4 Firmware Security

- All firmware images must be signed with operator-approved RSA-2048 or ECDSA-P256 key pair

- Signature verification is performed before any firmware image is written to flash storage

- Secure boot chain validates each boot stage (bootloader → kernel → rootfs) against stored signing keys

- OTA download must occur over HTTPS; HTTP downloads rejected even if signature verification passes

- A/B partition scheme ensures a verified fallback firmware is always available for automatic rollback

## 38.5 Container Security (App Framework)

- OCI container runtime (Dobby/crun) enforces Linux namespace isolation for all downloadable applications

- Containers run without host network access by default — network access explicitly granted per container by operator policy

- Root filesystem is mounted read-only for all containers; writable volumes are explicitly declared

- Linux capabilities are dropped to the minimum set required — CAP_NET_ADMIN and CAP_NET_RAW not granted by default

- Seccomp filters applied to all containers to restrict available system calls

- cgroup resource limits (CPU, memory, I/O) enforced per container to prevent denial-of-service from misbehaving apps

## 38.6 Security Testing Requirements

| Security Test Area | Test Requirements |
| --- | --- |
| Firewall Validation | Validate all default-blocked ports are unreachable from WAN; validate stateful connection tracking allows return traffic; validate SYN flood defense activates under load |
| Wi-Fi Security Handshake | Full WPA3-SAE handshake protocol trace; validate PMK derivation and MIC verification; test with non-compliant supplicant versions |
| Management Channel TLS | Certificate validation enforcement; TLS version downgrade prevention; cipher suite conformance |
| Firmware Signature | Reject tampered image (bit-flip in payload); reject image signed with expired key; reject image with unknown signer |
| Container Isolation | Attempt container escape via /proc, /sys, /dev; validate namespace isolation prevents host process visibility; validate filesystem isolation |
| Credential Security | Validate no default/blank passwords exist at factory; validate SSID credentials are cryptographically random |
| CVE Scanning | Static analysis (cppcheck, clang-tidy) for CORE components; OpenSSL, libcurl, kernel version currency checks against CVE database |

# 39. Performance Requirements

Performance requirements define the minimum acceptable operational characteristics for each device profile. These requirements represent the floor — production implementations should exceed these values.

## 39.1 System Performance

| Metric | Target | Measurement Method |
| --- | --- | --- |
| Cold Boot Time | < 120 seconds from power-on to fully operational (WAN up, Wi-Fi beaconing, RBUS ready) | Automated HIL timer from power-on to first successful RBUS get |
| Warm Restart Time | < 45 seconds from reboot trigger to operational state | Automated HIL timer from reboot signal to RBUS ready |
| WAN Failover Time | < 60 seconds from primary WAN failure to secondary WAN traffic forwarding | Automated ping continuous stream; measure gap duration |
| WAN Restore Time | < 30 seconds from primary WAN recovery to primary WAN selected | Automated ping measurement from link-up event to restore complete |
| Memory Usage at Idle | < 60% of available RAM consumed by RDK-B middleware at steady state | top/free command via remote_debugger; 30-minute steady-state measurement |
| CPU Usage at Idle | < 15% average CPU utilization with no active traffic or diagnostics | mpstat 30-second average via remote_debugger |
| RBUS get/set Latency | < 100ms round-trip for local TR-181 parameter get/set under normal load | Automated measurement across 1000 sequential get/set operations |
| Management Command SLA | Configuration change reflected in running config within 30 seconds of cloud delivery | USP command issued to ACS; timestamp when RBUS parameter reflects change |

## 39.2 Wi-Fi Performance Requirements

| Profile / Band | Minimum Throughput | Test Conditions |
| --- | --- | --- |
| Wi-Fi 6 (802.11ax) — 5 GHz | 600 Mbps aggregate (uplink + downlink) | Single client, 3m distance, anechoic chamber, iPerf3 TCP |
| Wi-Fi 6E (802.11ax) — 6 GHz | 800 Mbps aggregate | Single client, 3m distance, anechoic chamber, iPerf3 TCP |
| Wi-Fi 7 (802.11be) — 5+6 GHz MLO | 1.2 Gbps aggregate | Single client MLO capable, 3m, anechoic chamber, iPerf3 TCP |
| EasyMesh Backhaul (5 GHz) | 400 Mbps | Extender to gateway backhaul link, iPerf3 TCP |
| EasyMesh End-to-End | 300 Mbps | Client on extender through mesh to gateway WAN, iPerf3 TCP |
| Client Association Time | < 3 seconds from scan to associated and DHCP complete | Automated client association measurement, 10-trial average |
| Roaming Time (802.11kv) | < 150ms BSS transition time | FTAA (Fast Transition Authenticated) measurement, 10-trial average |

## 39.3 WAN Performance Requirements

| Metric | Target | Notes |
| --- | --- | --- |
| Routing Throughput | Line rate for the WAN interface (1 Gbps for GbE WAN, 2.5 Gbps for multi-gig) | IMIX traffic profile, hardware acceleration expected |
| NAT Sessions | Minimum 50,000 concurrent NAT sessions | Measured under sustained load with session table inspection |
| DHCP Lease Acquisition | < 10 seconds from DHCPDISCOVER to DHCPACK with IP assigned to LAN client | Measured on LAN client from DHCP request to IP reachable |
| IPv6 RA Response | < 2 seconds from client RS to receiving RA with prefix | Measured from RS send to prefix available on client interface |
| DNS Response Time | < 50ms for cached responses, < 200ms for forwarded queries | Measured from LAN client, against local resolver; 100-query average |

## 39.4 Reliability Requirements

| Metric | Target |
| --- | --- |
| Uptime / MTBF | > 99.9% uptime over 30-day fleet sample (< 43 minutes downtime per device per month) |
| Crash Rate | < 0.1% unplanned restarts per device per day |
| OTA Success Rate | > 99% successful firmware update completions; 100% rollback on failure (no brick) |
| Log Upload Reliability | > 99.5% DCM log upload success rate over 30-day fleet sample |
| Factory Reset Reliability | 100% clean post-reset boot across 50-cycle soak test |
| Memory Stability | No memory leak exceeding 1MB/hour over 7-day continuous operation soak |

# 40. Testing Core

Testing is a first-class citizen in the RDK-B platform. The Testing Core defines the mandatory validation framework for the RDK Platform layer — covering CORE component behavior, data model correctness, integration, regression stability, and production readiness. Every CORE component requires documented test coverage before release.

Testing Philosophy RDK-B platform testing is structured around "test the interface, not the implementation." Component tests validate TR-181 data model behavior and RBUS API correctness. Integration tests validate end-to-end feature correctness across the platform stack. This layering ensures failures are localized and attributable. HAL implementation testing is outside this scope and is governed by the HAL specification.

## 40.1 Test Framework Architecture

The RDK Platform Test Suite covers L2 and L3. L4 is operator-owned. L1 HAL validation is vendor-owned and governed separately.

| Layer | Description | Owner |
| --- | --- | --- |
| L4: End-to-End / Acceptance | Complete user scenarios from cloud management to device behavior. Operator ODQ campaigns. | Operator + SI |
| L3: Integration / Feature | Interactions between two or more CORE RDK-B components across the platform layer. | RDK Community + OEM |
| L2: Component / Unit | Single CORE component in isolation. TR-181 data model, RBUS API, state machine, sysevent behavior. | Component Owner |
| L1: HAL Validation | Governed by HAL specification and Vendor Test Suite. Out of scope for this section. | SoC Vendor |

## 40.2 RDK Platform Test Suite (Mandatory for all CORE Components — L2 + L3)

The RDK Platform Test Suite defines the mandatory test coverage for every CORE RDK-B component. A component cannot be declared CORE and cannot be included in a Core release without evidence of coverage across all six categories below.

### 40.2.1 Mandatory Test Categories

Category 1 — Data Model

- All owned TR-181 parameters are readable

- Parameter count is stable across builds — any deviation is a blocking regression

- TR-181 values are consistent with runtime state at all times

- Write operations reflect in runtime state within a defined time bound

Category 2 — State Machine

- Every state is reachable and externally observable via RBUS or sysevent

- Every transition fires on the correct trigger and completes within defined timing

- A stable state exists for every degraded or partial input condition — no thrash, no spin

Category 3 — RBUS and Events

- All get/set operations return correct values

- Events fire on state change within the defined timing window

- All registrations and subscriptions are restored correctly after component restart

- No orphaned subscriptions or resource leaks after restart or failure

Category 4 — E2E Scenarios

- Happy path — trigger to operational state

- Failover — primary fails, secondary activates within SLA

- Restore — primary recovers within SLA

- Restart recovery — component resumes correct state after restart

- Stress — 20 repeated cycles with no memory leak and no hung state

Category 5 — Negative and Boundary

- Component operates correctly when optional dependencies are absent

- Invalid inputs rejected and correct error returned

- Behavior correct at minimum and maximum values of all timeout and retry constants

- No resource leak after failure or teardown

Category 6 — Post-Reboot

- PSM-persisted configuration restored correctly after reboot

- Component reaches operational state within platform boot SLA

- All owned RBUS parameters and sysevent keys repopulated with live values after reboot

### 40.2.2 Timing SLA Gates

Every CORE component must define timing SLAs from source and enforce them as blocking CI gates:

- State transition time

- Feature activation time

- Failover and restore time

- Restart recovery time

- RBUS get/set latency

SLA values must be documented per component. CI fails if any SLA regresses.

### 40.2.3 Test Evidence Required for Release

For every CORE component the following must be attached to the release ticket:

- Test case list with pass/fail result per category

- Timing SLA values and CI gate results

- Stress cycle result (minimum 20 cycles)

- Post-reboot assertion result

- Known failures with root cause and disposition

No CORE component is release-eligible without this evidence.

## 40.3 OneWiFi Test Suite (OneWiFiTestSuite)

OneWiFiTestSuite (github.com/rdkcentral/OneWiFiTestSuite) is the dedicated platform test suite for the Wi-Fi LAN feature area, covering L2 and L3 for OneWiFi and EasyMesh components. It covers:

- Radio bring-up and configuration across all supported bands (2.4 / 5 / 5L / 5U / 6 GHz)

- Security: WPA2-Personal, WPA2-Enterprise, WPA3-Personal (SAE), WPA3-Enterprise, WPA3-Transition, WPA3 Enhanced Open (OWE), WPA3-PCM

- Multi-BSSID (VAP) creation, configuration, and isolation

- DFS radar detection and channel switch announcement (CSA) compliance

- 802.11kv neighbor reports and BSS transition management frames

- EasyMesh R6: Controller/Agent pairing, topology convergence, STA steering, channel utilization reporting

- Wi-Fi 7: 320 MHz channel, SLO/MLO link setup/teardown, data model parameter validation

- Passpoint / Hotspot 2.0: GAS/ANQP query/response, Hotspot 2.0 indication element validation

- Band steering: trigger conditions, steering request generation, and client response handling

- WPS PBC and PIN mode: session initiation, credential exchange, and AP configuration update

## 40.4 Regression Campaign Structure

| Campaign | Duration | Trigger | Scope |
| --- | --- | --- | --- |
| Smoke Test | 10–30 min | Per build (daily CI) | Critical path: boot, WAN connectivity, Wi-Fi association, RBUS get/set, basic routing |
| Sanity Test | 1–2 hrs | Per branch merge | All CORE features for target profile; config persistence, factory reset |
| Feature Regression | 4–8 hrs | Weekly or per RC | Full CORE + OPT matrix for target profile including edge cases and negative tests |
| Full Platform Regression | 1–3 days | Per release | Complete matrix across all profiles and SoC platforms, including stress and soak |
| Operator ODQ Campaign | 1–4 weeks | Per operator qualification milestone | Operator-specified security, performance, interoperability, and compliance |

### 40.5 CI/CD Pipeline

### 40.5.1 Build CI

- Every component repository runs unit tests and static analysis (cppcheck, clang-tidy) on each push

- Yocto/BitBake builds triggered for each platform layer change, validating compile-time correctness

- Binary size and memory footprint metrics tracked over time to detect regressions

- AUTOREV injection patterns automatically pull the latest validated HEAD of each component repository

- TR-181 parameter count validated per CORE component on every build — deviation triggers blocking warning

- Timing SLA gates enforced as blocking checks — build fails on SLA regression

### 40.5.2 Hardware-in-the-Loop (HIL) CI

- Automated HIL labs at RDK Central and major operator/OEM facilities run smoke and sanity tests on every nightly build

- RDK Platform Test Suite Categories 1–3 (Data Model, State Machine, RBUS) run on every nightly HIL build

- Results published to a central dashboard; failures automatically create bug tickets

- Test environments defined as code (Docker containers for test controller, network topology definitions) for reproducibility

### 40.5.3 TIP Wi-Fi Validation

- TIP validation campaigns validate Wi-Fi interoperability between RDK-B implementations and third-party clients/APs

- OneWiFi AP interoperability tested against OneWiFi client and AP modes

## 40.6 Lab Topology

- Device Under Test (DUT): the gateway running the RDK-B build under test

- WAN Simulator: DOCSIS CMTS, Ethernet WAN switch, DSL DSLAM, or cellular access point emulator

- LAN Test Clients: wired and wireless clients for traffic generation and connectivity validation

- Test Controller: Linux server running Python-based RDK-B L2/L3 test harness, connected to DUT via SSH and RBUS

- Traffic Generator: iperf3, Spirent, or IXIA for throughput and latency benchmarking

- Packet Capture: Wireshark/tcpdump at WAN, LAN, and management interfaces for protocol-level validation

## 40.7 Acceptance Test Criteria (ATC)

| ATC Item | Acceptance Criterion |
| --- | --- |
| Boot Time | Operational state within 120 seconds of cold-start power-on |
| WAN Failover | Secondary WAN active within 60 seconds of primary failure; traffic resumption confirmed |
| Wi-Fi Throughput (Wi-Fi 6) | Minimum 600 Mbps aggregate at 3m in anechoic chamber |
| Wi-Fi Throughput (Wi-Fi 7) | Minimum 1.2 Gbps aggregate MLO at 3m in anechoic chamber |
| Memory at Idle | < 60% of available RAM consumed by RDK-B middleware |
| RBUS Latency | < 100ms round-trip for TR-181 get/set under normal load |
| Management Command SLA | Configuration change reflected within 30 seconds of cloud delivery |
| Log Upload Reliability | > 99.5% DCM upload success rate over 30-day fleet sample |
| Crash Rate | < 0.1% unplanned restarts per device per day over deployed fleet |
| Factory Reset | Clean operational state within 180 seconds; 50-cycle soak without failure |
| OTA Success Rate | > 99% success rate; automatic rollback on failure — no bricked devices |
| Memory Stability | < 1MB/hour memory leak over 7-day continuous soak |
| Data Model Stability | TR-181 parameter count per CORE component stable across three consecutive builds |
| State Machine Observability | All SM states observable via RBUS or sysevent within one poll interval |
| Restart Recovery | Component operational state restored within 30 seconds of restart |
| Stress Cycle Stability | 20 failover/restore cycles with no memory growth and no hung state |

# 41. Legacy Component Migration Plan

The RDK-B platform carries a set of legacy components that were written before the current architectural standards were established. These components use outdated IPC mechanisms, monolithic structure, and have excessive build-time dependencies. This section identifies each legacy component, its replacement path, and migration status.

| Legacy Component | Issue | Migration Target | Status |
| --- | --- | --- | --- |
| Provisioning and Management (P&M) | Monolithic shell scripts; thousands of lines with undefined scope; contains networking, firewall, routing, and system init mixed together | Split into discrete components: network-init, firewall-manager, routing-manager; each with RBUS interface | Planned — RDK9 program |
| Utopia | sysEvent IPC and sysConfig storage; legacy IPC mechanism not based on RBUS; difficult to test | Replace sysEvent with RBUS event pub/sub; replace sysConfig with PSM | In Progress — RBUS migration ongoing |
| CCSP Common Library (common-library) | Excessive memory allocation; legacy CCSP IPC shim; needed only for backward compatibility with old components | Eliminate as components migrate to native RBUS APIs | In Progress — each component migrated individually |
| CCSP WiFi Agent (CcspWiFiAgent) | Duplicate with OneWiFi; legacy CCSP-based Wi-Fi management; hard-coded dependencies | Fully replaced by OneWiFi | Complete — CcspWiFiAgent deprecated |
| Harvester | Hard-coded WebPA telemetry pipeline; not profile-configurable; legacy approach | Replaced by T2 Telemetry 2.0 profile-driven framework | Legacy — retained for backward compatibility only; not for new dev |
| SNMP Protocol Agent | Legacy SNMP management; not recommended for new operator deployments | Replaced by USP/TR-369 for new operator management infrastructure | Legacy — maintained for operators still using SNMP NMS |
| notify-component | Hard-coded DM value change notification; not flexible | Replaced by RBUS event pub/sub subscriptions | Legacy — deprecation planned |
| CcspEPONAgent (EPON) | Legacy EPON ONU agent using CCSP IPC | Replace with RBUS-native EPON manager if required | Legacy — not for new dev; EPON largely obsolete |
| DMCLI (data-model-cli) | Legacy CCSP command-line data model access tool | Replace with RBUS CLI tool | Legacy — available for backward compat; RBUS CLI preferred |

## 41.1 Migration Principles

- No new components should use SysEvent, sysConfig, DMCLI, or CCSP Common Lib IPC — all new code must use RBUS and PSM

- Existing components are migrated incrementally — priority given to components with the most downstream dependents

- Each migrated component must pass the complete existing test suite plus new RBUS-based interface tests before the legacy IPC is removed

- Migration tracking: a published dependency matrix is maintained and updated with each release showing current IPC/storage mechanism per component

- No component is marked migrated unless it has zero remaining sysEvent/DMCLI/Common Lib usage verified by CI static analysis

# 42. Component Repository Reference

All repositories are hosted at https://github.com/rdkcentral/ unless otherwise noted.

| Component / Repository | GitHub URL |
| --- | --- |
| OneWiFi | https://github.com/rdkcentral/OneWifi |
| unified-wifi-mesh | https://github.com/rdkcentral/unified-wifi-mesh |
| ieee1905-rs | https://github.com/rdkcentral/ieee1905-rs |
| mesh-agent | https://github.com/rdkcentral/mesh-agent |
| hotspot | https://github.com/rdkcentral/hotspot |
| OneWifiTestSuite | https://github.com/rdkcentral/OneWifiTestSuite |
| ethernet-agent | https://github.com/rdkcentral/ethernet-agent |
| moca-agent | https://github.com/rdkcentral/moca-agent |
| rdkb-halif-moca | https://github.com/rdkcentral/rdkb-halif-moca |
| rdkb-halif-test-moca | https://github.com/rdkcentral/rdkb-halif-test-moca |
| webui | https://github.com/rdkcentral/webui |
| webui-bwg | https://github.com/rdkcentral/webui-bwg |
| parodus / start-parodus / xmidt-org | https://github.com/xmidt-org/parodus2ccsp https://github.com/xmidt-org/parodus https://github.com/rdkcentral/start-parodus |
| usp-pa-vendor-rdk | https://github.com/rdkcentral/usp-pa-vendor-rdk |
| tr069-protocol-agent | https://github.com/rdkcentral/tr069-protocol-agent |
| snmp-protocol-agent | https://github.com/rdkcentral/snmp-protocol-agent |
| xconf-client / rfc | https://github.com/rdkcentral/xconf-client https://github.com/rdkcentral/rfc |
| WebConfig / WebconfigFramework | https://github.com/rdkcentral/webconfig https://github.com/rdkcentral/WebconfigFramework |
| open-virtual-switch-agent | https://github.com/rdkcentral/open-virtual-switch-agent |
| telemetry (T2) | https://github.com/rdkcentral/telemetry |
| crashupload | https://github.com/rdkcentral/crashupload |
| harvester | https://github.com/rdkcentral/harvester |
| dcm-agent | https://github.com/rdkcentral/dcm-agent |
| test-and-diagnostic | https://github.com/rdkcentral/test-and-diagnostic |
| rdk-speedtest-cli | https://github.com/rdkcentral/rdk-speedtest-cli |
| ipoe-health-check | https://github.com/rdkcentral/ipoe-health-check |
| remote_debugger / rdm-agent | https://github.com/rdkcentral/remote_debugger https://github.com/rdkcentral/rdm-agent |
| cpuprocanalyzer | https://github.com/rdkcentral/cpuprocanalyzer |
| notify-component | https://github.com/rdkcentral/notify-component |
| sys_mon_tools | https://github.com/rdkcentral/sys_mon_tools |
| breakpad_wrapper | https://github.com/rdkcentral/breakpad_wrapper |
| rdk_logger | https://github.com/rdkcentral/rdk_logger |
| syslog_helper | https://code.rdkcentral.com/r/plugins/gitiles/rdk/components/generic/syslog_helper/ |
| rbus | https://github.com/rdkcentral/rbus |
| data-model-cli (DMCLI) | https://github.com/rdkcentral/data-model-cli |
| common-library | https://github.com/rdkcentral/common-library |
| inter-device-manager | https://github.com/rdkcentral/inter-device-manager |
| json-hal-library | https://github.com/rdkcentral/json-hal-library |
| BartonCore | https://github.com/rdkcentral/BartonCore |
| BartonCommon | https://github.com/rdkcentral/BartonCommon |
| home-security | https://github.com/rdkcentral/home-security |
| DSM | https://github.com/rdkcentral/DSM |
| Dobby | https://github.com/rdkcentral/Dobby |
| media-terminal-adapter-agent | https://github.com/rdkcentral/media-terminal-adapter-agent |
| rdkb-halif-mta | https://github.com/rdkcentral/rdkb-halif-mta |
| rdkb-halif-test-mta | https://github.com/rdkcentral/rdkb-halif-test-mta |
| telco-voice-manager | https://github.com/rdkcentral/telco-voice-manager |
| hal-voice-asterisk | https://github.com/rdkcentral/hal-voice-asterisk |
| power-manager | https://github.com/rdkcentral/power-manager |
| rdk-halif-power_manager | https://github.com/rdkcentral/rdk-halif-power_manager |
| rdk-halif-test-power_manager | https://github.com/rdkcentral/rdk-halif-test-power_manager |
| platform-manager | https://github.com/rdkcentral/platform-manager |
| rdkb-halif-fwupgrade | https://github.com/rdkcentral/rdkb-halif-fwupgrade |
| rdkb-halif-test-fwupgrade | https://github.com/rdkcentral/rdkb-halif-test-fwupgrade |
| lan-manager-lite | https://github.com/rdkcentral/lan-manager-lite |
| secure-upnp | https://github.com/rdkcentral/secure-upnp |
| provisioning-and-management | https://github.com/rdkcentral/provisioning-and-management |
| utopia | https://github.com/rdkcentral/utopia |
| component-registry | https://github.com/rdkcentral/component-registry |
| miscellaneous-broadband | https://github.com/rdkcentral/miscellaneous-broadband |
| sysint-broadband | https://github.com/rdkcentral/sysint-broadband |
| common_utilities | https://github.com/rdkcentral/common_utilities |
| led-manager | https://github.com/rdkcentral/led-manager |
| persistent-storage-manager | https://github.com/rdkcentral/persistent-storage-manager |
| core-net-library | https://github.com/rdkcentral/core-net-library |
| firewall-manager | https://github.com/rdkcentral/firewall-manager |
| wan-manager | https://github.com/rdkcentral/wan-manager |
| ppp-manager | https://github.com/rdkcentral/ppp-manager |
| cable-modem-agent | https://github.com/rdkcentral/cable-modem-agent |
| xdsl-manager | https://github.com/rdkcentral/xdsl-manager |
| gpon-manager | https://github.com/rdkcentral/gpon-manager |
| cellular-manager | https://github.com/rdkcentral/cellular-manager |
| cellular-modem-manager | https://github.com/rdkcentral/cellular-modem-manager |
| dhcp-manager | https://github.com/rdkcentral/dhcp-manager |
| xdns | https://github.com/rdkcentral/xdns |
| advanced-security | https://github.com/rdkcentral/advanced-security |
| libSyscallWrapper | https://github.com/rdkcentral/libSyscallWrapper |
| rdk-cert-config | https://github.com/rdkcentral/rdk-cert-config |
| rdkssa | https://github.com/rdkcentral/rdkssa |
| rdk-libunpriv | https://github.com/rdkcentral/rdk-libunpriv |
| rdkb-apparmor-profiles | https://github.com/rdkcentral/rdkb-apparmor-profiles |

# 43. Revision History

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 2.0 | 2026-06 | RDK Central | v8 update: 26 features across 7 device profiles. Aligned all profile matrix cells with RDK-B_Component_List_2026_V1. Added Feature 2 (WiFi Hotspot), Feature 25 (Parental Control / Browsing Security), Feature 26 (Code Security). Updated Core Features column definition. Corrected 40+ profile cell values across Wi-Fi, Diagnostics, DHCP, Routing, and IPC features. Updated Core Features column tags for miscellaneous-broadband, sysint-broadband, common_utilities. |